3GPP TSG-RAN WG4 Meeting #88	R4-1810023

# Gothenburg, Sweden, August 20 – 24 , 2018

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draft | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on updating requirement for BWP switching delay in TS38.133 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Mediatek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-08-10 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | The current requirements for BWP switching delay are still TBD in spec |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the requirement of BWP switching delay with exact values |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirement for BWP switching delay is not completely |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  | TS 38.521-3 |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

----------------------START OF CHANGES----------------------------

8.6 Active BWP switch delay

8.6.1	Introduction

The requirements in this section apply for a UE configured with more than one BWP on PCell or any activated SCell in standalone NR, or PSCell or any activated SCell in SCG in EN-DC. UE shall complete the switch of active DL and/or UL BWP within the delay defined in this section.

8.6.2	UE active BWP switch delay

For DCI-based BWP switch, after the UE receives BWP switching request at slot n on a serving cell, UE shall be able to receive PDSCH (for DL active BWP switch) or transmit PUSCH (for UL active BWP switch) on the new BWP on the serving cell on which BWP switch occurs no later than at slot n+Y.

For timer-based BWP switch, the UE shall start BWP switch at slot n, where n is the beginning of a subframe (FR1) or half-subframe (FR2) immediately after a BWP-inactivity timer expires on a serving cell, and the UE shall be able to receive PDSCH (for DL active BWP switch) or transmit PUSCH (for UL active BWP switch) on the new BWP on the serving cell on which BWP switch occurs no later than at slot n+Y.

The UE is not required to transmit UL signals or receive DL signals during time duration Y on the cell where DCI-based BWP switch or timer-based BWP switch occurs.

Depending on UE capability, UE shall finish BWP switch within the time duration Y defined in Table 8.6.2-1, if the BWP switch involves changing the center frequency of the BWP, changing its  bandwidth and/or changing any baseband parameter specified in TS 38.331 [2].

Table 8.6.2-1: BWP switch delay with center frequency or BW changing

|  | NR Slot length (ms) | BWP switch delay Y (slots) |  |
| --- | --- | --- | --- |
|  |  | Type 1Note 1 | Type 2Note 1 |
| 0 | 1 | [3] | [2] |
| 1 | 0.5 | [6] | [4] |
| 2 | 0.25 | [12] | [8] |
| 3 | 0.125 | [24] | [16] |
| Note 1: Depends on UE capability. Note 2: If the BWP switch involves changing of SCS, the BWP switch delay is determined by the larger one between the SCS before BWP switch and the SCS after BWP switch. |  |  |  |

Depending on UE capability, UE shall finish BWP switch within the time duration Y defined in Table 8.6.2-2, if the BWP switch involves changing only the SCS.

Table 8.6.2-2: BWP switch delay without center frequency or BW changing

|  | NR Slot length (ms) | BWP switch delay Y (slots) |  |
| --- | --- | --- | --- |
|  |  | Type 1Note 1 | Type 2Note 1 |
| 0 | 1 | [2] | [2] |
| 1 | 0.5 | [4] | [3] |
| 2 | 0.25 | [8] | [6] |
| 3 | 0.125 | [16] | [12] |
| Note 1: Depends on UE capability. Note 2: The BWP switch delay is determined by the larger one between the SCS before BWP switch and the SCS after BWP switch. |  |  |  |

----------------------END OF CHANGES----------------------------