3GPP TSG-RAN4 Meeting #88	R4-1810073

# Gothenburg, Sweden, 20 - 24 Aug 2018

Source: 	Qualcomm Incorporated

Title: 	AMPR for sub-PRB for CAT M1

Agenda item:	6.5.2.2

Document for:	Approval

# Introduction

In the past meeting, [1] proposed AMPR for CATM1 for some NS values. In this paper, some of our simulation results for the same are presented.

# Discussion

Simulations to compute AMPR with 2, 3 and 6 subcarriers for CAT M1 were performed for NS_03 and NS_04.

Proposal 1:

Table 6.2.4-2xxx: A-MPR for "NS_04“ for Cat-M1 with sub-PRB allocation

| BW [MHz] | 5 MHz |  |
| --- | --- | --- |
| (NBindex,SCstart) | (0, ≤9) | (0, ≤9) |
| Lcsc | 2 | 3,6 |
| AMPR [dB] | ≤0.5 | ≤1 |

Proposal 2:

Table 6.2.4-2xxx: A-MPR for "NS_03“ for Cat-M1 with sub-PRB allocation

| BW [MHz] | 5 MHz |  |  |  |
| --- | --- | --- | --- | --- |
| (NBindex,SCstart) | (0, ≤9) | (0, ≥63) | (0, ≤9) | (0, ≥63) |
| Lcsc | 2 | 2 | 3,6 | 3,6 |
| AMPR [dB] | ≤0.5 | ≤0.5 | ≤1 | ≤1 |

# Conclusion

In this paper, proposals for AMPR for CAT M1 for NS_03 and NS_04 have been made in Proposals 1 and 2.

# References

[1] R4-1807454, updated A-MPR of CAT-M1 device for sub 1GHz for subPRB allocation, Busan, Korea, 21 May - 25 May 2018