3GPP TSG-RAN WG4 Meeting #88	R4-1810143

Göteborg, Sweden, 20 – 24 August 2018

Agenda Item:	6.5.6.2

Source:	Ericsson

Title:	Discussion on CQI requirements for eFeMTC

Document for:	Discussion

# 1	Introduction

RAN4#87 agreed with the way forward on eFeMTC UE demodulation and CSI reporting requirements [1].

- CQI definition with the CQ table right. Test point(s) are chosen so that UE report CQI indices 11 or more for 64QAM (See Table 4) - CQI definition with the CQ table right. Test point(s) are chosen so that UE report CQI indices for QPSK or 16QAM (See Table 5).

This contribution shows our simulation results for new CQI table and discusses the test point.

# 2	Discussion

Figure 1 shows the median of reported CQI for 4-bit CQI table 5 (CQI table for BL/CE UE supporting 64QAM) and CQI table 6 (CQI table for BL/CE UE supporting wide SNR range).

Figure 1	Median CQI for 4-bit CQI table 5 and 6.

## 2.1	Simulation results for CQI table 5

According to the agreement in the last meeting, we set test points corresponding to CQI index is 11 or larger for CQI table 5. From our simulation results, it is observed that the test point is SNR=11dB or larger. If we consider the coding rate of 0.5, we can select the test points corresponding to CQI index 11 (CR=0.45) or 12 (CR=0.55). If we choose SNR test points corresponding to CQI index 11, some UE may select CQI index 10 which corresponding to 16QAM. Considering the margin, we propose to set the test points corresponding to CQI index 12, that is SNR = 13/14dB.

For reference we should the distribution of reported CQI and BLER with median CQI+1 in Table 1.

Proposal 1: For the CQI definition test with CQI table 5, set test points to SNR=13/14dB. Reuse the same requirements used for the existing CQI definition test.

Table 1	Simulation results of CQI definition test with table 5.

| SNR [dB] | Median CQI | Percentage of reported CQI indexes which is median CQI, median CQI +/- 1 | BLER at median CQI | BLER at Median CQI +1 |
| --- | --- | --- | --- | --- |
| 11 | 11 | 100% | 0.00 | 1.00 |
| 12 | 11 | 100% | 0.00 | 0.59 |
| 13 | 12 | 100% | 0.00 | 1.00 |
| 14 | 12 | 100% | 0.00 | 0.79 |
| 15 | 13 | 100% | 0.00 | 1.00 |
| 16 | 13 | 100% | 0.00 | 0.80 |
| 17 | 14 | 100% | 0.00 | 1.00 |
| 18 | 14 | 100% | 0.00 | 0.97 |

## 2.2	Simulation results for CQI table 6

For CQI table 6, on the other hand, the agreement is to set SNR test points corresponding to QPSK (CQI index: 1 to 8) or 16QAM (CQI index: 9 to 12). Our interest is the region where the CQI table supports repetition (CQI index 1 to 4), however the corresponding test points are below -6dB. Considering the accuracy of SNR estimation, it is not realistic to set such low SNR test points. Other candidates are close to QPSK 1/3 (CQI index = 7) and/or 16QAM 1/2 (CQI index = 10). From the simulation results, the corresponding test points are SNR=1/2dB for QPSK and SNR=8/9dB for 16QAM. For reference we should the distribution of reported CQI and BLER with median CQI+/-1 in Table 2.

Proposal 2: For the CQI definition test with CQI table 6, set test points to SNR=1/2dB and SNR=8/9dB. Reuse the same requirements used for the existing CQI definition test.

Table 2	Simulation results of CQI definition test with table 6.

| SNR [dB] | Median CQI | Percentage of reported CQI indexes which is median CQI, median CQI +/- 1 | BLER at median CQI | BLER at Median CQI -1 | BLER at Median CQI +1 |
| --- | --- | --- | --- | --- | --- |
| -2 | 6 | 100 | 0.28 | 0.00 | 1.00 |
| -1 | 6 | 100 | 0.00 | 0.00 | 1.00 |
| 0 | 6 | 100 | 0.00 | 0.00 | 0.80 |
| 1 | 7 | 100 | 0.01 | 0.00 | 1.00 |
| 2 | 7 | 100 | 0.00 | 0.00 | 0.98 |
| 3 | 8 | 100 | 0.06 | 0.00 | 1.00 |
| 4 | 8 | 100 | 0.00 | 0.00 | 1.00 |
| 5 | 9 | 100 | 0.53 | 0.00 | 1.00 |
| 6 | 9 | 100 | 0.00 | 0.00 | 1.00 |
| 7 | 9 | 100 | 0.00 | 0.00 | 0.65 |
| 8 | 10 | 100 | 0.00 | 0.00 | 1.00 |
| 9 | 10 | 100 | 0.00 | 0.00 | 0.99 |
| 10 | 11 | 100 | 0.07 | 0.00 | 1.00 |
| 11 | 11 | 100 | 0.01 | 0.00 | 0.99 |
| 12 | 12 | 100 | 0.04 | 0.00 | 1.00 |

# 3	Summary

Proposal 1: For the CQI definition test with CQI table 5, set test points to SNR=13/14dB. Reuse the same requirements used for the existing CQI definition test.

Proposal 2: For the CQI definition test with CQI table 6, set test points to SNR=1/2dB and SNR=8/9dB. Reuse the same requirements used for the existing CQI definition test.

# 4	References

R4-1808009, “WF on eFeMTC UE demodulation and CSI requirements for BL UEs”, Huawei, HiSilicon.

# Appendix

Table 3	4-bit CQI Table 3. (CQI table for BL/CE UE introduced in Rel-13).

| CQI index | modulation | code rate x 1024 x | efficiency x |
| --- | --- | --- | --- |
| 0 | out of range |  |  |
| 1 | QPSK | 40 | 0.0781 |
| 2 | QPSK | 78 | 0.1523 |
| 3 | QPSK | 120 | 0.2344 |
| 4 | QPSK | 193 | 0.3770 |
| 5 | QPSK | 308 | 0.6016 |
| 6 | QPSK | 449 | 0.8770 |
| 7 | QPSK | 602 | 1.1758 |
| 8 | 16QAM | 378 | 1.4766 |
| 9 | 16QAM | 490 | 1.9141 |
| 10 | 16QAM | 616 | 2.4063 |
| 11 | Reserved | Reserved | Reserved |
| 12 | Reserved | Reserved | Reserved |
| 13 | Reserved | Reserved | Reserved |
| 14 | Reserved | Reserved | Reserved |
| 15 | Reserved | Reserved | Reserved |

Table 4	4-bit CQI Table 5. (CQI table for BL/CE UE supporting 64QAM).

| CQI index | modulation | code rate x 1024 | efficiency |
| --- | --- | --- | --- |
| 0 | out of range |  |  |
| 1 | QPSK | 40 | 0.0781 |
| 2 | QPSK | 78 | 0.1523 |
| 3 | QPSK | 120 | 0.2344 |
| 4 | QPSK | 193 | 0.3770 |
| 5 | QPSK | 308 | 0.6016 |
| 6 | QPSK | 449 | 0.8770 |
| 7 | QPSK | 602 | 1.1758 |
| 8 | 16QAM | 378 | 1.4766 |
| 9 | 16QAM | 490 | 1.9141 |
| 10 | 16QAM | 616 | 2.4063 |
| 11 | 64QAM | 466 | 2.7305 |
| 12 | 64QAM | 567 | 3.3223 |
| 13 | 64QAM | 666 | 3.9023 |
| 14 | 64QAM | 772 | 4.5234 |
| 15 | 64QAM | 873 | 5.1152 |

Table 5	4-bit CQI Table 6. (CQI table for BL/CE UE supporting wide SNR range).

| CQI index | modulation | code rate x 1024 | repetition |
| --- | --- | --- | --- |
| 0 | out of range |  |  |
| 1 | QPSK | 56 | 32 |
| 2 | QPSK | 207 | 16 |
| 3 | QPSK | 266 | 4 |
| 4 | QPSK | 195 | 2 |
| 5 | QPSK | 142 | 1 |
| 6 | QPSK | 266 | 1 |
| 7 | QPSK | 453 | 1 |
| 8 | QPSK | 637 | 1 |
| 9 | 16QAM | 423 | 1 |
| 10 | 16QAM | 557 | 1 |
| 11 | 16QAM | 696 | 1 |
| 12 | 16QAM | 845 | 1 |
| 13 | 64QAM | 651 | 1 |
| 14 | 64QAM | 780 | 1 |
| 15 | 64QAM | 888 | 1 |