3GPP TSG-RAN WG4 Meeting #88	R4-1810144

Göteborg, Sweden, 20 – 24 August 2018

Agenda Item:	6.5.7.1

Source:	Ericsson

Title:	Discussion on BS demodulation requirements for eFeMTC

Document for:	Discussion

# 1	Introduction

RAN4#87 agreed with the way forward on BS demodulation requirements for eFeMTC [1] as follows.

- RAN4 will introduce new PUSCH demodulation requirements with subPRB transmission - RAN4 will specify the demodulation requirements with 2 of 3 sub-carriers pi/2 BPSK - FFS: CEModeA and/or CEModeB - FFS: 3/6 subcarriers with QPSK demodulation requirements - RAN4 will wait for RAN1 conclusion on the detailed specification - RAN4 will introduce new PUSCH demodulation requirements for CEModeA with the following condition. Interested companies are encouraged provide the simulation results for alignment - 8 PUSCH repetitions - FRC A3-2 (QPSK 1/3, TBS=600bit, 6PRB) - Bandwidth 3, 5, 10, 15, and 20MHz - ETU200 - Antenna configuration: 1x2

In this contribution, we discuss the open issues on subPRB transmission and show simulation result for high speed scenario.

# 2	Discussion

## 2.1	PUSCH sub-PRB allocation

### 2.1.1	Summary of RAN1 agreements

RAN1 completed the sub-PRB specification and it is summarized in Table 1.

Table 1	Summary of PUSCH sub-PRB allocation.

| Sub-PRB allocation type | RU size | Modulation | Number of Rus | MCS/TBS |
| --- | --- | --- | --- | --- |
| 2 of 3 subcarriers | 8ms | Pi/2 BPSK | 1, 2, and 4 for CE Mode A 2 and 4 for CE Mode B | 8 values for MCS index (0-7) TBS: See Table 2 |
| 3 subcarriers | 4ms | QPSK |  |  |
| 6 subcarriers | 2ms | QPSK |  |  |

Table 2	TBS table for subPRB PUSCH

| I_TBS | NPRB |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 |  |  |  |
| 0 | 16 | 32 | 56 | 88 | 120 | 152 |  |  |  |
| 1 | 24 | 56 | 88 | 144 | 176 | 208 |  |  |  |
| 2 | 32 | 72 | 144 | 176 | 208 | 256 |  |  |  |
| 3 | 40 | 104 | 176 | 208 | 256 | 328 |  |  |  |
| 4 | 56 | 120 | 208 | 256 | 328 | 408 |  |  |  |
| 5 | 72 | 144 | 224 | 328 | 424 | 504 |  |  |  |
| 6 | 328 | 176 | 256 | 392 | 504 | 600 |  |  | 1 RU |
| 7 | 104 | 224 | 328 | 472 | 584 | 712 |  |  | 2 RUs |
| 8 | 120 | 256 | 392 | 536 | 680 | 808 |  |  | 4 RUs |
| 9 | 136 | 296 | 456 | 616 | 776 | 936 |  |  |  |
| 10 | 144 | 328 | 504 | 680 | 872 | 10321000 |  |  |  |

### 2.1.2	Simulation assumption of 2 of 3 sub-carriers pi/2 BPSK

Figure 1 illustrates the RAN1 assumption of 2 out of 3 sub-carriers (sub-PRB) Pi/2 transmitter [3]. From the FRC design point of view, this modulator is equivalent to the BPSK modulation with 2 sub-carriers (1 bit/symbol).

Figure 1	Signal flow of 2 out of 3 sub-carriers Pi/2 PBSK transmitter.

RAN1 keeps one DMRS symbol is transmitted every slot (0.5ms) per subcarrier. This means 12 of 14 OFDM symbols per sub-carrier per subframe (1ms) can be used for PUSCH transmission in the case of normal CP. In the case of 8ms RU length, the channel bits per RU is given as 192 bits (=12 symbols/sub-carrier/1ms * 2 sub-carriers * 8ms * 1bit/symbol).

Table 3 is our proposal of FRC for PUSCH demodulation with sub-PRB transmission. In principle it is equivalent with A16-2 specified in TS36.104 (See appendix) used for NB-IoT NPUSCH format 1 15kHz single tone.

Table 3	Proposed FRC for PUSCH demodulation with sub-PRB transmission.

| Reference channel | Axx-1 |
| --- | --- |
| Diversity | No |
| Allocated resource blocks | 1 |
| Number of subcarriers | 2 out of 3 |
| DFT-OFDM Symbols per subframe | 12 |
| Modulation | π/2 BPSK |
| Code rate | 1/3 |
| MCS index | 0 |
| Payload size (bits) | 32 |
| Transport block CRC (bits) | 24 |
| Code block CRC size (bits) | 0 |
| Number of code blocks - C | 1 |
| Total number of bits per resource unit | 192 |
| Total symbols per resource unit | 192 |
| Number of RUs | 1 |
| Tx time (ms) | 8 |

Only the difference between NB-IoT NPUSCH format 1 single tone transmission and eMTC 2 of 3 subcarriers transmission is the RV sequence pattern. In NB-IoT NPUSH format 1 single tone, DCI N0 indicates the initial RV value, RV0 or RV2. In principle, UE starts with the signaled RV and changes every RU transmission. For example, if the initial RV is set to RV2, the RV sequence is {2, 0, 2, 0, 2, …}

On the other hand, for the PUSCH subPRB allocation, DCI 6-0A (for CE Mode A) indicates the initial RV value from RV0, RV1, RV2, or RV3, and RV changes every RU transmission. For example, if the initial RV is set to RV2, the RV sequence is {2, 3, 1, 0, 2, 3, …}. For CE Mode B, no initial RV is indicated in DCI 6-0B, and the fixed RV sequence {0, 2, 3, 1, 0, …} is used.

Table 4 is the NPUSCH format 1 15kHz single tone requirements specified in TS36.104. Since the FRC is equivalent, we think the required SNR values are same or very close to these values. For repetition number 1 scenario, if the initial RV is set to RV0, the RV sequence is also same and therefore we can reuse the same requirements. For the repetition levels 16 or 64, RAN4 may need study the performance difference due to the different RV sequence.

Table 4	NPUSCH format 1 (single tone, 15kHz) demodulation requirements (TS36.104 Table 8.5.1.1.1-2).

| Number of TX antennas | Number of RX antennas | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Repetition number | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 15KHz | 1 | ETU 1Hz Low | A16-2 | 1 | 70% | -2.1 |
|  |  |  |  |  |  | 16 | 70% | -8.8 |
|  |  |  |  |  |  | 64 | 70% | -12.6 |

Proposal 1: For PUSCH demodulation requirements with 2 of 3 sub-carriers pi/2 BPSK, RAN4 introduces the following new demodulation requirements by reusing the NPUSCH format 1 15kHz single tone test cases:

- CE Mode A: 1 and 16 repetitions, ETU1 1x2, FRC A16-2

- CE Mode B: 64 repetitions, ETU1 1x2 FRC A16-2

Proposal 2: For PUSCH demodulation requirements with 2 of 3 sub-carriers pi/2 BPSK, RAN4 will study the performance difference due to the RV sequence difference from NPUSCH format 1.

## 2.2	PUSCH CE Mode A with high Doppler

As Rel-15 eMTC WID mentions, one of the objectives is to specify new demodulation requirements for high UE velocity in CE Mode A [2]. TS36.104 specifies one set of BS demodulation requirements for PUSCH with CE Mode A as shown in Table 5 and Table 6. We propose to specify the BS demodulation requirement by reusing the same configuration except for the propagation condition. Table 7 shows our ideal simulation results with EPA200.

Table 5	Test parameter for PUSCH CE mode A

| Parameter | unit | Mode A |
| --- | --- | --- |
| Maximum number of HARQ transmissions |  | 4 |
| RV sequences |  | 0, 2, 3, 1, 0, 2, 3, 1 |
| Number of PUSCH repetitions |  | 8 |
| Frequency hopping |  | ON |
| Frequency hopping interval | subframes | 4: FDD 5: TDD |

Table 6	Test case for PUSCH CE Mode A

| Number of TX antennas | Number of RX antennas | CE Mode | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | Mode A | ETU 200Hz Low | A3-2 | 70% |

Table 7	Simulation results for EPA200.

| Channel Bandwidth | 3MHz | 5MHz | 10MHz | 15MHz | 20MHz |
| --- | --- | --- | --- | --- | --- |
| SNR for 70% of the maximum throughput | -8.9 | -8.9 | -8.9 | -8.9 | -8.9 |

Proposal 3: RAN4 consider our simulation results for EPA200 for specifying PUSCH demodulation requirements with high Doppler scenario.

# 3	Conclusion

Proposal 1: For PUSCH demodulation requirements with 2 of 3 sub-carriers pi/2 BPSK, RAN4 introduces the following new demodulation requirements by reusing the NPUSCH format 1 15kHz single tone test cases:

- CE Mode A: 1 and 16 repetitions, ETU1 1x2, FRC A16-2

- CE Mode B: 64 repetitions, ETU1 1x2 FRC A16-2

Proposal 2: For PUSCH demodulation requirements with 2 of 3 sub-carriers pi/2 BPSK, RAN4 will study the performance difference due to the RV sequence difference from NPUSCH format 1.

Proposal 3: RAN4 consider our simulation results for EPA200 for specifying PUSCH demodulation requirements with high Doppler scenario.

# 4	References

R4-1807976, “Way forward on BS demodulation requirements for eFeMTC”, Ericsson.

RP-172811, “Revised WID on Even further enhanced MTC for LTE”, Ericsson, Qualcomm.

R1-1801448, “PUSCH Sub-PRB Design”, Sierra Wireless.

# Appendix

Table A.16.1-1: FRC parameters for NB-IoT NPUSCH format 1

| Reference channel | A16-1 | A16-2 | A16-3 | A16-4 | A16-5 |
| --- | --- | --- | --- | --- | --- |
| Subcarrier spacing (kHz) | 3.75 | 15 | 15 | 15 | 15 |
| Number of allocated subcarriers | 1 | 1 | 3 | 6 | 12 |
| Diversity | No | No | No | No | No |
| Modulation | BPSK | BPSK | QPSK | QPSK | QPSK |
| ITBS / IRU | 0 / 1 | 0 / 1 | 3 / 0 | 7 / 0 | 9 / 0 |
| Payload size (bits) | 32 | 32 | 40 | 104 | 136 |
| Allocated resource unit | 2 | 2 | 1 | 1 | 1 |
| Code rate (target) | 1/3 | 1/3 | 1/3 | 1/3 | 2/3 |
| Code rate (effective) | 0.29 | 0.29 | 0.22 | 0.44 | 0.56 |
| Transport block CRC (bits) | 24 | 24 | 24 | 24 | 24 |
| Code block CRC size (bits) | 0 | 0 | 0 | 0 | 0 |
| Number of code blocks - C | 1 | 1 | 1 | 1 | 1 |
| Total number of bits per resource unit | 96 | 96 | 288 | 288 | 288 |
| Total symbols per resource unit | 96 | 96 | 144 | 144 | 144 |
| Channel estimation length (ms) Note 1 | 16 | 4 | 4 | 4 | 2 (when repetition = 2) 4 (when repetition > 2) |
| Note 1: Channel estimation lengths are included in the table for information only. |  |  |  |  |  |