3GPP TSG-RAN WG4 Meeting 88	R4-1810148

Göteborg, Sweden, 20 – 24 August 2018

Agenda Item:	6.7.7.1.1

Source:	Ericsson

Title:	Simulation results of PDSCH for sTTI UE demodulation requirements

Document for:	Discussion

# 1	Introduction

RAN4#AH1807 agreed with the way forward on UE demodulation requirements for sTTI [1].

- Slot-based PDSCH demodulation requirements - In slot-based PDSCH test cases, for a TB first transmitted at slot n (=0,1), any retransmission can only happen on the slots satisfying following table - When the NAK is received at slot m, retransmission can happen no earlier than slot m+4 - Subslot-based PDSCH demodulation requirements - In subslot-based PDSCH test cases, for a TB first transmitted at subslot n (=1,2,3,4,5), any retransmission can only happen on the subslots satisfying following table - When the NAK is received at sub-slot m, retransmission can happen no earlier than sub-slot m+4 or m+6, depending on UE capability - Slot/Subslot-PDSCH demodulation requirements - No PDSCH allocation on subframe 0 and subframe 5 for all sPDSCH test cases - Target coding rate 0.45 for subslot-based CRS-based PDSCH demodulation - Consider 2 NZP CSI-RS ports for DMRS-based slot/subslot-PDSCH test cases - Companies are encouraged to provide the simulation results of sPDSCH based on the agreement - Simulation assumption: R4-1805493

This contribution provides the simulation results based on the agreement.

# 2	Simulation results

RAN4#AH1807 agreed to create a rule for the retransmission scheduling to avoid the coding rate change between the initial transmission and retransmissions. Table 1 and Table 2 show the retransmission rules for slot-PDSCH and subslot-PDSCH, respectively. For DMRS-based subslot-PDSCH, as we also discussed in [3], if BS does not schedule PDSCH in subslot 1, BS does not schedule PDSCH in subslot 2, and if BS does not schedule PDSCH in subslot 3, BS should not schedule PDSCH in subslot 4.

Table 1	TBS scheduling for slot-PDSCH.

| Slot for the initial transmission | Slots used for the retransmission |
| --- | --- |
| Slot 0 | Slot 0 |
| Slot 1 | Slot 1 |

Table 2	TBS scheduling for subslot-PDSCH.

| Subslot for the initial transmission | Subslots used for the retransmission |
| --- | --- |
| Subslot 1 | Subslot 1, Subslot 5 |
| Subslot 2 | Subslot 2, Subslot 4 |
| Subslot 3 | Subslot 3 |
| Subslot 4 | Subslot 2, Subslot 4 |
| Subslot 5 | Subslot 1, Subslot 5 |

Figure 1 shows the simulation result of CRS-based (TM3) slot-PDSCH and subslot-PDSCH. The detailed parameters such as TBS is given in A.1.1.

Figure 2 shows the simulation result of DMRS-based (TM9) slot-PDSCH and subslot-PDSCH. The detailed parameters such as TBS is given in A.1.2.

Figure 1	Simulation results of slot/subslot-PDSCH TM3 dual layer.

Figure 2	Simulation results of slot/subslot-DPSCH TM9 dual layer.

Table 3 summarizes the SNR achieving 70% of the maximum throughput. We propose RAN4 considers our simulation results for UE demodulation requirement.

Table 3	Simulation results of Slot-PDSCH and Subslot-PDSCH.

|  | Slot-PDSCH | Subslot-PDSCH |
| --- | --- | --- |
| CRS-based (TM3) | 11.1dB | 9.5dB |
| DMRS-based (TM9) | 3.8dB | 3.8dB |

# References

- R4-1809375, “Way forward on reference channels for sTTI demodulation and CSI requirements”, Qualcomm, Ericsson, Huawei.

- R4-1808812, “Update of simulation assumption for sTTI UE demodulation and CSI requirements”, Ericsson.

- R4-1808808, “Simulation results of PDSCH for sTTI UE demodulation requirements”, Ericsson.

# Appendix

### A.1	FRC tables

#### A.1.1	CRS-based PDSCH

##### A.1.1.1	Slot-based transmission

|  | Unit | Test 1-2 (Ref. R.11) |
| --- | --- | --- |
| Description |  | 4 CRS ports |
| Bandwidth | MHz | 10 |
| Allocated resource blocks |  | 50 |
| Modulation |  | 16QAM |
| Target coding rate |  | 1/2 |
| Number of HARQ processes |  | TBD |
| Maximum number of HARQ transmissions |  | 4 |
| Information bit payload per slot (two codewords) |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 | Bits |  |
| Slot index 0 | Bits | 11448 |
| Slot index 1 | Bits | 14112 |
| For Sub-Frames 0,5 |  | N/A |
| Binary channel bits per slot |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 | Bits |  |
| Slot index 0 | Bits | 22400 |
| Slot index 1 | Bits | 28288 |
| For Sub-Frames 0,5 |  | N/A |
| Note: Given per component carrier |  |  |

##### A.1.1.2	Subslot-based transmission

|  | Unit | Test 1-2 (Ref. R.11) |
| --- | --- | --- |
| Description |  | 4 CRS ports |
| Bandwidth | MHz | 10 |
| Allocated resource blocks |  | 50 |
| Modulation |  | 16QAM |
| Target coding rate |  | 1/2 |
| Number of HARQ processes |  | 8 |
| Maximum number of HARQ transmissions |  | 4 |
| Information bit payload per subslot (Two codewords) |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 |  |  |
| Subslot index 1 | Bits | 5160 |
| Subslot index 2 | Bits | 3880 |
| Subslot index 3 | Bits | 2664 |
| Subslot index 4 | Bits | 3880 |
| Subslot index 5 | Bits | 5160 |
| For Sub-Frames 0,5 |  | N/A |
| Binary channel bits per subslot |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 |  |  |
| Subslot index 1 | Bits | 12032 |
| Subslot index 2 | Bits | 8832 |
| Subslot index 3 | Bits | 5888 |
| Subslot index 4 | Bits | 8832 |
| Subslot index 5 | Bits | 12288 |
| For Sub-Frames 0,5 |  | N/A |
| Note: Given per component carrier |  |  |

#### A.1.2	DMRS-based PDSCH

##### A.1.2.1	Slot-based transmission

|  | Unit | Test 2 (Ref. R.66) |
| --- | --- | --- |
| Bandwidth | MHz | 10 |
| Allocated resource blocks |  | 50 |
| Modulation |  | QPSK |
| Target coding rate |  | 1/3 |
| Number of HARQ processes |  | [8] |
| Maximum number of HARQ transmissions |  | 4 |
| Information bit payload per slot (One codeword) |  |  |
| For Sub-Frames 1,3,4,6,8,9 |  |  |
| Slot index 0 | Bits | 3624 |
| Slot index 1 | Bits | 4392 |
| For Sub-Frames 2,7 (2 NZP-CSI-RS ports) |  |  |
| Slot index 0 | Bits | 2856 |
| Slot index 1 | Bits | 4392 |
| For Sub-Frames 0,5 |  | N/A |
| Binary channel bits per slot |  |  |
| For Sub-Frames 1,3,4,6,8,9 |  |  |
| Slot index 0 | Bits | 10000 |
| Slot index 1 | Bits | 13360 |
| For Sub-Frames 2,7 (2 NZP-CSI-RS ports) |  |  |
| Slot index 0 | Bits | 9600 |
| Slot index 1 | Bits | 13360 |
| For Sub-Frames 0,5 |  | N/A |
| Note: CSI-RS configuration = 0 for NZP-CSI-RS Note: Given per component carrier |  |  |

##### A.1.2.2	Subslot-based transmission

|  | Unit | Test 2 (Ref. R.44) |
| --- | --- | --- |
| Bandwidth | MHz | 10 |
| Allocated resource blocks |  | 50 |
| Modulation |  | 16QAM |
| Target coding rate |  | 1/2 |
| Number of HARQ processes |  | FFS |
| Maximum number of HARQ transmissions |  | 4 |
| Information bit payload per subslots (One codeword) |  |  |
| For Sub-Frames 1,3,4,6,8,9 |  |  |
| Subslot index 1 | Bits | 1736 |
| Subslot index 2 | bits | 1192 |
| Subslot index 3 | Bits | 776 |
| Subslot index 4 | Bits | 1192 |
| Subslot index 5 | Bits | 1736 |
| For Sub-Frames 2,7 (2 NZP-CSI-RS ports) |  |  |
| Subslot index 1 | Bits | 1736 |
| Subslot index 2 | bits | 1192 |
| Subslot index 3 | Bits | 776 |
| Subslot index 4 | Bits | 1192 |
| Subslot index 5 | Bits | 1736 |
| For Sub-Frames 0,5 |  | N/A |
| Binary channel bits per subslots |  |  |
| For Sub-Frames 1,3,4,6,8,9 |  |  |
| Subslot index 1 | Bits | 4960 |
| Subslot index 2 | bits | 4032 |
| Subslot index 3 | Bits | 2688 |
| Subslot index 4 | Bits | 4032 |
| Subslot index 5 | Bits | 5088 |
| For Sub-Frames 2,7 (2 NZP-CSI-RS ports) |  |  |
| Subslot index 1 | Bits | 4960 |
| Subslot index 2 | bits | 3696 |
| Subslot index 3 | Bits | 2688 |
| Subslot index 4 | Bits | 4032 |
| Subslot index 5 | Bits | 5088 |
| For Sub-Frames 0,5 |  | N/A |
| Note: CSI-RS configuration = 0 for NZP-CSI-RS Note: Given per component carrier |  |  |