3GPP TSG-RAN WG4 Meeting #88              	R4-1810879

Gothenburg, SE, 20th – 24th Aug, 2018

Source:	Nokia, Nokia Shanghai Bell

Title:	On remaining issues for NR PUCCH demodulation

Agenda item:	7.13.2.3

Document for:	Discussion

Introduction

At RAN4#AH-1807 the scope of the work in Rel-15 and the simulation assumptions for alignment have been agreed, and some open issues on PUCCH demodulation are listed for further discussion in [1].In this paper, we will provide our views on the configurations for NR PUCCH demodulation work.

Discussion

It is agreed to define requirements for PUCCH format 0,1,2,3,4 and the applicability for each format is based on BS declaration. LTE test metric can be re-used in NR PUCCH performance requirements. But for NR PUCCH formats 2 and 3, CRC will be used when the payload size is greater than 11, and we think either “DTX to ACK” plus “missed ACK” or “BLER” can be used. The coverage related features, i.e. pi/2-BPSK and multi-slot PUCCH are for further study. The requirements of multiple user test cases with PUCCH format 4 will be done after single user tests cases are completed. The test setup for PUCCH format 0, 1, 2 and 3 are defined as following:

Table 1 Test setup for PUCCH Performance requirements

| PUCCH format | Format 0 | Format 1 | Format 2 | Format 3 |
| --- | --- | --- | --- | --- |
| Number of OFDM-symbols | 1, 2 | 6, 14 | 1, 2 | 4, 14 |
| UCI bit number | 1 | 2 | 4, 22 | 16 |
| Number of PRB | 1 | 1 | 4, 9 | 1 |
| initialCyclicShift | 0 | 0 | - | - |
| startingSymbolIndex | 13 for 1 OFDM symbol, 12 for 2 OFDM symbol | 0 | 13 for 1 OFDM symbol, 12 for 2 OFDM symbol | 0 |
| The index of the orthogonal sequence i | - | 0 | - | - |
| DMRS pattern | - | - | - | with additional DMRS and without additional DMRS |
| Test metric | DTX to Ack probability <1% Missed Ack probability < 1% | DTX to Ack probability <1% Missed Ack probability < 1% NACK to Ack probability < 0.1% | DTX to Ack probability <1%, Missed Ack probability < 1%, if number of bits <= 11 and BLER if number of bits > 11 | DTX to Ack probability <1%, Missed Ack probability < 1%, if number of bits <= 11 and BLER if number of bits > 11 |

Consider the test setup for NR PUCCH performance requirements.

Conclusion

In this paper, we provided our views on open issues for NR PUCCH demodulation work.

Proposal 1:Consider the test setup for NR PUCCH performance requirements.

References

- R4-1809556, Way forward on NR PUCCH demodulation performance in Rel-15, Huawei, Ericsson, Nokia, Nokia Shanghai Bell, Samsung, ZTE