3GPP TSG-RAN WG4 Meeting #88                	R4-1811118

Gothenburg, Sweden, 20-24 August 2018

Source:	Nokia, Nokia Shanghai Bell

Title:	Usage of SSB and SMTC periodicity in delay requirements

Agenda item:	7.11.1

Document for:	Discussion

Introduction

In this contribution we discuss the use of SSB and SMTC periodicity in handover, PSCell addition and SCell activation delay requirements as a continuation to the discussion around the topic in Montreal AH.

Discussion

In the last meeting an LS [1] was sent to RAN2 with request to update 36.331 and 38.331 specifications with SSB-related assistance information. The contents of the LS are repeated below.

1. Overall Description:

RAN4 would like to inform RAN2 that some of the SSB-related assistance information are missing in current TS 36.331 spec and TS 38.331 spec.

- In TS 38.331,

  - For NR PSCell/SCell addition and handover procedure,

    - The assistance information about SSB timing offset for the target cell is missing in ServingCellConfigCommon.

    - It is suggested to provide additional optional information for timing offset of SSB for the target cell based on the timing reference of current serving cell in ServingCellConfigCommon to avoid UE blindly search for the SSB of target cell the whole SSB period.

  - For RRC Release with redirection to an inter-frequency NR cell,

    - The SCS of SSB and SMTC are missing in RedirectedCarrierInfo.

    - It is suggested to provide additional optional information for SCS of SSB and SMTC periodicity/offset/duration in RedirectedCarrierInfo to provide the needed information for UE to search for the SSB of target cell.

- In TS 36.331

  - For RRC Release with redirection to an inter-RAT NR cell,

    - The SCS of SSB and SMTC are missing in RedirectedCarrierInfo.

    - It is suggested to provide additional optional information of SCS of SSB and SMTC periodicity/offset/duration in RedirectedCarrierInfo to provide the needed information for UE to search for the SSB of target cell.

Related to this LS, it was proposed in [2] that SSB periodicity should be used instead of SMTC periodicity in the delay requirements for handover, PSCell addition and SCell activation. This proposal is related to the note under the handover requirements:

NOTE 2: if the SMTC periodicity is not configured, the term SMTC periodicity in Tsearch and T∆ shall be deemed to be replaced with SSB periodicity.

SSB periodicity may be shorter than SMTC periodicity, so in case SSB periodicity is available at the UE, using this as the requirement would potentially lead to a shorter delay. It depends on RAN2 agreements whether SSB periodicity is always known by the UE in handover etc. but if SSB periodicity is available, we see a benefit for using this in the requirements. On the other hand, if SSB periodicity is not available, the UE should use SMTC periodicity.

We would propose to wait for RAN2 to conclude with the RAN4 request in the LS, and based on this, change the requirements such that they are primarily based on SSB periodicity, and if SSB periodicity is not known by the UE, the requirement is based on SMTC periodicity.

Proposal 1: Define handover, PSCell addition and SCell activation requirements based on SSB periodicity. If SSB periodicity is not available at the UE, SMTC periodicity is used instead.

Conclusion

In this contribution we have discussed the usage of SSB/SMTC periodicity in handover, SCell activation and PSCell addition delay requirements. We have made the following proposal:

Proposal 1: Define handover, PSCell addition and SCell activation requirements based on SSB periodicity. If SSB periodicity is not available at the UE, SMTC periodicity is used instead.

- References

[1] R4-1809547, LS on missing SSB information, RAN4 to RAN2, 3GPP TSG-RAN WG4 Meeting #AH1807, Montreal, Canada, July 2nd –6th, 2018.

[2] R4-1808841, Missing SSB information for target NR cell, MediaTek Inc., 3GPP TSG-RAN WG4 Meeting #AH1807, Montreal, Canada, July 2nd –6th, 2018.