3GPP TSG-RAN WG4 Meeting #88                                                                                                              R4-1810247

# Gothenburg, SE, 20th – 24th Aug, 2018

Agenda Item:	7.12.3.7

Source: 	MediaTek Inc.

Title: 	SSB-based RLM test cases for SA

Document for:	Discussion

1	Introduction

In this paper, we provide SSB-based RLM test cases for SA.

2	Overview of SA NR RLM test cases

In the following subsections, the following settings are adopted.

- SSB and other DL signals and channels have the same numerology

- 1 SSB per SSB burst for both FR1 and FR2 tests

- CSI-RS based beam management is considered in both FR1 and FR2

  - Evaluation period with N=1 in FR2 is tested

- RLM-RS that partially overlapped with measurement gap is considered in OOS test cases

To reduce the number of NR RLM test cases, NR RLM test cases are provided in sections 2.1-2.12. The overview on SA NR RLM test cases is shown below.

| Section 2.1 | Non-DRX | FR1 | FDD | OOS | w/o meas. gap | Test 1-A: SCS 15KHz, Test 1-B: SCS 30KHz |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | w/ meas. gap | Test 2-A: SCS 15KHz, Test 2-B: SCS 30KHz |
| Section 2.2 |  |  |  | INS | w/o meas. gap | Test 1-A: SCS 15KHz, Test 1-B: SCS 30KHz |
|  |  |  |  |  | w/ meas. gap | Test 2-A: SCS 15KHz, Test 2-B: SCS 30KHz |
| Section 2.3 |  |  | TDD | OOS | w/o meas. gap | Test 1-A: SCS 15KHz, Test 1-B: SCS 30KHz |
|  |  |  |  |  | w/ meas. gap | Test 2-A: SCS 15KHz, Test 2-B: SCS 30KHz |
| Section 2.4 |  |  |  | INS | w/o meas. gap | Test 1-A: SCS 15KHz, Test 1-B: SCS 30KHz |
|  |  |  |  |  | w/ meas. gap | Test 2-A: SCS 15KHz, Test 2-B: SCS 30KHz |
| Section 2.5 |  | FR2 | TDD | OOS | w/o meas. gap | Test 1: SCS 120KHz |
|  |  |  |  |  | w/ meas. gap | Test 2: SCS 120KHz |
| Section 2.6 |  |  |  | INS | w/o meas. gap | Test 1: SCS 120KHz |
|  |  |  |  |  | w/ meas. gap | Test 2: SCS 120KHz |
| Section 2.7 | DRX | FR1 | FDD | OOS | w/o meas. gap | Test 1-A: SCS 15KHz, Test 1-B: SCS 30KHz |
|  |  |  |  |  | w/ meas. gap | Test 2-A: SCS 15KHz, Test 2-B: SCS 30KHz |
| Section 2.8 |  |  |  | INS | w/o meas. gap | Test 1-A: SCS 15KHz, Test 1-B: SCS 30KHz |
|  |  |  |  |  | w/ meas. gap | None |
| Section 2.9 |  |  | TDD | OOS | w/o meas. gap | Test 1-A: SCS 15KHz, Test 1-B: SCS 30KHz |
|  |  |  |  |  | w/ meas. gap | Test 2-A: SCS 15KHz, Test 2-B: SCS 30KHz |
| Section 2.10 |  |  |  | INS | w/o meas. gap | Test 1-A: SCS 15KHz, Test 1-B: SCS 30KHz |
|  |  |  |  |  | w/ meas. gap | None |
| Section 2.11 |  | FR2 | TDD | OOS | w/o meas. gap | Test 1: SCS 120KHz |
|  |  |  |  |  | w/ meas. gap | Test 2: SCS 120KHz |
| Section 2.12 |  |  |  | INS | w/o meas. gap | Test 1: SCS 120KHz |
|  |  |  |  |  | w/ meas. gap | None |

Table 1: Overview on SA NR RLM tests

2.1                SA NR FDD FR1 OOS testing in non-DRX mode

One SS/PBCH per SSB burst is configured for cell 1. In Tables 2-4, we provide the test parameters for FDD FR1 out-of-sync testing in non-DRX mode and the key points are highlighted.

Table 2: General test parameters for NR FR1 FDD out-of-sync testing in non-DRX mode

| Parameter |  | Unit | Value |  |  |  | Coment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1-A | Test 1-B | Test 2-A | Test 2-B |  |
| PDCCH parameters |  |  | [R. TBD FDD] | [R. TBD FDD] | [R. TBD FDD] | [R. TBD FDD] | As specified in Table TBD. None of the PDCCH are intended for the UE under test |
| SSB Periodicity |  | ms | [20] | [20] | [20] | [20] | One SS/PBCH per SSB burst |
| OCNG parameters |  |  | TBD | TBD | TBD | TBD |  |
| Active cell |  |  | Cell 1 | Cell 1 | Cell 1 | Cell 1 | Cell 1 is on NR RF channel number 1 |
| CP length |  |  | Normal | Normal | Normal | Normal |  |
| NR RF Channel Number |  |  | 1 | 1 | 1 | 1 | One NR FDD FR1 carrier frequency is used. |
| NR Channel Bandwidth (BWchannel) |  | MHz | [10] | [40] | [10] | [40] |  |
| Subcarrier Spacing |  | KHz | [15] | [30] | [15] | [30] |  |
| Correlation Matrix and Antenna Configuration |  |  | [1x2 Low] | [1x2 Low] | [1x2 Low] | [1x2 Low] |  |
| Out of sync transmission parameters (Note 1) | DCI format |  | 1-0 | 1-0 | 1-0 | 1-0 | As defined in clause 7.3.1.2.1 in TS 38.212 |
|  | Number of Control OFDM symbols |  | 2 | 2 | 2 | 2 | Out of sync threshold Qout and the corresponding hypothetical PDCCH transmission parameters are as specified in clause 8.1.2 and Table 8.1.2.1-1 respectively. |
|  | Aggregation level | CCE | 8 | 8 | 8 | 8 |  |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 4 | 4 | 4 | 4 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 4 | 4 | 4 | 4 |  |
|  | DMRS precoder granularity |  | REG bundle size | REG bundle size | REG bundle size | REG bundle size |  |
|  | REG bundle size |  | 6 | 6 | 6 | 6 |  |
| DRX |  |  | OFF | OFF | OFF | OFF |  |
| Gap pattern ID |  |  | [N.A.] | [N.A.] | *[gp0] | *[gp0] | *RLM RS is partially overlapped with measurement gap when measurement gap is configured. P = 2 in evaluation period |
| Layer 3 filtering |  |  | Enabled | Enabled | Enabled | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 0 | 0 | 0 | 0 | T310 is disabled |
| T311 timer |  | ms | 1000 | 1000 | 1000 | 1000 | T311 is enabled |
| Number of NZP CSI-RS resource within the NZP CSI-RS resource set for beam management |  |  | TBD | TBD | TBD | TBD | For BM, repetition is ON and NZP CSI-RS resources are QCL-TypeD with SSB for RLM, e.g., 4 or 8 |
| NZP CSI-RS periodicity |  | slot | [5] | [10] | [5] | [10] |  |
| Periodic CSI reporting |  |  | PUCCH | PUCCH | PUCCH | PUCCH | reportQuantity is set as cri-RSRP |
| CSI reporting periodicity |  | slot | [5] | [10] | [5] | [10] | FDD and TDD can have the same reporting behavior |
| Propagation channel |  |  | [AWGN] | [AWGN] | [AWGN] | [AWGN] |  |
| T1 |  | s | 1 | 1 | 1 | 1 |  |
| T2 |  | s | 0.4 | 0.4 | 0.4 | 0.4 |  |
| T3 |  | s | [0.6] | [0.6] | [0.6] | [0.6] |  |
| T4 |  | s | [0.24] | [0.24] | [0.44] | [0.44] |  |
| Note 1: PDCCH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel. Note 2: SSB and DL data have the same numerology. Note 3: preambleTransMax value of [200], periodicity of PRACH occasion of [160 ms] and ra-ResponseWindow of [sl2] areconfigured for beam failure recovery. |  |  |  |  |  |  |  |

Table 3: Cell specific test parameters for NR FDD (cell # 1) for out-of-sync radio link monitoring tests #1-A, #1-B, # 2-A and #2-B in non-DRX mode.

| Parameter | Unit | Test 1-A |  |  | Test 1-B |  |  | Test 2-A |  |  | Test 2-B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| NR RF Channel Number |  | 1 |  |  | 1 |  |  | 1 |  |  | 1 |  |  |
| BWchannel | MHz | [10] |  |  | [40] |  |  | [10] |  |  | [40] |  |  |
| Subcarrier Spacing | KHz | [15] |  |  | [30] |  |  | [15] |  |  | [30] |  |  |
| Antenna Configuration |  | [1x2 low] |  |  | [1x2 low] |  |  | [1x2 low] |  |  | [1x2 low] |  |  |
| OCNG Pattern |  | TBD |  |  | TBD |  |  | TBD |  |  | TBD |  |  |
| PDCCH_beta | dB | 4 |  |  | 4 |  |  | 4 |  |  | 4 |  |  |
| PDCCH_DMRS_beta | dB | 4 |  |  | 4 |  |  | 4 |  |  | 4 |  |  |
| PBCH_beta | dB | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  |
| PSS_beta | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_beta | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_beta | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_betaNote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| SNR Note 8,9 | dB | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | dBm/15 kHz | [-98] |  |  | [-98] |  |  | [-98] |  |  | [-98] |  |  |
| Propagation condition |  | [AWGN] |  |  | [AWGN] |  |  | [AWGN] |  |  | [AWGN] |  |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CS-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure TBD. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is TBD. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4: Measurement gap configuration for NR FDD out-of-sync tests in non-DRX mode.

| Field | Test 2-A | Test 2-B | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| gapOffset | [0] | [0] | As specified in clause 6.3.2 in TS 38.331. SSB for RLM shall partially overlap with measurement gap. |

Figure 1: SNR variation for out-of-sync testing.

Test Requirements:

The UE behavior in each test during time durations T1, T2 and T3 shall be as follows:

During the period from time point A to time point B the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

The UE shall stop transmitting uplink signal no later than time point C (T4 second after the start of the time duration T3).

The rate of correct events observed during repeated tests shall be at least 90%.

2.2                SA NR FDD FR1 INS testing in non-DRX mode

One SS/PBCH per SSB burst is configured for cell 1. In Tables 5-8, we provide the test parameters for FDD FR1 in-sync testing in non-DRX mode. Compared to section 2.1 (FDD FR1 OOS testing in non-DRX mode), we only highlight the differences.

Table 5: General test parameters for NR FR1 FDD in-sync testing in non-DRX mode

| Parameter |  | Unit | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1-A | Test 1-B | Test 2-A | Test 2-B |  |
| PDCCH parameters |  |  | [R. TBD FDD] | [R. TBD FDD] | [R. TBD FDD] | [R. TBD FDD] | As specified in Table TBD. None of the PDCCH are intended for the UE under test |
| SSB Periodicity |  | ms | [20] | [20] | [20] | [20] | One SS/PBCH per SSB burst |
| OCNG parameters |  |  | TBD | TBD | TBD | TBD |  |
| Active cell |  |  | Cell 1 | Cell 1 | Cell 1 | Cell 1 | Cell 1 is on NR RF channel number 1 |
| CP length |  |  | Normal | Normal | Normal | Normal |  |
| NR RF Channel Number |  |  | 1 | 1 | 1 | 1 | One NR FDD FR1 carrier frequency is used. |
| NR Channel Bandwidth (BWchannel) |  | MHz | [10] | [40] | [10] | [40] |  |
| Subcarrier Spacing |  | KHz | [15] | [30] | [15] | [30] |  |
| Correlation Matrix and Antenna Configuration |  |  | [1x2 Low] | [1x2 Low] | [1x2 Low] | [1x2 Low] |  |
| In sync transmission parameters (Note 3) | DCI format |  | 1-0 | 1-0 | 1-0 | 1-0 | As defined in clause 7.3.1.2.1 in TS 38.212 |
|  | Number of Control OFDM symbols |  | 2 | 2 | 2 | 2 | In sync threshold Qin and the corresponding hypothetical PDCCH transmission parameters are as specified in clause 8.1.2 and Table 8.1.2.1-2 respectively. |
|  | Aggregation level | CCE | 4 | 4 | 4 | 4 |  |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 0 | 0 | 0 | 0 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 0 | 0 | 0 | 0 |  |
|  | DMRS precoder granularity |  | REG bundle size | REG bundle size | REG bundle size | REG bundle size |  |
|  | REG bundle size |  | 6 | 6 | 6 | 6 |  |
| Out of sync transmission parameters (Note 3) | DCI format |  | 1-0 | 1-0 | 1-0 | 1-0 | As defined in clause 7.3.1.2.1 in TS 38.212 |
|  | Number of Control OFDM symbols |  | 2 | 2 | 2 | 2 | Out of sync threshold Qout and the corresponding hypothetical PDCCH transmission parameters are as specified in clause 8.1.2 and Table 8.1.2.1-1 respectively. |
|  | Aggregation level | CCE | 8 | 8 | 8 | 8 |  |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 4 | 4 | 4 | 4 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 4 | 4 | 4 | 4 |  |
|  | DMRS precoder granularity |  | REG bundle size | REG bundle size | REG bundle size | REG bundle size |  |
|  | REG bundle size |  | 6 | 6 | 6 | 6 |  |
| DRX |  |  | OFF | OFF | OFF | OFF |  |
| Gap pattern ID |  |  | [N.A.] | [N.A.] | *[gp0] | *[gp0] | *RLM RS is partially overlapped with measurement gap when measurement gap is configured. P = 2 in evaluation period |
| Layer 3 filtering |  |  | Enabled | Enabled | Enabled | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 2000 | 2000 | 2000 | 2000 | T310 is enabled |
| T311 timer |  | ms | 1000 | 1000 | 1000 | 1000 | T311 is enabled |
| Number of NZP CSI-RS resource within the NZP CSI-RS resource set for beam management |  |  | TBD | TBD | TBD | TBD | For BM, repetition is ON and NZP CSI-RS resources are QCL-TypeD with SSB for RLM |
| NZP CSI-RS periodicity |  | slot | [5] | [10] | [5] | [10] |  |
| Periodic CSI reporting |  |  | PUCCH | PUCCH | PUCCH | PUCCH | reportQuantity is set as cri-RSRP |
| CSI reporting periodicity |  | slot | [5] | [10] | [5] | [10] | FDD and TDD can have the same reporting behavior |
| Propagation channel |  |  | [AWGN] | [AWGN] | [AWGN] | [AWGN] |  |
| T1 |  | s | 0.5 | 0.5 | 0.5 | 0.5 |  |
| T2 |  | s | 0.4 | 0.4 | 0.4 | 0.4 |  |
| T3 |  | s | [1.46] | [1.46] | [1.36] | [1.36] | T310 >= T3+T4+TEvaluate_in + SSB burst duration |
| T4 |  | s | 0.4 | 0.4 | 0.4 | 0.4 |  |
| T5 |  | s | [1] | [1] | [1.1] | [1.1] | Same overall testing time |
| T6 |  | s | [0.42] | [0.42] | [0.72] | [0.72] | T6 > TEvaluate_out + T310 + 40 – T3 –T4 |
| Note 1: PDCCH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel. Note 2: SSB and DL data have the same numerology. Note 3: preambleTransMax value of [200], periodicity of PRACH occasion of [160 ms] and ra-ResponseWindow of [sl2] are configured for beam failure recovery. |  |  |  |  |  |  |  |

Table 6: Cell specific test parameters for NR FDD (cell # 1) for in-sync radio link monitoring tests #1-A, and #1-B in non-DRX mode.

| Parameter | Unit | Test 1-A |  |  |  |  | Test 1-B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 | T1 | T2 | T3 | T4 | T5 |
| NR RF Channel Number |  | 1 |  |  |  |  | 1 |  |  |  |  |
| BWchannel | MHz | [10] |  |  |  |  | [40] |  |  |  |  |
| Subcarrier Spacing | KHz | [15] |  |  |  |  | [30] |  |  |  |  |
| Antenna Configuration |  | [1x2 low] |  |  |  |  | [1x2 low] |  |  |  |  |
| OCNG Pattern |  | TBD |  |  |  |  | TBD |  |  |  |  |
| PDCCH_beta | dB | 4 |  |  |  |  | 4 |  |  |  |  |
| PDCCH_DMRS_beta | dB | 4 |  |  |  |  | 4 |  |  |  |  |
| PBCH_beta | dB | 0 |  |  |  |  | 0 |  |  |  |  |
| PSS_beta | dB |  |  |  |  |  |  |  |  |  |  |
| SSS_beta | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_beta | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_betaNote 1 | dB |  |  |  |  |  |  |  |  |  |  |
| SNR Note 8,9 | dB | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | dBm/15 kHz | [-98] |  |  |  |  | [-98] |  |  |  |  |
| Propagation condition |  | [AWGN] |  |  |  |  | [AWGN] |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CS-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure TBD. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 is modified as specified in section TBD. |  |  |  |  |  |  |  |  |  |  |  |

Table 7: Cell specific test parameters for NR FDD (cell # 1) for in-sync radio link monitoring tests #2-A and #2-B in non-DRX mode.

| Parameter | Unit | Test 2-A |  |  |  |  | Test 2-B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 | T1 | T2 | T3 | T4 | T5 |
| NR RF Channel Number |  | 1 |  |  |  |  | 1 |  |  |  |  |
| BWchannel | MHz | [10] |  |  |  |  | [40] |  |  |  |  |
| Subcarrier Spacing | KHz | [15] |  |  |  |  | [30] |  |  |  |  |
| Antenna Configuration |  | [1x2 low] |  |  |  |  | [1x2 low] |  |  |  |  |
| OCNG Pattern |  | TBD |  |  |  |  | TBD |  |  |  |  |
| PDCCH_beta | dB | 4 |  |  |  |  | 4 |  |  |  |  |
| PDCCH_DMRS_beta | dB | 4 |  |  |  |  | 4 |  |  |  |  |
| PBCH_beta | dB | 0 |  |  |  |  | 0 |  |  |  |  |
| PSS_beta | dB |  |  |  |  |  |  |  |  |  |  |
| SSS_beta | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_beta | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_betaNote 1 | dB |  |  |  |  |  |  |  |  |  |  |
| SNR Note 8,9 | dB | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | dBm/15 kHz | [-98] |  |  |  |  | [-98] |  |  |  |  |
| Propagation condition |  | [AWGN] |  |  |  |  | [AWGN] |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CS-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure TBD. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 is modified as specified in section TBD. |  |  |  |  |  |  |  |  |  |  |  |

Table 8: Measurement gap configuration for NR FDD in-sync tests in non-DRX mode.

| Field | Test 2-A | Test 2-B | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| gapOffset | [TBD] | [TBD] | As specified in clause 6.3.2 in TS 38.331. SSB for RLM shall partially overlap with measurement gap. |

Figure 2: SNR variation for in-sync testing.

Test Requirements:

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (T6 second after the start of time duration T5) the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

The rate of correct events observed during repeated tests shall be at least 90%.

2.3                SA NR TDD FR1 OOS testing in non-DRX mode

One SS/PBCH per SSB burst is configured for cell 1. In Tables 9-11, we provide the test parameters for TDD FR1 out-of-sync testing in non-DRX mode. Compared to section 2.1 (FDD FR1 OOS testing in non-DRX mode), we only highlight the differences.

Table 9: General test parameters for NR FR1 TDD out-of-sync testing in non-DRX mode

| Parameter |  | Unit | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1-A | Test 1-B | Test 2-A | Test 2-B |  |
| PDCCH parameters |  |  | [R. TBD TDD] | [R. TBD TDD] | [R. TBD TDD] | [R. TBD TDD] | As specified in Table TBD. None of the PDCCH are intended for the UE under test |
| SSB Periodicity |  | ms | [20] | [20] | [20] | [20] | One SS/PBCH per SSB burst |
| OCNG parameters |  |  | TBD | TBD | TBD | TBD |  |
| Active cell |  |  | Cell 1 | Cell 1 | Cell 1 | Cell 1 | Cell 1 is on NR RF channel number 1 |
| CP length |  |  | Normal | Normal | Normal | Normal |  |
| NR RF Channel Number |  |  | 1 | 1 | 1 | 1 | One NR TDD FR1 carrier frequency is used. |
| NR Channel Bandwidth (BWchannel) |  | MHz | [10] | [40] | [10] | [40] |  |
| Subcarrier Spacing |  | KHz | [15] | [30] | [15] | [30] |  |
| Correlation Matrix and Antenna Configuration |  |  | [1x2 Low] | [1x2 Low] | [1x2 Low] | [1x2 Low] |  |
| Out of sync transmission parameters (Note 3) | DCI format |  | 1-0 | 1-0 | 1-0 | 1-0 | As defined in clause 7.3.1.2.1 in TS 38.212 |
|  | Number of Control OFDM symbols |  | 2 | 2 | 2 | 2 | Out of sync threshold Qout and the corresponding hypothetical PDCCH transmission parameters are as specified in clause 8.1.2 and Table 8.1.2.1-1 respectively. |
|  | Aggregation level | CCE | 8 | 8 | 8 | 8 |  |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 4 | 4 | 4 | 4 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 4 | 4 | 4 | 4 |  |
|  | DMRS precoder granularity |  | REG bundle size | REG bundle size | REG bundle size | REG bundle size |  |
|  | REG bundle size |  | 6 | 6 | 6 | 6 |  |
| DRX |  |  | OFF | OFF | OFF | OFF |  |
| Gap pattern ID |  |  | [N.A.] | [N.A.] | *[gp0] | *[gp0] | *RLM RS is partially overlapped with measurement gap when measurement gap is configured. P = 2 in evaluation period |
| Layer 3 filtering |  |  | Enabled | Enabled | Enabled | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 0 | 0 | 0 | 0 | T310 is disabled |
| T311 timer |  | ms | 1000 | 1000 | 1000 | 1000 | T311 is enabled |
| Number of NZP CSI-RS resource within the NZP CSI-RS resource set for beam management |  |  | TBD | TBD | TBD | TBD | For BM, repetition is ON and NZP CSI-RS resources are QCL-TypeD with SSB for RLM |
| NZP CSI-RS periodicity |  | slot | [5] | [10] | [5] | [10] |  |
| Periodic CSI reporting |  |  | PUCCH | PUCCH | PUCCH | PUCCH | reportQuantity is set as cri-RSRP |
| CSI reporting periodicity |  | slot | [5] | [10] | [5] | [10] | FDD and TDD can have the same reporting behavior |
| Propagation channel |  |  | [AWGN] | [AWGN] | [AWGN] | [AWGN] |  |
| T1 |  | s | 1 | 1 | 1 | 1 |  |
| T2 |  | s | 0.4 | 0.4 | 0.4 | 0.4 |  |
| T3 |  | s | [0.6] | [0.6] | [0.6] | [0.6] |  |
| T4 |  | s | [0.24] | [0.24] | [0.44] | [0.44] |  |
| Note 1: PDCCH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel. Note 2: SSB and DL data have the same numerology. Note 3: preambleTransMax value of [200], periodicity of PRACH occasion of [160 ms] and ra-ResponseWindow of [sl2] are configured for beam failure recovery. |  |  |  |  |  |  |  |

Table 10: Cell specific test parameters for NR TDD (cell # 1) for out-of-sync radio link monitoring tests #1-A, #1-B, # 2-A and #2-B in non-DRX mode.

| Parameter | Unit | Test 1-A |  |  | Test 1-B |  |  | Test 2-A |  |  | Test 2-B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| NR RF Channel Number |  | 1 |  |  | 1 |  |  | 1 |  |  | 1 |  |  |
| BWchannel | MHz | [10] |  |  | [40] |  |  | [10] |  |  | [40] |  |  |
| Subcarrier Spacing | KHz | [15] |  |  | [30] |  |  | [15] |  |  | [30] |  |  |
| Antenna Configuration |  | [1x2 low] |  |  | [1x2 low] |  |  | [1x2 low] |  |  | [1x2 low] |  |  |
| OCNG Pattern |  | TBD |  |  | TBD |  |  | TBD |  |  | TBD |  |  |
| PDCCH_beta | dB | 4 |  |  | 4 |  |  | 4 |  |  | 4 |  |  |
| PDCCH_DMRS_beta | dB | 4 |  |  | 4 |  |  | 4 |  |  | 4 |  |  |
| PBCH_beta | dB | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  |
| PSS_beta | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_beta | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_beta | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_betaNote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| SNR Note 8,9 | dB | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | dBm/15 kHz | [-98] |  |  | [-98] |  |  | [-98] |  |  | [-98] |  |  |
| Propagation condition |  | [AWGN] |  |  | [AWGN] |  |  | [AWGN] |  |  | [AWGN] |  |  |
| Note 1: tdd-UL-DL-configuration is TBD. Note 2: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: NZP CS-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 5: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 6: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 7: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 8: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 9: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure TBD. Note 10: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is TBD. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 11: Measurement gap configuration for NR TDD out-of-sync tests in non-DRX mode.

| Field | Test 2-A | Test 2-B | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| gapOffset | [0] | [0] | As specified in clause 6.3.2 in TS 38.331. SSB for RLM shall partially overlap with measurement gap. |

Figure 3: SNR variation for out-of-sync testing.

Test Requirements:

The UE behavior in each test during time durations T1, T2 and T3 shall be as follows:

During the period from time point A to time point B the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

The UE shall stop transmitting uplink signal no later than time point C (T4 second after the start of the time duration T3).

The rate of correct events observed during repeated tests shall be at least 90%.

2.4                SA NR TDD FR1 INS testing in non-DRX mode

One SS/PBCH per SSB burst is configured for cell 1. In Tables 12-15, we provide the test parameters for TDD FR1 in-sync testing in non-DRX mode. Compared to section 2.2 (FR1 FDD INS testing in non-DRX mode), we only highlight the differences.

Table 12: General test parameters for NR FR1 TDD in-sync testing in non-DRX mode

| Parameter |  | Unit | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1-A | Test 1-B | Test 2-A | Test 2-B |  |
| PDCCH parameters |  |  | [R. TBD TDD] | [R. TBD TDD] | [R. TBD TDD] | [R. TBD TDD] | As specified in Table TBD. None of the PDCCH are intended for the UE under test |
| SSB Periodicity |  | ms | [20] | [20] | [20] | [20] | One SS/PBCH per SSB burst |
| OCNG parameters |  |  | TBD | TBD | TBD | TBD |  |
| Active cell |  |  | Cell 1 | Cell 1 | Cell 1 | Cell 1 | Cell 1 is on NR RF channel number 1 |
| CP length |  |  | Normal | Normal | Normal | Normal |  |
| NR RF Channel Number |  |  | 1 | 1 | 1 | 1 | One NR TDD FR1 carrier frequency is used. |
| NR Channel Bandwidth (BWchannel) |  | MHz | [10] | [40] | [10] | [40] |  |
| Subcarrier Spacing |  | KHz | [15] | [30] | [15] | [30] |  |
| Correlation Matrix and Antenna Configuration |  |  | [1x2 Low] | [1x2 Low] | [1x2 Low] | [1x2 Low] |  |
| In sync transmission parameters (Note 3) | DCI format |  | 1-0 | 1-0 | 1-0 | 1-0 | As defined in clause 7.3.1.2.1 in TS 38.212 |
|  | Number of Control OFDM symbols |  | 2 | 2 | 2 | 2 | In sync threshold Qin and the corresponding hypothetical PDCCH transmission parameters are as specified in clause 8.1.2 and Table 8.1.2.1-2 respectively. |
|  | Aggregation level | CCE | 4 | 4 | 4 | 4 |  |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 0 | 0 | 0 | 0 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 0 | 0 | 0 | 0 |  |
|  | DMRS precoder granularity |  | REG bundle size | REG bundle size | REG bundle size | REG bundle size |  |
|  | REG bundle size |  | 6 | 6 | 6 | 6 |  |
| Out of sync transmission parameters (Note 3) | DCI format |  | 1-0 | 1-0 | 1-0 | 1-0 | As defined in clause 7.3.1.2.1 in TS 38.212 |
|  | Number of Control OFDM symbols |  | 2 | 2 | 2 | 2 | Out of sync threshold Qout and the corresponding hypothetical PDCCH transmission parameters are as specified in clause 8.1.2 and Table 8.1.2.1-1 respectively. |
|  | Aggregation level | CCE | 8 | 8 | 8 | 8 |  |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 4 | 4 | 4 | 4 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 4 | 4 | 4 | 4 |  |
|  | DMRS precoder granularity |  | REG bundle size | REG bundle size | REG bundle size | REG bundle size |  |
|  | REG bundle size |  | 6 | 6 | 6 | 6 |  |
| DRX |  |  | OFF | OFF | OFF | OFF |  |
| Gap pattern ID |  |  | [N.A.] | [N.A.] | *[gp0] | *[gp0] | *RLM RS is partially overlapped with measurement gap when measurement gap is configured. P = 2 in evaluation period |
| Layer 3 filtering |  |  | Enabled | Enabled | Enabled | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 2000 | 2000 | 2000 | 2000 | T310 is enabled |
| T311 timer |  | ms | 1000 | 1000 | 1000 | 1000 | T311 is enabled |
| Number of NZP CSI-RS resource within the NZP CSI-RS resource set for beam management |  |  | TBD | TBD | TBD | TBD | For BM, repetition is ON and NZP CSI-RS resources are QCL-TypeD with SSB for RLM |
| NZP CSI-RS periodicity |  | slot | [5] | [10] | [5] | [10] |  |
| Periodic CSI reporting |  |  | PUCCH | PUCCH | PUCCH | PUCCH | reportQuantity is set as cri-RSRP |
| CSI reporting periodicity |  | slot | [5] | [10] | [5] | [10] | FDD and TDD can have the same reporting behavior |
| Propagation channel |  |  | [AWGN] | [AWGN] | [AWGN] | [AWGN] |  |
| T1 |  | s | 0.5 | 0.5 | 0.5 | 0.5 |  |
| T2 |  | s | 0.4 | 0.4 | 0.4 | 0.4 |  |
| T3 |  | s | [1.46] | [1.46] | [1.36] | [1.36] | T310 >= T3+T4+TEvaluate_in + SSB burst duration |
| T4 |  | s | 0.4 | 0.4 | 0.4 | 0.4 |  |
| T5 |  | s | [1] | [1] | [1.1] | [1.1] | Same overall testing time |
| T6 |  | S | [0.42] | [0.42] | [0.72] | [0.72] | T6 > TEvaluate_out + T310 + 40 – T3 –T4 |
| Note 1: PDCCH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel. Note 2: SSB and DL data have the same numerology. Note 3: preambleTransMax value of [200], periodicity of PRACH occasion of [160 ms] and ra-ResponseWindow of [sl2] are configured for beam failure recovery. |  |  |  |  |  |  |  |

Table 13: Cell specific test parameters for NR TDD (cell # 1) for in-sync radio link monitoring tests #1-A, and #1-B in non-DRX mode.

| Parameter | Unit | Test 1-A |  |  |  |  | Test 1-B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 | T1 | T2 | T3 | T4 | T5 |
| NR RF Channel Number |  | 1 |  |  |  |  | 1 |  |  |  |  |
| BWchannel | MHz | [10] |  |  |  |  | [40] |  |  |  |  |
| Subcarrier Spacing | KHz | [15] |  |  |  |  | [30] |  |  |  |  |
| Antenna Configuration |  | [1x2 low] |  |  |  |  | [1x2 low] |  |  |  |  |
| OCNG Pattern |  | TBD |  |  |  |  | TBD |  |  |  |  |
| PDCCH_beta | dB | 4 |  |  |  |  | 4 |  |  |  |  |
| PDCCH_DMRS_beta | dB | 4 |  |  |  |  | 4 |  |  |  |  |
| PBCH_beta | dB | 0 |  |  |  |  | 0 |  |  |  |  |
| PSS_beta | dB |  |  |  |  |  |  |  |  |  |  |
| SSS_beta | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_beta | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_betaNote 1 | dB |  |  |  |  |  |  |  |  |  |  |
| SNR Note 8,9 | dB | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | dBm/15 kHz | [-98] |  |  |  |  | [-98] |  |  |  |  |
| Propagation condition |  | [AWGN] |  |  |  |  | [AWGN] |  |  |  |  |
| Note 1: tdd-UL-DL-configuration is TBD. Note 2: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: NZP CS-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 5: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 6: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 7: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 8: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 9: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure TBD. Note 10: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 is modified as specified in section TBD. |  |  |  |  |  |  |  |  |  |  |  |

Table 14: Cell specific test parameters for NR TDD (cell # 1) for in-sync radio link monitoring tests #2-A and #2-B in non-DRX mode.

| Parameter | Unit | Test 2-A |  |  |  |  | Test 2-B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 | T1 | T2 | T3 | T4 | T5 |
| NR RF Channel Number |  | 1 |  |  |  |  | 1 |  |  |  |  |
| BWchannel | MHz | [10] |  |  |  |  | [40] |  |  |  |  |
| Subcarrier Spacing | KHz | [15] |  |  |  |  | [30] |  |  |  |  |
| Antenna Configuration |  | [1x2 low] |  |  |  |  | [1x2 low] |  |  |  |  |
| OCNG Pattern |  | TBD |  |  |  |  | TBD |  |  |  |  |
| PDCCH_beta | dB | 4 |  |  |  |  | 4 |  |  |  |  |
| PDCCH_DMRS_beta | dB | 4 |  |  |  |  | 4 |  |  |  |  |
| PBCH_beta | dB | 0 |  |  |  |  | 0 |  |  |  |  |
| PSS_beta | dB |  |  |  |  |  |  |  |  |  |  |
| SSS_beta | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_beta | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_betaNote 1 | dB |  |  |  |  |  |  |  |  |  |  |
| SNR Note 8,9 | dB | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | dBm/15 kHz | [-98] |  |  |  |  | [-98] |  |  |  |  |
| Propagation condition |  | [AWGN] |  |  |  |  | [AWGN] |  |  |  |  |
| Note 1: tdd-UL-DL-configuration is TBD. Note 2: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: NZP CS-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 5: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 6: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 7: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 8: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 9: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure TBD. Note 10: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 is modified as specified in section TBD. |  |  |  |  |  |  |  |  |  |  |  |

Table 15: Measurement gap configuration for NR TDD in-sync tests in non-DRX mode.

| Field | Test 2-A | Test 2-B | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| gapOffset | [TBD] | [TBD] | As specified in clause 6.3.2 in TS 38.331. SSB for RLM shall partially overlap with measurement gap. |

Figure 4: SNR variation for in-sync testing.

Test Requirements:

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (T6 second after the start of time duration T5) the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

The rate of correct events observed during repeated tests shall be at least 90%.

2.5                SA NR TDD FR2 OOS testing in non-DRX mode

One SSB per SSB burst is configured for cell 1. In Tables 16-18, we provide the test parameters for TDD FR2 out-of-sync testing in non-DRX mode. Compared to section 2.3 (FR1 TDD OOS testing in non-DRX mode), we only highlight the differences.

Table 16: General test parameters for NR FR2 TDD out-of-sync testing in non-DRX mode

| Parameter |  | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| PDCCH parameters |  |  | [R. TBD TDD] | [R. TBD TDD] | As specified in Table TBD. None of the PDCCH are intended for the UE under test |
| SSB Periodicity |  | ms | [20] | [20] | One SSB per SSB burst |
| OCNG parameters |  |  | TBD | TBD |  |
| Active cell |  |  | Cell 1 | Cell 1 | Cell 1 is on NR RF channel number 1 |
| CP length |  |  | Normal | Normal |  |
| NR RF Channel Number |  |  | 1 | 1 | One NR TDD FR2 carrier frequency is used. |
| NR Channel Bandwidth (BWchannel) |  | MHz | [100] | [100] |  |
| Subcarrier Spacing |  | KHz | [120] | [120] |  |
| Correlation Matrix and Antenna Configuration |  |  | [1x2 Low] | [1x2 Low] |  |
| Out of sync transmission parameters (Note 3) | DCI format |  | 1-0 | 1-0 | As defined in clause 7.3.1.2.1 in TS 38.212 |
|  | Number of Control OFDM symbols |  | 2 | 2 | Out of sync threshold Qout and the corresponding hypothetical PDCCH transmission parameters are as specified in clause 8.1.2 and Table 8.1.2.1-1 respectively. |
|  | Aggregation level | CCE | 8 | 8 |  |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 4 | 4 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 4 | 4 |  |
|  | DMRS precoder granularity |  | REG bundle size | REG bundle size |  |
|  | REG bundle size |  | 6 | 6 |  |
| DRX |  |  | OFF | OFF |  |
| Gap pattern ID |  |  | [N.A.] | *[gp0] | *RLM RS is partially overlapped with measurement gap when measurement gap is configured. P = 2 in evaluation period |
| Layer 3 filtering |  |  | Enabled | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 0 | 0 | T310 is disabled |
| T311 timer |  | ms | 1000 | 1000 | T311 is enabled |
| Number of NZP CSI-RS resource within the NZP CSI-RS resource set 1 for beam management |  |  | TBD | TBD | For BM, repetition is ON and NZP CSI-RS resources are QCL-TypeD with SSB for RLM |
| NZP CSI-RS periodicity in NZP CSI-RS resource set 1 |  | slot | [40] | [40] |  |
| Periodic CSI reporting |  |  | PUCCH | PUCCH | reportQuantity is set as cri-RSRP |
| CSI reporting periodicity |  | slot | [40] | [40] | FDD and TDD can have the same reporting behavior |
| Propagation channel |  |  | [AWGN] | [AWGN] |  |
| T1 |  | s | 1 | 1 |  |
| T2 |  | s | 0.4 | 0.4 |  |
| T3 |  | s | [0.6] | [0.6] |  |
| T4 |  | s | [0.24] | [0.44] |  |
| Note 1: PDCCH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel. Note 2: SSB and DL data have the same numerology. Note 3: preambleTransMax value of [200], periodicity of PRACH occasion of [160 ms] and ra-ResponseWindow of [sl40] are configured for beam failure recovery. |  |  |  |  |  |

Table 17: Cell specific test parameters for NR TDD (cell # 1) for out-of-sync radio link monitoring tests #1 and #2 in non-DRX mode.

| Parameter | Unit | Test 1 |  |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| NR RF Channel Number |  | 1 |  |  | 1 |  |  |
| BWchannel | MHz | [100] |  |  | [100] |  |  |
| Subcarrier Spacing | KHz | [120] |  |  | [120] |  |  |
| Antenna Configuration |  | [1x2 low] |  |  | [1x2 low] |  |  |
| OCNG Pattern |  | TBD |  |  | TBD |  |  |
| PDCCH_beta | dB | 4 |  |  | 4 |  |  |
| PDCCH_DMRS_beta | dB | 4 |  |  | 4 |  |  |
| PBCH_beta | dB | 0 |  |  | 0 |  |  |
| PSS_beta | dB |  |  |  |  |  |  |
| SSS_beta | dB |  |  |  |  |  |  |
| PDSCH_beta | dB |  |  |  |  |  |  |
| OCNG_betaNote 1 | dB |  |  |  |  |  |  |
| SNR Note 8,9 | dB | TBD | TBD | TBD | TBD | TBD | TBD |
|  | dBm/15 kHz | [-98] |  |  | [-98] |  |  |
| Propagation condition |  | [AWGN] |  |  | [AWGN] |  |  |
| Note 1: tdd-UL-DL-configuration is TBD. Note 2: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: NZP CS-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 5: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 6: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 7: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 8: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 9: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure TBD. Note 10: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is TBD. |  |  |  |  |  |  |  |  |

Table 18: Measurement gap configuration for NR TDD out-of-sync tests in non-DRX mode.

| Field | Test 2 | Comment |
| --- | --- | --- |
|  | Value |  |
| gapOffset | [0] | As specified in clause 6.3.2 in TS 38.331. SSB for RLM shall partially overlap with measurement gap. |

Figure 5: SNR variation for out-of-sync testing.

Test Requirements:

The UE behavior in each test during time durations T1, T2 and T3 shall be as follows:

During the period from time point A to time point B the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

The UE shall stop transmitting uplink signal no later than time point C (T4 second after the start of the time duration T3).

The rate of correct events observed during repeated tests shall be at least 90%.

2.6                SA NR TDD FR2 INS testing in non-DRX mode

One SSB per SSB burst is configured for cell 1. In Tables 19-21, we provide the test parameters for TDD FR2 in-sync testing in non-DRX mode. Compared to section 2.4 (FR1 TDD INS testing in non-DRX mode), we only highlight the differences.

Table 19: General test parameters for NR FR2 TDD in-sync testing in non-DRX mode

| Parameter |  | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| PDCCH parameters |  |  | [R. TBD TDD] | [R. TBD TDD] | As specified in Table TBD. None of the PDCCH are intended for the UE under test |
| SSB Periodicity |  | ms | [20] | [20] | One SSB per SSB burst |
| OCNG parameters |  |  | TBD | TBD |  |
| Active cell |  |  | Cell 1 | Cell 1 | Cell 1 is on NR RF channel number 1 |
| CP length |  |  | Normal | Normal |  |
| NR RF Channel Number |  |  | 1 | 1 | One NR TDD FR2 carrier frequency is used. |
| NR Channel Bandwidth (BWchannel) |  | MHz | [100] | [100] |  |
| Subcarrier Spacing |  | KHz | [120] | [120] |  |
| Correlation Matrix and Antenna Configuration |  |  | [1x2 Low] | [1x2 Low] |  |
| In sync transmission parameters (Note 3) | DCI format |  | 1-0 | 1-0 | As defined in clause 7.3.1.2.1 in TS 38.212 |
|  | Number of Control OFDM symbols |  | 2 | 2 | In sync threshold Qin and the corresponding hypothetical PDCCH transmission parameters are as specified in clause 8.1.2 and Table 8.1.2.1-2 respectively. |
|  | Aggregation level | CCE | 4 | 4 |  |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 0 | 0 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 0 | 0 |  |
|  | DMRS precoder granularity |  | REG bundle size | REG bundle size |  |
|  | REG bundle size |  | 6 | 6 |  |
| Out of sync transmission parameters (Note 3) | DCI format |  | 1-0 | 1-0 | As defined in clause 7.3.1.2.1 in TS 38.212 |
|  | Number of Control OFDM symbols |  | 2 | 2 | Out of sync threshold Qout and the corresponding hypothetical PDCCH transmission parameters are as specified in clause 8.1.2 and Table 8.1.2.1-1 respectively. |
|  | Aggregation level | CCE | 8 | 8 |  |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 4 | 4 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 4 | 4 |  |
|  | DMRS precoder granularity |  | REG bundle size | REG bundle size |  |
|  | REG bundle size |  | 6 | 6 |  |
| DRX |  |  | OFF | OFF |  |
| Gap pattern ID |  |  | [N.A.] | *[gp0] | *RLM RS is partially overlapped with measurement gap when measurement gap is configured. P = 2 in evaluation period |
| Layer 3 filtering |  |  | Enabled | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 2000 | 2000 | T310 is enabled |
| T311 timer |  | ms | 1000 | 1000 | T311 is enabled |
| Number of NZP CSI-RS resource within the NZP CSI-RS resource set 1 for beam management |  |  | TBD | TBD | For BM, repetition is ON and NZP CSI-RS resources are QCL-TypeD with SSB for RLM |
| NZP CSI-RS periodicity in NZP CSI-RS resource set 1 |  | slot | [40] | [40] |  |
| Periodic CSI reporting |  |  | PUCCH | PUCCH | reportQuantity is set as cri-RSRP |
| CSI reporting periodicity |  | slot | [40] | [40] | FDD and TDD can have the same reporting behavior |
| Propagation channel |  |  | [AWGN] | [AWGN] |  |
| T1 |  | s | 0.5 | 0.5 |  |
| T2 |  | s | 0.4 | 0.4 |  |
| T3 |  | s | [1.46] | [1.36] | T310 >= T3+T4+TEvaluate_in + SSB burst duration |
| T4 |  | s | 0.4 | 0.4 |  |
| T5 |  | s | [1] | [1.1] | Same overall testing time |
| T6 |  | s | [0.42] | [0.72] | T6 > TEvaluate_out + T310 + 40 – T3 –T4 |
| Note 1: PDCCH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel. Note 2: SSB and DL data have the same numerology. Note 3: preambleTransMax value of [200], periodicity of PRACH occasion of [160 ms] and ra-ResponseWindow of [sl40] are configured for beam failure recovery. |  |  |  |  |  |

Table 20: Cell specific test parameters for NR TDD (cell # 1) for in-sync radio link monitoring tests #1 and test #2 in non-DRX mode.

| Parameter | Unit | Test 1 |  |  |  |  | Test 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 | T1 | T2 | T3 | T4 | T5 |
| NR RF Channel Number |  | 1 |  |  |  |  | 1 |  |  |  |  |
| BWchannel | MHz | [100] |  |  |  |  | [100] |  |  |  |  |
| Subcarrier Spacing | KHz | [120] |  |  |  |  | [120] |  |  |  |  |
| Antenna Configuration |  | [1x2 low] |  |  |  |  | [1x2 low] |  |  |  |  |
| OCNG Pattern |  | TBD |  |  |  |  | TBD |  |  |  |  |
| PDCCH_beta | dB | 4 |  |  |  |  | 4 |  |  |  |  |
| PDCCH_DMRS_beta | dB | 4 |  |  |  |  | 4 |  |  |  |  |
| PBCH_beta | dB | 0 |  |  |  |  | 0 |  |  |  |  |
| PSS_beta | dB |  |  |  |  |  |  |  |  |  |  |
| SSS_beta | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_beta | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_betaNote 1 | dB |  |  |  |  |  |  |  |  |  |  |
| SNR Note 8,9 | dB | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | dBm/15 kHz | [-98] |  |  |  |  | [-98] |  |  |  |  |
| Propagation condition |  | [AWGN] |  |  |  |  | [AWGN] |  |  |  |  |
| Note 1: tdd-UL-DL-configuration is TBD. Note 2: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: NZP CS-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 5: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 6: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 7: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 8: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 9: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure TBD. Note 10: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 is modified as specified in section TBD. |  |  |  |  |  |  |  |  |  |  |  |

Table 21: Measurement gap configuration for NR TDD in-sync tests in non-DRX mode.

| Field | Test 2 | Comment |
| --- | --- | --- |
|  | Value |  |
| gapOffset | [TBD] | As specified in clause 6.3.2 in TS 38.331. SSB for RLM shall partially overlap with measurement gap. |

Figure 6: SNR variation for in-sync testing.

Test Requirements:

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (T6 second after the start of time duration T5) the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

The rate of correct events observed during repeated tests shall be at least 90%.

2.7                SA NR FDD FR1 OOS testing in DRX mode

One SS/PBCH per SSB burst is configured for cell 1. In Tables 22-26, we provide the test parameters for FDD FR1 out-of-sync testing in DRX mode. Compared to section 2.1 (FDD FR1 OOS in non-DRX mode), we only highlight the differences.

Table 22: General test parameters for NR FR1 FDD out-of-sync testing in DRX mode

| Parameter |  | Unit | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1-A | Test 1-B | Test 2-A | Test 2-B |  |
| PDCCH parameters |  |  | [R. TBD FDD] | [R. TBD FDD] | [R. TBD FDD] | [R. TBD FDD] | As specified in Table TBD. None of the PDCCH are intended for the UE under test |
| SSB Periodicity |  | ms | [20] | [20] | [20] | [20] | One SS/PBCH per SSB burst |
| OCNG parameters |  |  | TBD | TBD | TBD | TBD |  |
| Active cell |  |  | Cell 1 | Cell 1 | Cell 1 | Cell 1 | Cell 1 is on NR RF channel number 1 |
| CP length |  |  | Normal | Normal | Normal | Normal |  |
| NR RF Channel Number |  |  | 1 | 1 | 1 | 1 | One NR FDD FR1 carrier frequency is used. |
| NR Channel Bandwidth (BWchannel) |  | MHz | [10] | [40] | [10] | [40] |  |
| Subcarrier Spacing |  | KHz | [15] | [30] | [15] | [30] |  |
| Correlation Matrix and Antenna Configuration |  |  | [1x2 Low] | [1x2 Low] | [1x2 Low] | [1x2 Low] |  |
| Out of sync transmission parameters (Note 3) | DCI format |  | 1-0 | 1-0 | 1-0 | 1-0 | As defined in clause 7.3.1.2.1 in TS 38.212 |
|  | Number of Control OFDM symbols |  | 2 | 2 | 2 | 2 | Out of sync threshold Qout and the corresponding hypothetical PDCCH transmission parameters are as specified in clause 8.1.2 and Table 8.1.2.1-1 respectively. |
|  | Aggregation level | CCE | 8 | 8 | 8 | 8 |  |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 4 | 4 | 4 | 4 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 4 | 4 | 4 | 4 |  |
|  | DMRS precoder granularity |  | REG bundle size | REG bundle size | REG bundle size | REG bundle size |  |
|  | REG bundle size |  | 6 | 6 | 6 | 6 |  |
| DRX cycle |  | ms | 1280 | 1280 | 40 | 40 | See Table TBD. |
| Gap pattern ID |  |  | [N.A.] | [N.A.] | *[gp0] | *[gp0] | *RLM RS is partially overlapped with measurement gap when measurement gap is configured. P = 2 in evaluation period |
| Layer 3 filtering |  |  | Enabled | Enabled | Enabled | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 0 | 0 | 0 | 0 | T310 is disabled |
| T311 timer |  | ms | 1000 | 1000 | 1000 | 1000 | T311 is enabled |
| Number of NZP CSI-RS resource within the NZP CSI-RS resource set for beam management |  |  | TBD | TBD | TBD | TBD | For BM, repetition is ON and NZP CSI-RS resources are QCL-TypeD with SSB for RLM |
| NZP CSI-RS periodicity |  | slot | [5] | [10] | [5] | [10] |  |
| Periodic CSI reporting |  |  | PUCCH | PUCCH | PUCCH | PUCCH | reportQuantity is set as cri-RSRP |
| CSI reporting periodicity |  | slot | [5] | [10] | [5] | [10] | FDD and TDD can have the same reporting behavior |
| Propagation channel |  |  | [AWGN] | [AWGN] | [AWGN] | [AWGN] |  |
| T1 |  | s | 32 | 32 | 4 | 4 |  |
| T2 |  | s | 12.8 | 12.8 | 1.6 | 1.6 |  |
| T3 |  | s | 13 | 13 | 1.8 | 1.8 |  |
| T4 |  | s | [12.9] | [12.9] | [1.3] | [1.3] | TEvaluate_out +100 ms |
| Note 1: PDCCH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel. Note 2: SSB and DL data have the same numerology. Note 3: preambleTransMax value of [200], periodicity of PRACH occasion of [160 ms] and ra-ResponseWindow of [sl2] are configured for beam failure recovery. |  |  |  |  |  |  |  |

Table 23: Cell specific test parameters for NR FDD (cell # 1) for out-of-sync radio link monitoring tests #1-A, #1-B, # 2-A and #2-B in DRX mode.

| Parameter | Unit | Test 1-A |  |  | Test 1-B |  |  | Test 2-A |  |  | Test 2-B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| NR RF Channel Number |  | 1 |  |  | 1 |  |  | 1 |  |  | 1 |  |  |
| BWchannel | MHz | [10] |  |  | [40] |  |  | [10] |  |  | [40] |  |  |
| Subcarrier Spacing | KHz | [15] |  |  | [30] |  |  | [15] |  |  | [30] |  |  |
| Antenna Configuration |  | [1x2 low] |  |  | [1x2 low] |  |  | [1x2 low] |  |  | [1x2 low] |  |  |
| OCNG Pattern |  | TBD |  |  | TBD |  |  | TBD |  |  | TBD |  |  |
| PDCCH_beta | dB | 4 |  |  | 4 |  |  | 4 |  |  | 4 |  |  |
| PDCCH_DMRS_beta | dB | 4 |  |  | 4 |  |  | 4 |  |  | 4 |  |  |
| PBCH_beta | dB | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  |
| PSS_beta | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_beta | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_beta | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_betaNote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| SNR Note 8,9 | dB | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | dBm/15 kHz | [-98] |  |  | [-98] |  |  | [-98] |  |  | [-98] |  |  |
| Propagation condition |  | [AWGN] |  |  | [AWGN] |  |  | [AWGN] |  |  | [AWGN] |  |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CS-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure TBD. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is TBD. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 24: Measurement gap configuration for NR FDD out-of-sync tests in DRX mode.

| Field | Test 2-A | Test 2-B | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| gapOffset | [0] | [0] | As specified in clause 6.3.2 in TS 38.331. SSB for RLM shall partially overlap with measurement gap. |

Table 25: DRX-Configuration for NR FDD out-of-sync tests

| Field | Test 1-A | Test 1-B | Test 2-A | Test 1-B | Comment |
| --- | --- | --- | --- | --- | --- |
|  | Value | Value | Value | Value |  |
| drx-onDurationTimer | [ms6] | [ms6] | [ms6] | [ms6] | As specified in section 6.3.2 in 3GPP TS 38.331 |
| drx-InactivityTimer | [ms1] | [ms1] | [ms1] | [ms1] |  |
| drx-RetransmissionTimerDL | [sl1] | [sl1] | [sl1] | [sl1] |  |
| drx-RetransmissionTimerUL | [sl1] | [sl1] | [sl1] | [sl1] |  |
| longDRX-CycleStartOffset | [ms1280] | [ms1280] | [ms40] | [ms40] |  |
| shortDRX | disable | disable | disable | disable |  |

Table 26: TimeAlignmentTimer -Configuration for NR FDD out-of-sync testing

| Field | Test 1-A | Test 1-B | Test 2-A | Test 2-B | Comment |
| --- | --- | --- | --- | --- | --- |
|  | Value | Value | Value | Value |  |
| TimeAlignmentTimer | [Infinity] | infinity | infinity | infinity | As specified in section 6.3.2 in 3GPP TS 38.331 |
| periodicityAndOffset in SchedulingRequestResourceConfig | [sl5] | [sl10] | [sl5] | [sl10] | For further information see section 6.3.2 in 3GPP TS 38.331 and section 9.2.2 in 3GPP TS 38.213 |

Figure 7: SNR variation for out-of-sync testing.

Test Requirements:

The UE behavior in each test during time durations T1, T2 and T3 shall be as follows:

During the period from time point A to time point B the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

The UE shall stop transmitting uplink signal no later than time point C (T4 second after the start of the time duration T3).

The rate of correct events observed during repeated tests shall be at least 90%.

2.8                SA NR FDD FR1 INS testing in DRX mode

One SS/PBCH per SSB burst is configured for cell 1. In Tables 27-30, we provide the test parameters for FDD FR1 in-sync testing in DRX mode. Compared to section 2.7 (FDD FR1 INS testing in non-DRX mode), we only highlight the differences.

Table 27: General test parameters for NR FR1 FDD in-sync testing in DRX mode

| Parameter |  | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1-A | Test 1-B |  |
| PDCCH parameters |  |  | [R. TBD FDD] | [R. TBD FDD] | As specified in Table TBD. None of the PDCCH are intended for the UE under test |
| SSB Periodicity |  | ms | [20] | [20] | One SS/PBCH per SSB burst |
| OCNG parameters |  |  | TBD | TBD |  |
| Active cell |  |  | Cell 1 | Cell 1 | Cell 1 is on NR RF channel number 1 |
| CP length |  |  | Normal | Normal |  |
| NR RF Channel Number |  |  | 1 | 1 | One NR FDD FR1 carrier frequency is used. |
| NR Channel Bandwidth (BWchannel) |  | MHz | [10] | [40] |  |
| Subcarrier Spacing |  | KHz | [15] | [30] |  |
| Correlation Matrix and Antenna Configuration |  |  | [1x2 Low] | [1x2 Low] |  |
| In sync transmission parameters (Note 3) | DCI format |  | 1-0 | 1-0 | As defined in clause 7.3.1.2.1 in TS 38.212 |
|  | Number of Control OFDM symbols |  | 2 | 2 | In sync threshold Qin and the corresponding hypothetical PDCCH transmission parameters are as specified in clause 8.1.2 and Table 8.1.2.1-2 respectively. |
|  | Aggregation level | CCE | 4 | 4 |  |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 0 | 0 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 0 | 0 |  |
|  | DMRS precoder granularity |  | REG bundle size | REG bundle size |  |
|  | REG bundle size |  | 6 | 6 |  |
| Out of sync transmission parameters (Note 3) | DCI format |  | 1-0 | 1-0 | As defined in clause 7.3.1.2.1 in TS 38.212 |
|  | Number of Control OFDM symbols |  | 2 | 2 | Out of sync threshold Qout and the corresponding hypothetical PDCCH transmission parameters are as specified in clause 8.1.2 and Table 8.1.2.1-1 respectively. |
|  | Aggregation level | CCE | 8 | 8 |  |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 4 | 4 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 4 | 4 |  |
|  | DMRS precoder granularity |  | REG bundle size | REG bundle size |  |
|  | REG bundle size |  | 6 | 6 |  |
| DRX cycle |  | ms | 40 | 40 |  |
| Gap pattern ID |  |  | [N.A.] | [N.A.] |  |
| Layer 3 filtering |  |  | Enabled | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 2000 | 2000 | T310 is enabled |
| T311 timer |  | ms | 1000 | 1000 | T311 is enabled |
| Number of NZP CSI-RS resource within the NZP CSI-RS resource set for beam management |  |  | TBD | TBD | For BM, repetition is ON and NZP CSI-RS resources are QCL-TypeD with SSB for RLM |
| NZP CSI-RS periodicity |  | slot | [5] | [10] |  |
| Periodic CSI reporting |  |  | PUCCH | PUCCH | reportQuantity is set as cri-RSRP |
| CSI reporting periodicity |  | slot | [5] | [10] | FDD and TDD can have the same reporting behavior |
| Propagation channel |  |  | [AWGN] | [AWGN] |  |
| T1 |  | s | 4 | 4 |  |
| T2 |  | s | 1.6 | 1.6 |  |
| T3 |  | s | [1.36] | [1.36] | T310 >= T3+T4+TEvaluate_in + SSB burst duration |
| T4 |  | s | 0.4 | 0.4 |  |
| T5 |  | s | 4 | 4 | Same overall testing time |
| T6 |  | s | [1] | [1] | T6 > TEvaluate_out + T310 + 40 – T3 –T4 |
| Note 1: PDCCH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel. Note 2: SSB and DL data have the same numerology. Note 3: preambleTransMax value of [200], periodicity of PRACH occasion of [160 ms] and ra-ResponseWindow of [sl2] are configured for beam failure recovery. |  |  |  |  |  |

Table 28: Cell specific test parameters for NR FDD (cell # 1) for in-sync radio link monitoring tests #1-A, and #1-B in DRX mode.

| Parameter | Unit | Test 1-A |  |  |  |  | Test 1-B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 | T1 | T2 | T3 | T4 | T5 |
| NR RF Channel Number |  | 1 |  |  |  |  | 1 |  |  |  |  |
| BWchannel | MHz | [10] |  |  |  |  | [40] |  |  |  |  |
| Subcarrier Spacing | KHz | [15] |  |  |  |  | [30] |  |  |  |  |
| Antenna Configuration |  | [1x2 low] |  |  |  |  | [1x2 low] |  |  |  |  |
| OCNG Pattern |  | TBD |  |  |  |  | TBD |  |  |  |  |
| PDCCH_beta | dB | 4 |  |  |  |  | 4 |  |  |  |  |
| PDCCH_DMRS_beta | dB | 4 |  |  |  |  | 4 |  |  |  |  |
| PBCH_beta | dB | 0 |  |  |  |  | 0 |  |  |  |  |
| PSS_beta | dB |  |  |  |  |  |  |  |  |  |  |
| SSS_beta | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_beta | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_betaNote 1 | dB |  |  |  |  |  |  |  |  |  |  |
| SNR Note 8,9 | dB | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | dBm/15 kHz | [-98] |  |  |  |  | [-98] |  |  |  |  |
| Propagation condition |  | [AWGN] |  |  |  |  | [AWGN] |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CS-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 5: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 6: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 7: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure TBD. Note 8: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 is modified as specified in section TBD. |  |  |  |  |  |  |  |  |  |  |  |

Table 29: DRX-Configuration for NR FDD out-of-sync tests

| Field | Test 1-A | Test 1-B | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| drx-onDurationTimer | [ms6] | [ms6] | As specified in section 6.3.2 in 3GPP TS 38.331 |
| drx-InactivityTimer | [ms1] | [ms1] |  |
| drx-RetransmissionTimerDL | [sl1] | [sl1] |  |
| drx-RetransmissionTimerUL | [sl1] | [sl1] |  |
| longDRX-CycleStartOffset | [ms40] | [ms40] |  |
| shortDRX | disable | disable |  |

Table 30: TimeAlignmentTimer -Configuration for NR FDD out-of-sync testing

| Field | Test 1-A | Test 1-B | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| TimeAlignmentTimer | infinity | infinity | As specified in section 6.3.2 in 3GPP TS 38.331 |
| periodicityAndOffset in SchedulingRequestResourceConfig | [sl5] | [sl10] | For further information see section 6.3.2 in 3GPP TS 38.331 and section 9.2.2 in 3GPP TS 38.213, |

Figure 8: SNR variation for in-sync testing.

Test Requirements:

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (T6 second after the start of time duration T5) the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

The rate of correct events observed during repeated tests shall be at least 90%.

2.9                SA NR TDD FR1 OOS testing in DRX mode

One SS/PBCH per SSB burst is configured for cell 1. In Tables 31-35, we provide the test parameters for TDD FR1 out-of-sync testing in DRX mode. Compared to section 2.7 (FDD FR1 OOS testing in DRX mode), we only highlight the differences.

Table 31: General test parameters for NR FR1 TDD out-of-sync testing in DRX mode

| Parameter |  | Unit | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1-A | Test 1-B | Test 2-A | Test 2-B |  |
| PDCCH parameters |  |  | [R. TBD TDD ] | [R. TBD TDD ] | [R. TBD TDD ] | [R. TBD TDD ] | As specified in Table TBD. None of the PDCCH are intended for the UE under test |
| SSB Periodicity |  | ms | [20] | [20] | [20] | [20] | One SS/PBCH per SSB burst |
| OCNG parameters |  |  | TBD | TBD | TBD | TBD |  |
| Active cell |  |  | Cell 1 | Cell 1 | Cell 1 | Cell 1 | Cell 1 is on NR RF channel number 1 |
| CP length |  |  | Normal | Normal | Normal | Normal |  |
| NR RF Channel Number |  |  | 1 | 1 | 1 | 1 | One NR TDD FR1 carrier frequency is used. |
| NR Channel Bandwidth (BWchannel) |  | MHz | [10] | [40] | [10] | [40] |  |
| Subcarrier Spacing |  | KHz | [15] | [30] | [15] | [30] |  |
| Correlation Matrix and Antenna Configuration |  |  | [1x2 Low] | [1x2 Low] | [1x2 Low] | [1x2 Low] |  |
| Out of sync transmission parameters (Note 3) | DCI format |  | 1-0 | 1-0 | 1-0 | 1-0 | As defined in clause 7.3.1.2.1 in TS 38.212 |
|  | Number of Control OFDM symbols |  | 2 | 2 | 2 | 2 | Out of sync threshold Qout and the corresponding hypothetical PDCCH transmission parameters are as specified in clause 8.1.2 and Table 8.1.2.1-1 respectively. |
|  | Aggregation level | CCE | 8 | 8 | 8 | 8 |  |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 4 | 4 | 4 | 4 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 4 | 4 | 4 | 4 |  |
|  | DMRS precoder granularity |  | REG bundle size | REG bundle size | REG bundle size | REG bundle size |  |
|  | REG bundle size |  | 6 | 6 | 6 | 6 |  |
| DRX cycle |  | ms | 1280 | 1280 | 40 | 40 | See Table TBD. |
| Gap pattern ID |  |  | [N.A.] | [N.A.] | *[gp0] | *[gp0] | *RLM RS is partially overlapped with measurement gap when measurement gap is configured. P = 2 in evaluation period |
| Layer 3 filtering |  |  | Enabled | Enabled | Enabled | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 0 | 0 | 0 | 0 | T310 is disabled |
| T311 timer |  | ms | 1000 | 1000 | 1000 | 1000 | T311 is enabled |
| Number of NZP CSI-RS resource within the NZP CSI-RS resource set for beam management |  |  | TBD | TBD | TBD | TBD | For BM, repetition is ON and NZP CSI-RS resources are QCL-TypeD with SSB for RLM |
| NZP CSI-RS periodicity |  | slot | [5] | [10] | [5] | [10] |  |
| Periodic CSI reporting |  |  | PUCCH | PUCCH | PUCCH | PUCCH | reportQuantity is set as cri-RSRP |
| CSI reporting periodicity |  | slot | [5] | [10] | [5] | [10] | FDD and TDD can have the same reporting behavior |
| Propagation channel |  |  | [AWGN] | [AWGN] | [AWGN] | [AWGN] |  |
| T1 |  | s | 32 | 32 | 4 | 4 |  |
| T2 |  | s | 12.8 | 12.8 | 1.6 | 1.6 |  |
| T3 |  | s | 13 | 13 | 1.8 | 1.8 |  |
| T4 |  | s | [12.9] | [12.9] | [1.3] | [1.3] | TEvaluate_out +100 ms |
| Note 1: PDCCH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel. Note 2: SSB and DL data have the same numerology. Note 3: preambleTransMax value of [200], periodicity of PRACH occasion of [160 ms] and ra-ResponseWindow of [sl2] are configured for beam failure recovery. |  |  |  |  |  |  |  |

Table 32: Cell specific test parameters for NR TDD (cell # 1) for out-of-sync radio link monitoring tests #1-A, #1-B, # 2-A and #2-B in DRX mode.

| Parameter | Unit | Test 1-A |  |  | Test 1-B |  |  | Test 2-A |  |  | Test 2-B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| NR RF Channel Number |  | 1 |  |  | 1 |  |  | 1 |  |  | 1 |  |  |
| BWchannel | MHz | [10] |  |  | [40] |  |  | [10] |  |  | [40] |  |  |
| Subcarrier Spacing | KHz | [15] |  |  | [30] |  |  | [15] |  |  | [30] |  |  |
| Antenna Configuration |  | [1x2 low] |  |  | [1x2 low] |  |  | [1x2 low] |  |  | [1x2 low] |  |  |
| OCNG Pattern |  | TBD |  |  | TBD |  |  | TBD |  |  | TBD |  |  |
| PDCCH_beta | dB | 4 |  |  | 4 |  |  | 4 |  |  | 4 |  |  |
| PDCCH_DMRS_beta | dB | 4 |  |  | 4 |  |  | 4 |  |  | 4 |  |  |
| PBCH_beta | dB | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  |
| PSS_beta | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_beta | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_beta | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_betaNote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| SNR Note 8,9 | dB | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | dBm/15 kHz | [-98] |  |  | [-98] |  |  | [-98] |  |  | [-98] |  |  |
| Propagation condition |  | [AWGN] |  |  | [AWGN] |  |  | [AWGN] |  |  | [AWGN] |  |  |
| Note 1: tdd-UL-DL-configuration is TBD. Note 2: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: NZP CS-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 5: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 6: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 7: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 8: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 9: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure TBD. Note 10: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is TBD. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 33: Measurement gap configuration for NR TDD out-of-sync tests in DRX mode.

| Field | Test 2-A | Test 2-B | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| gapOffset | [0] | [0] | As specified in clause 6.3.2 in TS 38.331. SSB for RLM shall partially overlap with measurement gap. |

Table 34: DRX-Configuration for NR TDD out-of-sync tests

| Field | Test 1-A | Test 1-B | Test 2-A | Test 1-B | Comment |
| --- | --- | --- | --- | --- | --- |
|  | Value | Value | Value | Value |  |
| drx-onDurationTimer | [ms6] | [ms6] | [ms6] | [ms6] | As specified in section 6.3.2 in 3GPP TS 38.331 |
| drx-InactivityTimer | [ms1] | [ms1] | [ms1] | [ms1] |  |
| drx-RetransmissionTimerDL | [sl1] | [sl1] | [sl1] | [sl1] |  |
| drx-RetransmissionTimerUL | [sl1] | [sl1] | [sl1] | [sl1] |  |
| longDRX-CycleStartOffset | [ms1280] | [ms1280] | [ms40] | [ms40] |  |
| shortDRX | disable | disable | disable | disable |  |

Table 35: TimeAlignmentTimer -Configuration for NR TDD out-of-sync testing

| Field | Test 1-A | Test 1-B | Test 2-A | Test 2-B | Comment |
| --- | --- | --- | --- | --- | --- |
|  | Value | Value | Value | Value |  |
| TimeAlignmentTimer | [Infinity] | infinity | infinity | infinity | As specified in section 6.3.2 in 3GPP TS 38.331 |
| periodicityAndOffset in SchedulingRequestResourceConfig | [sl5] | [sl10] | [sl5] | [sl10] | For further information see section 6.3.2 in 3GPP TS 38.331 and section 9.2.2 in 3GPP TS 38.213, |

Figure 9: SNR variation for out-of-sync testing.

Test Requirements:

The UE behavior in each test during time durations T1, T2 and T3 shall be as follows:

During the period from time point A to time point B the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

The UE shall stop transmitting uplink signal no later than time point C (T4 second after the start of the time duration T3).

The rate of correct events observed during repeated tests shall be at least 90%.

2.10             SA NR TDD FR1 INS testing in DRX mode

One SS/PBCH per SSB burst is configured for cell 1. In Tables 36-39, we provide the test parameters for TDD FR1 in-sync testing in DRX mode. Compared to section 2.8 (FDD FR1 INS testing in DRX mode), we only highlight the differences.

Table 36: General test parameters for NR FR1 TDD in-sync testing in DRX mode

| Parameter |  | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1-A | Test 1-B |  |
| PDCCH parameters |  |  | [R. TBD TDD] | [R. TBD TDD] | As specified in Table TBD. None of the PDCCH are intended for the UE under test |
| SSB Periodicity |  | ms | [20] | [20] | One SS/PBCH |
| OCNG parameters |  |  | TBD | TBD |  |
| Active cell |  |  | Cell 1 | Cell 1 | Cell 1 is on NR RF channel number 1 |
| CP length |  |  | Normal | Normal |  |
| NR RF Channel Number |  |  | 1 | 1 | One NR TDD FR1 carrier frequency is used. |
| NR Channel Bandwidth (BWchannel) |  | MHz | [10] | [40] |  |
| Subcarrier Spacing |  | KHz | [15] | [30] |  |
| Correlation Matrix and Antenna Configuration |  |  | [1x2 Low] | [1x2 Low] |  |
| In sync transmission parameters (Note 3) | DCI format |  | 1-0 | 1-0 | As defined in clause 7.3.1.2.1 in TS 38.212 |
|  | Number of Control OFDM symbols |  | 2 | 2 | In sync threshold Qin and the corresponding hypothetical PDCCH transmission parameters are as specified in clause 8.1.2 and Table 8.1.2.1-2 respectively. |
|  | Aggregation level | CCE | 4 | 4 |  |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 0 | 0 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 0 | 0 |  |
|  | DMRS precoder granularity |  | REG bundle size | REG bundle size |  |
|  | REG bundle size |  | 6 | 6 |  |
| Out of sync transmission parameters (Note 3) | DCI format |  | 1-0 | 1-0 | As defined in clause 7.3.1.2.1 in TS 38.212 |
|  | Number of Control OFDM symbols |  | 2 | 2 | Out of sync threshold Qout and the corresponding hypothetical PDCCH transmission parameters are as specified in clause 8.1.2 and Table 8.1.2.1-1 respectively. |
|  | Aggregation level | CCE | 8 | 8 |  |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 4 | 4 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 4 | 4 |  |
|  | DMRS precoder granularity |  | REG bundle size | REG bundle size |  |
|  | REG bundle size |  | 6 | 6 |  |
| DRX cycle |  | ms | 40 | 40 |  |
| Gap pattern ID |  |  | [N.A.] | [N.A.] |  |
| Layer 3 filtering |  |  | Enabled | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 2000 | 2000 | T310 is enabled |
| T311 timer |  | ms | 1000 | 1000 | T311 is enabled |
| Number of NZP CSI-RS resource within the NZP CSI-RS resource set for beam management |  |  | TBD | TBD | For BM, repetition is ON and NZP CSI-RS resources are QCL-TypeD with SSB for RLM |
| NZP CSI-RS periodicity |  | slot | [5] | [10] |  |
| Periodic CSI reporting |  |  | PUCCH | PUCCH | reportQuantity is set as cri-RSRP |
| CSI reporting periodicity |  | slot | [5] | [10] | FDD and TDD can have the same reporting behavior |
| Propagation channel |  |  | [AWGN] | [AWGN] |  |
| T1 |  | s | 4 | 4 |  |
| T2 |  | s | 1.6 | 1.6 |  |
| T3 |  | s | [1.36] | [1.36] | T310 >= T3+T4+TEvaluate_in + SSB burst duration |
| T4 |  | s | 0.4 | 0.4 |  |
| T5 |  | s | 4 | 4 | Same overall testing time |
| T6 |  | s | [1] | [1] | T6 > TEvaluate_out + T310 + 40 – T3 –T4 |
| Note 1: PDCCH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel. Note 2: SSB and DL data have the same numerology. Note 3: preambleTransMax value of [200], periodicity of PRACH occasion of [160 ms] and ra-ResponseWindow of [sl2] are configured for beam failure recovery. |  |  |  |  |  |

Table 37: Cell specific test parameters for NR TDD (cell # 1) for in-sync radio link monitoring tests #1-A, and #1-B in DRX mode.

| Parameter | Unit | Test 1-A |  |  |  |  | Test 1-B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 | T1 | T2 | T3 | T4 | T5 |
| NR RF Channel Number |  | 1 |  |  |  |  | 1 |  |  |  |  |
| BWchannel | MHz | [10] |  |  |  |  | [40] |  |  |  |  |
| Subcarrier Spacing | KHz | [15] |  |  |  |  | [30] |  |  |  |  |
| Antenna Configuration |  | [1x2 low] |  |  |  |  | [1x2 low] |  |  |  |  |
| OCNG Pattern |  | TBD |  |  |  |  | TBD |  |  |  |  |
| PDCCH_beta | dB | 4 |  |  |  |  | 4 |  |  |  |  |
| PDCCH_DMRS_beta | dB | 4 |  |  |  |  | 4 |  |  |  |  |
| PBCH_beta | dB | 0 |  |  |  |  | 0 |  |  |  |  |
| PSS_beta | dB |  |  |  |  |  |  |  |  |  |  |
| SSS_beta | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_beta | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_betaNote 1 | dB |  |  |  |  |  |  |  |  |  |  |
| SNR Note 8,9 | dB | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | dBm/15 kHz | [-98] |  |  |  |  | [-98] |  |  |  |  |
| Propagation condition |  | [AWGN] |  |  |  |  | [AWGN] |  |  |  |  |
| Note 1: tdd-UL-DL-configuration is TBD. Note 2: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: NZP CS-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure TBD. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 is modified as specified in section TBD. |  |  |  |  |  |  |  |  |  |  |  |

Table 38: DRX-Configuration for NR TDD out-of-sync tests

| Field | Test 1-A | Test 1-B | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| drx-onDurationTimer | [ms6] | [ms6] | As specified in section 6.3.2 in 3GPP TS 38.331 |
| drx-InactivityTimer | [ms1] | [ms1] |  |
| drx-RetransmissionTimerDL | [sl1] | [sl1] |  |
| drx-RetransmissionTimerUL | [sl1] | [sl1] |  |
| longDRX-CycleStartOffset | [ms40] | [ms40] |  |
| shortDRX | disable | disable |  |

Table 39: TimeAlignmentTimer -Configuration for NR TDD out-of-sync testing

| Field | Test 1-A | Test 1-B | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| TimeAlignmentTimer | infinity | infinity | As specified in section 6.3.2 in 3GPP TS 38.331 |
| periodicityAndOffset in SchedulingRequestResourceConfig | [sl5] | [sl10] | For further information see section 6.3.2 in 3GPP TS 38.331 and section 9.2.2 in 3GPP TS 38.213, |

Figure 10: SNR variation for in-sync testing.

Test Requirements:

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (T6 second after the start of time duration T5) the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

The rate of correct events observed during repeated tests shall be at least 90%.

2.11             SA NR TDD FR2 OOS testing in DRX mode

One SSB per SSB burst is configured for cell 1. In Tables 40-44, we provide the test parameters for TDD FR2 out-of-sync testing in DRX mode. Compared to section 2.9 (TDD FR1 OOS testing in DRX mode), we only highlight the differences.

Table 40: General test parameters for NR FR2 TDD out-of-sync testing in DRX mode

| Parameter |  | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| PDCCH parameters |  |  | [R. TBD TDD] | [R. TBD TDD] | As specified in Table TBD. None of the PDCCH are intended for the UE under test |
| SSB Periodicity |  | ms | [20] | [20] | One SSB per SSB burst |
| OCNG parameters |  |  | TBD | TBD |  |
| Active cell |  |  | Cell 1 | Cell 1 | Cell 1 is on NR RF channel number 1 |
| CP length |  |  | Normal | Normal |  |
| NR RF Channel Number |  |  | 1 | 1 | One NR TDD FR2 carrier frequency is used. |
| NR Channel Bandwidth (BWchannel) |  | MHz | [100] | [100] |  |
| Subcarrier Spacing |  | KHz | [120] | [120] |  |
| Correlation Matrix and Antenna Configuration |  |  | [1x2 Low] | [1x2 Low] |  |
| Out of sync transmission parameters (Note 3) | DCI format |  | 1-0 | 1-0 | As defined in clause 7.3.1.2.1 in TS 38.212 |
|  | Number of Control OFDM symbols |  | 2 | 2 | Out of sync threshold Qout and the corresponding hypothetical PDCCH transmission parameters are as specified in clause 8.1.2 and Table 8.1.2.1-1 respectively. |
|  | Aggregation level | CCE | 8 | 8 |  |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 4 | 4 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 4 | 4 |  |
|  | DMRS precoder granularity |  | REG bundle size | REG bundle size |  |
|  | REG bundle size |  | 6 | 6 |  |
| DRX cycle |  |  | 1280 | 40 | See Table TBD. |
| Gap pattern ID |  |  | [N.A.] | *[gp0] | *RLM RS is partially overlapped with measurement gap when measurement gap is configured. P = 2 in evaluation period |
| Layer 3 filtering |  |  | Enabled | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 0 | 0 | T310 is disabled |
| T311 timer |  | ms | 1000 | 1000 | T311 is enabled |
| Number of NZP CSI-RS resource within the NZP CSI-RS resource set 1 for beam management |  |  | TBD | TBD | For BM, repetition is ON and NZP CSI-RS resources are QCL-TypeD with SSB for RLM |
| NZP CSI-RS periodicity in NZP CSI-RS resource set 1 |  | slot | [40] | [40] |  |
| Periodic CSI reporting |  |  | PUCCH | PUCCH | reportQuantity is set as cri-RSRP |
| CSI reporting periodicity |  | slot | [40] | [40] | FDD and TDD can have the same reporting behavior |
| Propagation channel |  |  | [AWGN] | [AWGN] |  |
| T1 |  | s | 32 | 4 |  |
| T2 |  | s | 12.8 | 1.6 |  |
| T3 |  | s | 13 | 1.8 |  |
| T4 |  | s | [12.9] | [1.3] |  |
| Note 1: PDCCH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel. Note 2: SSB and DL data have the same numerology. Note 3: preambleTransMax value of [200], periodicity of PRACH occasion of [160 ms] and ra-ResponseWindow of [sl40] are configured for beam failure recovery. |  |  |  |  |  |

Table 41: Cell specific test parameters for NR TDD (cell # 1) for out-of-sync radio link monitoring tests #1, and #2 in DRX mode.

| Parameter | Unit | Test 1 |  |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| NR RF Channel Number |  | 1 |  |  | 1 |  |  |
| BWchannel | MHz | [100] |  |  | [100] |  |  |
| Subcarrier Spacing | KHz | [120] |  |  | [120] |  |  |
| Antenna Configuration |  | [1x2 low] |  |  | [1x2 low] |  |  |
| OCNG Pattern |  | TBD |  |  | TBD |  |  |
| PDCCH_beta | dB | 4 |  |  | 4 |  |  |
| PDCCH_DMRS_beta | dB | 4 |  |  | 4 |  |  |
| PBCH_beta | dB | 0 |  |  | 0 |  |  |
| PSS_beta | dB |  |  |  |  |  |  |
| SSS_beta | dB |  |  |  |  |  |  |
| PDSCH_beta | dB |  |  |  |  |  |  |
| OCNG_betaNote 1 | dB |  |  |  |  |  |  |
| SNR Note 8,9 | dB | TBD | TBD | TBD | TBD | TBD | TBD |
|  | dBm/15 kHz | [-98] |  |  | [-98] |  |  |
| Propagation condition |  | [AWGN] |  |  | [AWGN] |  |  |
| Note 1: tdd-UL-DL-configuration is TBD. Note 2: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: NZP CS-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 5: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 6: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 7: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 8: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 9: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure TBD. Note 10: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is TBD. |  |  |  |  |  |  |  |

Table 42: Measurement gap configuration for NR TDD out-of-sync tests in DRX mode.

| Field | Test 2 | Comment |
| --- | --- | --- |
|  | Value |  |
| gapOffset | [0] | As specified in clause 6.3.2 in TS 38.331. SSB for RLM shall partially overlap with measurement gap. |

Table 43: DRX-Configuration for NR FDD out-of-sync tests

| Field | Test 1 | Test 2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| drx-onDurationTimer | [ms6] | [ms6] | As specified in section 6.3.2 in 3GPP TS 38.331 |
| drx-InactivityTimer | [ms1] | [ms1] |  |
| drx-RetransmissionTimerDL | [sl1] | [sl1] |  |
| drx-RetransmissionTimerUL | [sl1] | [sl1] |  |
| longDRX-CycleStartOffset | [ms1280] | [ms40] |  |
| shortDRX | disable | disable |  |

Table 44: TimeAlignmentTimer -Configuration for NR FDD out-of-sync testing

| Field | Test1 | Test2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| TimeAlignmentTimer | [Infinity] | infinity | As specified in section 6.3.2 in 3GPP TS 38.331 |
| periodicityAndOffset in SchedulingRequestResourceConfig | [sl40] | [sl40] | For further information see section 6.3.2 in 3GPP TS 38.331 and section 9.2.2 in 3GPP TS 38.213, |

Figure 11: SNR variation for out-of-sync testing.

Test Requirements:

The UE behavior in each test during time durations T1, T2 and T3 shall be as follows:

During the period from time point A to time point B the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

The UE shall stop transmitting uplink signal no later than time point C (T4 second after the start of the time duration T3).

The rate of correct events observed during repeated tests shall be at least 90%.

2.12             SA NR TDD FR2 INS testing in DRX mode

One SSB per SSB burst is configured for cell 1. In Tables 45-48, we provide the test parameters for TDD FR2 in-sync testing in DRX mode. Compared to section 2.10 (TDD FR1 INS testing in DRX mode), we only highlight the differences.

Table 45: General test parameters for NR FR2 TDD in-sync testing in DRX mode

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
|  |  |  | Test 1 |  |
| PDCCH parameters |  |  | [R. TBD TDD] | As specified in Table TBD. None of the PDCCH are intended for the UE under test |
| SSB Periodicity |  | ms | [20] | One SSB per SSB burst |
| OCNG parameters |  |  | TBD |  |
| Active cell |  |  | Cell 1 | Cell 1 is on NR RF channel number 1 |
| CP length |  |  | Normal |  |
| NR RF Channel Number |  |  | 1 | One NR TDD FR2 carrier frequency is used. |
| NR Channel Bandwidth (BWchannel) |  | MHz | [100] |  |
| Subcarrier Spacing |  | KHz | [120] |  |
| Correlation Matrix and Antenna Configuration |  |  | [1x2 Low] |  |
| In sync transmission parameters (Note 3) | DCI format |  | 1-0 | As defined in clause 7.3.1.2.1 in TS 38.212 |
|  | Number of Control OFDM symbols |  | 2 | In sync threshold Qin and the corresponding hypothetical PDCCH transmission parameters are as specified in clause 8.1.2 and Table 8.1.2.1-2 respectively. |
|  | Aggregation level | CCE | 4 |  |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 0 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 0 |  |
|  | DMRS precoder granularity |  | REG bundle size |  |
|  | REG bundle size |  | 6 |  |
| Out of sync transmission parameters (Note 3) | DCI format |  | 1-0 | As defined in clause 7.3.1.2.1 in TS 38.212 |
|  | Number of Control OFDM symbols |  | 2 | Out of sync threshold Qout and the corresponding hypothetical PDCCH transmission parameters are as specified in clause 8.1.2 and Table 8.1.2.1-1 respectively. |
|  | Aggregation level | CCE | 8 |  |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 4 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 4 |  |
|  | DMRS precoder granularity |  | REG bundle size |  |
|  | REG bundle size |  | 6 |  |
| DRX cycle |  | ms | 40 |  |
| Gap pattern ID |  |  | [N.A.] |  |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 2000 | T310 is enabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| Number of NZP CSI-RS resource within the NZP CSI-RS resource set 1 for beam management |  |  | TBD | For BM, repetition is ON and NZP CSI-RS resources are QCL-TypeD with SSB for RLM |
| NZP CSI-RS periodicity in NZP CSI-RS resource set 1 |  | slot | [40] |  |
| Periodic CSI reporting |  |  | PUCCH | reportQuantity is set as cri-RSRP |
| CSI reporting periodicity |  | slot | [40] | FDD and TDD can have the same reporting behavior |
| Propagation channel |  |  | [AWGN] |  |
| T1 |  | s | 4 |  |
| T2 |  | s | 1.6 |  |
| T3 |  | s | [1.36] | T310 >= T3+T4+TEvaluate_in + SSB burst duration |
| T4 |  | s | 0.4 |  |
| T5 |  | s | 4 | Same overall testing time |
| T6 |  | s | [1] | T6 > TEvaluate_out + T310 + 40 – T3 –T4 |
| Note 1: PDCCH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel. Note 2: SSB and DL data have the same numerology. Note 3: preambleTransMax value of [200], periodicity of PRACH occasion of [160 ms] and ra-ResponseWindow of [sl40] are configured for beam failure recovery. |  |  |  |  |

Table 46: Cell specific test parameters for NR TDD (cell # 1) for in-sync radio link monitoring tests #1 in DRX mode.

| Parameter | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 |
| NR RF Channel Number |  | 1 |  |  |  |  |
| BWchannel | MHz | [100] |  |  |  |  |
| Subcarrier Spacing | KHz | [120] |  |  |  |  |
| Antenna Configuration |  | [1x2 low] |  |  |  |  |
| OCNG Pattern |  | TBD |  |  |  |  |
| PDCCH_beta | dB | 4 |  |  |  |  |
| PDCCH_DMRS_beta | dB | 4 |  |  |  |  |
| PBCH_beta | dB | 0 |  |  |  |  |
| PSS_beta | dB |  |  |  |  |  |
| SSS_beta | dB |  |  |  |  |  |
| PDSCH_beta | dB |  |  |  |  |  |
| OCNG_betaNote 1 | dB |  |  |  |  |  |
| SNR Note 8,9 | dB | TBD | TBD | TBD | TBD | TBD |
|  | dBm/15 kHz | [-98] |  |  |  |  |
| Propagation condition |  | [AWGN] |  |  |  |  |
| Note 1: tdd-UL-DL-configuration is TBD. Note 2: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: NZP CS-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure TBD. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 is modified as specified in section TBD. |  |  |  |  |  |  |

Table 47: DRX-Configuration for NR TDD out-of-sync tests

| Field | Test 1 | Comment |
| --- | --- | --- |
|  | Value |  |
| drx-onDurationTimer | [ms6] | As specified in section 6.3.2 in 3GPP TS 38.331 |
| drx-InactivityTimer | [ms1] |  |
| drx-RetransmissionTimerDL | [sl1] |  |
| drx-RetransmissionTimerUL | [sl1] |  |
| longDRX-CycleStartOffset | [ms40] |  |
| shortDRX | disable |  |

Table 48: TimeAlignmentTimer -Configuration for NR TDD out-of-sync testing

| Field | Test 1 | Comment |
| --- | --- | --- |
|  | Value |  |
| TimeAlignmentTimer | infinity | As specified in section 6.3.2 in 3GPP TS 38.331 |
| periodicityAndOffset in SchedulingRequestResourceConfig | [sl40] | For further information see section 6.3.2 in 3GPP TS 38.331 and section 9.2.2 in 3GPP TS 38.213, |

Figure 12: SNR variation for in-sync testing.

Test Requirements:

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (T6 second after the start of time duration T5) the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

The rate of correct events observed during repeated tests shall be at least 90%.

3	Conclusion

We have the following proposal:

Proposal 1: Adopt Table 2-48 for SA NR SSB based RLM test cases.