3GPP TSG-RAN4 Meeting #88bis 	R4-1812140

# Chengdu, China, 8 – 12 Oct, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on deriveSSB-IndexFromCell for intra-frequency measurement in FR1 TDD (section 9.2.5, section 9.2.6) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-9-12 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In RAN1 #93 meeting, it was agreed that: UE may assume that in TDD bands useServingCellTimingForSync is assumed to be set to TRUE. Now this IE name is changed to “deriveSSB-IndexFromCell”, and RAN4 has clarified the deriveSSB-IndexFromCell tolerance in section 7.7 in TS38.133. However in the intra-frequency requirement sections only the requirement of FR2 was clarified under this deriveSSB-IndexFromCell = True. The clarification on intra-frequency cell identification/measurement requirement for FR1 TDD is also needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify on intra-frequency cell identification/measurement requirement for FR1 TDD with deriveSSB-IndexFromCell = True. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The intra-frequency cell identification/measurement requirement for FR1 TDD is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.5; 9.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of Change 1

### 9.2.5	Intrafrequency measurements with no measurement gaps

#### 9.2.5.1	Intrafrequency cell identification

Editor’s Note :The requirements below have been derived without considering gap sharing when all SMTC occasion are fully overlapping with measurement gaps.

The UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_without_index if UE is not indicated to report SSB based RRM measurement result with the associated SSB index, or the UE has been indicated that the neighbour cell is synchronous with the serving cell. Otherwise UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_with_index. The UE shall be able to identify a new detectable intra frequency SS block of an already detected cell within Tidentify_intra_without_index. It is assumed that  deriveSSB-IndexFromCellis always enabled for FR1 TDD and FR2, and hence the identification time is always Tidentify_intra_without_index

Tidentify_intra_without_index = Kca (TPSS/SSS_sync_intra + T SSB_measurement_period_intra) ms

Tidentify_intra_with_index = Kca (TPSS/SSS_sync_intra + T SSB_measurement_period_intra + TSSB_time_index_intra) ms

Where:

TPSS/SSS_sync_intra: it is the time period used in PSS/SSS detection given in table 9.2.5.1-1, 9.2.5.1-2, 9.2.5.1-4 (deactivated Scell) or 9.2.5.1-5 (deactivated SCell)

TSSB_time_index_intra: it is the time period used to acquire the index of the SSB being measured given in table 9.2.5.1-3 or  9.2.5.1-6 (deactivated SCell)

T SSB_measurement_period_intra: equal to a measurement period of SSB based measurement given in table 9.2.5.2-1, table 9.2.5.2-2 table 9.2.5.2-3 (deactivated Scell) or 9.2.5.2-4(deactivated SCell)

Kca: For FR1, Kca =1 for measurements on frequencies corresponding to PCell or PSCell, and Kca =number of configured SCells for measurements on frequencies corresponding to FR1 only SCells

Editor’s note : Kca for SCells on FR1 assumes that all Scell SMTC are overlapping(definition FFS). Kca definition may be  revised for non overlapping Scell SMTCs.

Editor’s note:Kca is FFS if any FR2 serving cells are configured

Mpss/sss_sync_w/o_gaps : For a UE supporting power class 1(fixed wireless access), Mpss/sss_sync=40. For a UE supporting power class 2(vehicle mounted), Mpss/sss_sync_w/o_gaps =[24].  For a UE supporting power class 3(handheld), Mpss/sss_sync_w/o_gaps =[24]. For a UE supporting power class 4, Mpss/sss_sync_w/o_gaps =TBD

Mmeas_period_w/o_gaps : For a UE supporting power class 1 (fixed wireless access), Mmeas_period_w/o_gaps =40. For a UE supporting power class 2 (vehicle mounted), Mmeas_period_w/o_gaps =[24]. For a UE supporting power class 3 (handheld), Mmeas_period_w/o_gaps =[24]. For a UE supporting power class 4, Mmeas_period_w/o_gaps =TBD.

End of Change 1

Start of Change 2

### 9.2.6	Intrafrequency measurements with measurement gaps

#### 9.2.6.1	Intra gap sharing

[Editor’s note : This is being studied]

#### 9.2.6.2	Intrafrequency cell identification

Editor’s Note : The impact of gap sharing between intrafrequency and interfrequency measurements has not been include in the requirements below.

The UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_without_index if UE is not indicated to report SSB based RRM measurement result with the associated SSB index, or the UE has been indicated that the neighbour cell is synchronous with the serving cell.. Otherwise UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_with_index. The UE shall be able to identify a new detectable intra frequency SS block of an already detected cell within Tidentify_intra_without_index. It is assumed that  deriveSSB-IndexFromCell is always enabled for FR1 TDD and FR2, and hence the identification time is always Tidentify_intra_without_index

Tidentify_intra_without_index = TPSS/SSS_sync_intra + T SSB_measurement_period_intra  ms

Tidentify_intra_with_index = TPSS/SSS_sync_ntra + T SSB_measurement_period_intra + TSSB_time_index_intra

Where:

TPSS/SSS_sync_intra: it is the time period used in PSS/SSS detection given in table 9.2.6.2-1, 9.2.6.2-2,  or 9.2.6.2-4 or 9.2.6.2-5 (deactivated SCell)

TSSB_time_index_intra: it is the time period used to acquire the index of the SSB being measured given in table 9.2.6.2-3or 9.2.6.2-6(deactivated SCell).

T SSB_measurement_period_intra: equal to a measurement period of SSB based measurement given in table 9.2.6.2-1, 9.2.6.2-2, 9.2.6.2-3 (deactivated Scell) or 9.2.6.2-4(deactivated SCell)

Mpss/sss_sync_with_gaps : For a UE supporting power class 1(fixed wireless access), Mpss/sss_sync with_gaps=40. For a UE supporting power class 2(vehicle mounted), Mpss/sss_sync with_gaps =[24].  For a UE supporting power class 3(handheld), Mpss/sss_sync with_gaps =[24]. For a UE supporting power class 4, Mpss/sss_sync with_gaps =TBD

Mmeas_period_ with_gaps: For a UE supporting power class 1(fixed wireless access), Mmeas_period_ with_gaps =40. For a UE supporting power class 2(vehicle mounted), Mmeas_period_ with_gaps =[24]. For a UE supporting power class 3(handheld), Mmeas_period_ with_gaps =[24]. For a UE supporting power class 4, Mmeas_period with_gaps =TBD.

End of Change 2