3GPP TSG-RAN WG4 Meeting #88bis 	R4-1812150

Chengdu, China, October 8 – 12, 2018

Agenda item:	7.11.5.3

Source:	Intel Corporation

Title:	Discussion on remaining issue for RRC re-establishment

Document for:	Discussion

# Introduction

In last RAN4 #88 meeting, a CR was endorsed [1] to update the RRC re-establishment RRM requirement, in which the time delay to identify the target NR cell (Tidentify_NR) was updated depending not only on whether the target NR cell is known cell or unknown cell and on the frequency range (FR) of the target NR cell, but or so the serving cell SCH Ês/Iot, with the justification that if UE is out-of-sync, then the UE may lost the timing of serving cell, such that the timing of the “known” target cell in memory is not reliable anymore. In this case UE needs to re-synchronize to the “known” cell again, which would introduce additional delay.

So in core requirement the delay is further differentiated by the serving cell SCH Ês/Iot threshold X, which is TBD and can be decided based on RLM test cases. In this contribution, we will discuss the value of X based on our simulation RLM result.

# Discussion

For information, here we duplicate the core requirement w.r.t the threshold XdB from TS38.133:

Table 6.2.1.2.1-1: Time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell Serving cell SCH Ês/Iot (dB); Frequency range (FR) of target NR cell; Tidentify_intra_NR [ms]. Known NR cell; Unknown NR cell. ≥ X; FR1; MAX (200 ms, [5] x TSMTC); MAX (800 ms, [10] x TSMTC). ≥ X; FR2; N/A; MAX (1000 ms, ([80]) x TSMTC)). < X; FR1; N/A; 800Note1. < X; FR2; N/A; 3420Note1. Note 1: TSMTC =20 ms when serving cell SCH Ês/Iot < X. Editor’s note: The SCH Ês/Iot threshold (i.e, value of X) can be later decided based on RLM test cases. Table 6.2.1.2.1-2: Time to identify target NR cell for RRC connection re-establishment to NR inter-frequency cell Serving cell SCH Ês/Iot (dB); Frequency range (FR) of target NR cell; Tidentify_inter_NR, i [ms]. Known NR cell; Unknown NR cell. ≥ X; FR1; MAX (200 ms, [6] x TSMTC, i); MAX (800 ms, [13] x TSMTC, i). ≥ X; FR2; N/A; MAX (1000 ms, ([104]) x TSMTC, i)). < X; FR1; N/A; 800Note1. < X; FR2; N/A; 4000Note1. Note 1: TSMTC,i =20 ms when serving cell SCH Ês/Iot < X.

As we all know RLM requirement is defined under certain scenario, i.e. with certain PDCCH transmission parameters which is defined in TS38.133 section 8.1. Take SSB based RLM out-of-sync for example. The hypothetical PDCCH BLER for Qout is 10% under the following configuration:

Table 8.1.2.1-1: PDCCH transmission parameters for out-of-sync (in TS38.133)

| Attribute | Value for BLER pair#0 | Value for BLER pair#1 |
| --- | --- | --- |
| DCI format | 1-0 | TBD |
| Number of control OFDM symbols | Same as the number of symbols of RMSI CORESET |  |
| Aggregation level (CCE) | 8 |  |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 4dB |  |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 4dB |  |
| Bandwidth (MHz) | Same as the number of PRBs of RMSI CORESET |  |
| Sub-carrier spacing (kHz) | Same as the SCS of RMSI CORESET |  |
| DMRS precoder granularity | REG bundle size |  |
| REG bundle size | 6 |  |
| CP length | Same as the CP length of RMSI CORESET |  |
| Mapping from REG to CCE | Distributed |  |

With this configuration, the SNR for out-of-sync can be determined by simulation. Note that the exact SNR highly depends on the transmission parameters. Our simulation results for PDCCH can be found in [2]:

Table 1.  PDCCH results for 2x2

| 2x2 | 10MHz; SCS: 15KHz; TDL-C | 40MHz; SCS: 30KHz;TDL-C | 100MHz; SCS: 120KHz;TDL-A |
| --- | --- | --- | --- |
| In-Sync SNR dB | -2.2 | -2.1 | -1.8 |
| Out-of-sync SNR dB | -11 | -9.6 | -10.8 |

Table 2.  PDCCH results for 2x4

| 2x4 | 10MHz; SCS: 15KHz; TDL-C | 40MHz; SCS: 30KHz;TDL-C |
| --- | --- | --- |
| In-Sync SNR dB | -5.6 | -5.3 |
| Out-of-sync SNR dB | -13.6 | -13.3 |

As can be seen above, SNRQout is different for different configuration. However, it is not desirable that the threshold X in the core requirement would vary depending on the transmission parameters. A simple way is to determine X by using the parameters defined in RLM core requirement, which is deemed as the typical scenario.

Observation 1: UE may declare out-of-sync at different SNR level, depending on transmission parameters, e.g. BW, CCE level, PDCCH power boosting and etc.

Proposal 1: a fixed threshold X is preferred, which can be derived from PDCCH demodulation performance by using the transmission parameters defined in RLM core requirement.

Following this principle, it is suggested to choose the highest SNRQout as the threshold such that it can cover all the scenarios. According to our simulation results, -9.6dB is the highest one. However, it would not be safe to directly use SNRQout as the threshold X in RRC re-establishment requirement, given the evaluation uncertainty in SNR estimation. The SNR estimation error based on SSB can be found in the following figure:

Figure 1 simulation results for SNR estimation error based on SSB

For instance, assuming the SNRQout is -9.6dB, the SNR estimation error is about 1.7dB according to Figure 1.So there is a chance that UE would not declare out-of-sync when the actual SNR is -9.6dB since the evaluation result of SNR inside the baseband could be 0~1.7dB higher. However, under such low SNR the UE can hardly maintain a valid timing under corresponding transmission parameters. To address this, we propose to consider SNR estimation error when determining the threshold X. According to our simulation result, -8dB could be safe.

Observation 2: SNR estimation error shall be considered when determining threshold X.

Proposal 2: [-8] dB could be considered for the threshold X.

# Conclusion

In this contribution, we further discuss the threshold X in RRC re-establishment core requirement. After discussion the following observations and proposals are provided:

Observation 1: UE may declare out-of-sync at different SNR level, depending on transmission parameters, e.g. BW, CCE level, PDCCH power boosting and etc.

Proposal 1: a fixed threshold X is preferred, which can be derived from PDCCH demodulation performance by using the transmission parameters defined in RLM core requirement.

Observation 2: SNR estimation error shall be considered when determining threshold X.

Proposal 2: [-8] dB could be considered for the threshold X.

# References

- R4-1811709, “RRC Re-establishment Requirements without Serving Cell Timing”, Ericsson, Mediatek

- R4-1812124, “Discussion about SNR levels for RLM test”, Intel Corporation