3GPP TSG-RAN WG4 Meeting #88bis	R4-1812172

Chengdu, China, 8th – 12th Oct, 2018

Agenda item:	7.12.6.11

Source: 		Intel Corporation

Title: 			On Remaining issues for Beam Failure Detection

Document for:	Discussion

# Introduction

In RAN4#88 beam failure detection was discussed and some open issues were still remaining. In this paper we discuss the open issues related to beam failure detection.

# Discussion

Based on the agreements in RAN1#94 SSB based Beam Failure Detection (BFD) is not supported. BFD shall only be based on periodic CSI-RS resources.

Proposal #1: Define only CSI-RS based Beam failure detection requirements in 38.133

Some of the open issues on CSI-RS based BFD are:

- Default value of BLERout for BFD

- Evaluation period  in FR2

- CSI-RS configuration for BFD

## Threshold for BFD

For beam failure detection (BFD) the UE monitors a list of resources () and declares a beam failure instance to higher layers if the SNR of all the BFD resources is below a threshold Qout,LR. Qout,LR corresponds to the SNR at BLERout of a hypothetical PDCCH transmission. BLERout corresponds to the default value of BLERout used for RLM. For RLM the BLERin and BLERout values are specified in TS 38.133 Table 8.1.1-1 and the corresponding BLER pair is selected based on the config parameter RLM-IS-OOS-thresholdConfig.

Table 1: Out-of-sync and in-sync block error rates

| Configuration | BLERout | BLERin |
| --- | --- | --- |
| 0 | 10% | 2% |
| 1 | TBD | TBD |

The BLERout for config 0 is 10% whereas the BLERout for config 1 is TBD. In the LS from RAN1 on the second BLER pair for RLM [1], the 2nd BLER pair is suggested to be higher than that with config 0.

Observation #1: The 2nd BLER pair for RLM shall be larger than that with config 0 of RLM-IS-OOS-thresholdConfig

With the guidance that BLERout for config 1 is larger than 10%, if the default value of BLERout (10%) is used as threshold for BFD irrespective of the BLERout used RLM, we don’t have the issue for out-of-sync being detected before beam failure. In light of the clarification from RAN1, the BLERout for BFD shall always be the default value of BLERout for RLM, which is 10%.

Proposal #2: BLERout threshold for beam failure detection shall be fixed to 10%

## Evaluation period

The evaluation period for CSI-RS based BFD in FR2 is defined as:

Table 8.5.3.2-2: Evaluation period TEvaluate_BFD_CSI-RS for FR2

| Configuration | TEvaluate_BFD_CSI-RS (ms) |
| --- | --- |
| non-DRX | max([50], [MBFD *P*N] * TCSI-RS) |
| DRX cycle ≤ 320ms | max([50], [1.5×MBFD*P*N]*max(TDRX, TCSI-RS)) |
| DRX cycle > 320ms | [MBFD *P*N] * TDRX |
| Note: TCSI-RS is the periodicity of CSI-RS resource in the set . TDRX is the DRX cycle length. |  |

Where

N is defined as:

-	N=1,

if UE is not provided higher layer parameter RadioLinkMonitoringRS and UE is provided by higher layer parameter TCI-state for PDCCH CSI-RS that has QCL-TypeD, or

if the CSI-RS configured for BFD is QCL-Type D with DM-RS for PDCCH and the QCL association is known to UE, or

if the CSI-RS resource configured for BFD is QCL-Type D and TDMed to CSI-RS resources configured for L1-RSRP reporting or SSBs configured for L1-RSRP reporting, all CSI-RS resources configured for BFD are mutually TDMed, and the QCL association is known to UE and a CSI report with L1-RSRP measurement for the CSI-RS configured for BFD has been made within [TBD]ms;

-	N=FFS, otherwise.

P is defined as:

P=1, when BFD-RS is not overlapped with measurement gap and also not overlapped with SMTC occasion.

-	P=1/(1 – TCSI-RS/MGRP) , when BFD-RS is partially overlapped with measurement gap and BFD-RS is not overlapped with SMTC occasion (TCSI-RS < MGRP)

-	P=1/(1 – TCSI-RS /TSMTCperiod), when BFD-RS is not overlapped with measurement gap and BFD-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod).

-	P is Psharing factor, when BFD-RS is not overlapped with measurement gap and BFD-RS is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod).

-	P is 1/(1- TCSI-RS /MGRP - TCSI-RS /TSMTCperiod), when BFD-RS is partially overlapped with measurement gap and BFD-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and

-	TSMTCperiod ≠ MGRP or

-	TSMTCperiod = MGRP and TCSI-RS < 0.5*TSMTCperiod

-	P is 1/(1- TCSI-RS /MGRP)* Psharing factor, when BFD-RS is partially overlapped with measurement gap and BFD-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and TSMTCperiod = MGRP  and TCSI-RS = 0.5*TSMTCperiod

-	P is 1/{1- TCSI-RS /min (TSMTCperiod ,MGRP)}, when BFD-RS is partially overlapped with measurement gap (TCSI-RS < MGRP) and BFD-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is partially or fully overlapped with measurement gap.

-	P is 1/(1- TCSI-RS /MGRP)* Psharing factor, when BFD-RS is partially overlapped with measurement gap and BFD-RS is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod) and SMTC occasion is partially overlapped with measurement gap (TSMTCperiod < MGRP)

-	Psharing factor is 3.

MBFD = 10, if the CSI-RS resource configured for BFD is transmitted with Density = 3.

We propose to delete the last condition for N=1 :

if the CSI-RS resource configured for BFD is QCL-Type D and TDMed to CSI-RS resources configured for L1-RSRP reporting or SSBs configured for L1-RSRP reporting, all CSI-RS resources configured for BFD are mutually TDMed, and the QCL association is known to UE and a CSI report with L1-RSRP measurement for the CSI-RS configured for BFD has been made within [TBD]ms;

When the BFD RS (CSI-RS/ SSB) are QCL-Type D and TDMed to CSI-RS or SSB resources for L1-RSRP reporting, the N=1 condition shall apply only if a CSI report with the L1-RSRP measurement of the BFD RS is already made. But it is rather difficult to establish the validity of such a report as it would depend on the propagation channel conditions, type of CSI-RS resources – periodic or aperiodic. Hence we propose to delete the 3rd condition for N=1.

Observation #2: It is not trivial to determine a time period for validity of a CSI report

Proposal #3: Delete the condition for N=1 for BFD resources associated with L1-RSRP reporting resources

For the condition when Rx beam sweeping is necessary for the CSI-RS resources configured for BFD, the scale factor could be set as N= maxNumberRxBeam. In case MaxNumberRxBeam is not signalled, N=8 shall be used. The RRC parameter maxNumberRxBeam is used to indicate to the network how many repetitions of CSI-RS are needed to perform beam refinement on UE side. Using this parameter for defining evaluation time requirements shall be confirmed with RAN2.

Proposal #4: For evaluation time in FR2 for CSI-RS based BFD define N= maxNumberRxBeam or 8

Proposal #5: Confirm with RAN2 the usage of maxNumberRxBeam for the purpose of evaluation period for BFD

## CSI-RS Configuration for BFD

The evaluation period of CSI-RS based BFD is based on CSI-RS configured with D=3. With CSI-RS configurations with smaller density, the performance of CSI-RS based measurements is degraded and might need more number of samples to meet the accuracy requirements. This would result in a larger evaluation period. To avoid additional requirements for CSI-RS based BFD we propose to define requirements with CSI-RS configuration of D=3.

Proposal #6: Define CSI-RS based beam failure detection requirements with CSI-RS configuration with D=3

# Conclusions

In this paper we discuss the open issues related to beam failure detection and provide our views. Our observations and proposals are summarized below:

Observation #1: The 2nd BLER pair for RLM shall be larger than that with config 0 of RLM-IS-OOS-thresholdConfig

Observation #2: It is not trivial to determine a time period for validity of a CSI report

Proposal #1: Define only CSI-RS based Beam failure detection requirements in 38.133

Proposal #2: BLERout threshold for beam failure detection shall be fixed to 10%

Proposal #3: Delete the condition for N=1 for BFD resources associated with L1-RSRP reporting resources

Proposal #4: For evaluation time in FR2 for CSI-RS based BFD define N= maxNumberRxBeam or 8

Proposal #5: Confirm with RAN2 the usage of maxNumberRxBeam for the purpose of evaluation period for BFD

Proposal #6: Define CSI-RS based beam failure detection requirements with CSI-RS configuration with D=3

# Reference

- R1-1809853, “LS on second BLER pair for RLM”, RAN1, Ericsson