3GPP TSG-RAN WG4 Meeting #88bis	R4-1812194

# Chengdu, China, 8th – 12th, October 2018

Agenda item:	7.6.14.5.2

Source: 	Intel Corporation

Title: 	UL CA MPR in FR2

Document for:	Approval

# Introduction

In this contribution, we provide our MPR evaluation results for UL contiguous CA.

# Discussion

We consider MPR for contiguous UL CA with up to 8CCs with UL CA BW up to 800MHz.

The MPR values for contiguous PRB allocations (with necessary inter-CC gaps) are determined in the following first.

In [1], the MPR values for PC3 in UL contiguous CA with contiguous PRB allocations were proposed as follow

Maximum power reduction (MPRC_CA) for UE power class 3 [1]

|  |  | Cumulative aggregated channel bandwidth |  |  |
| --- | --- | --- | --- | --- |
|  |  | < 400MHz | >=400M and <800M | >=800M and <=1400M |
| DFT-s-OFDM | Pi/2 BPSK | 5.0 | 7.7 | 8.2 |
|  | QPSK | 5.0 | 7.7 | 8.2 |
|  | 16 QAM | 6.0 | 8.2 | 8.7 |
|  | 64 QAM | 8.5 | 10.7 | 11.2 |
| CP-OFDM | QPSK | 5.0 | 7.2 | 7.7 |
|  | 16 QAM | 6.0 | 8.2 | 8.7 |
|  | 64 QAM | 8.5 | 10.7 | 11.2 |

In our MPR evaluations, we took agreed PC3 single CC CP-OFDM MPR for 400MHz BW as a starting point [2], and verified for CA case with BW <400Mhz. For >=400MHz and <=800MHz, up to 8CCs were evaluated. Our proposed MPR values are listed in the following table.

Maximum power reduction (MPRC_CA) for UE power class 3

|  |  | Cumulative aggregated channel bandwidth |  |
| --- | --- | --- | --- |
|  |  | < 400MHz | >=400M and <=800M |
| DFT-s-OFDM | Pi/2 BPSK | 5.0 | 7.5 |
|  | QPSK | 5.0 | 7.5 |
|  | 16 QAM | 6.5 | 9.0 |
|  | 64 QAM | 9.0 | 10.5 |
| CP-OFDM | QPSK | 5.0 | 7.5 |
|  | 16 QAM | 6.5 | 9.0 |
|  | 64 QAM | 9.0 | 10.5 |

For PC1 device, the proposed MPR values in [1] for contiguous PRB allocations (with necessary inter-CC gaps) were defined well.

Next, for UL contiguous CA with non-contiguous PRB allocations, we evaluated MPR values vs. A, where A = the ratio of the number of populated RBs over total RB allowance with same SCS in UL CA aggregated BW = 800MHz.

The following plots show PC1 and PC3 required power backoff vs A.

Figure 1 MPR values vs A for non-contiguous PRB allocations in contiguous CCs

Based on our evaluations, we propose the same PC3 MPR rule for non-contiguous RB allocations in [1]

MPRNCRB_CA = max(MPRC_CA, -10*A +7.0)

For PC1, we propose the following MPR rule for non-contiguous RB allocations

MPRNCRB_CA = max(MPRC_CA, -10*A +14.0)

Where:

A = NRB_alloc / NRB_agg_C.

NRB_alloc is the total number of simultaneously transmitted UL RBs

NRB_agg_C is the number of the aggregated RBs within the fully allocated UL contiguous CA aggregated bandwidth

We use MPRNCRB_CA instead of MPRNC_CA to represent MPR for non-contiguous PRB allocations in UL CA with contiguous CCs since nomenclature of MPRNC_CA can be reserved for UL CA with Non-Contiguous CCs.

# Conclusion

In this contribution, we propose MPR for UL CA with contiguous CCs (up to 8CCs) for PC1 and PC3 with CA up to BW 800MHz.

# References

- R4-1811487, “Update FR2 CA MPR”, Qualcomm. RAN4#88, Gothenburg, Sweden. August, 2018

- 38.101-2-f30