3GPP TSG-RAN WG4 Meeting #88Bis	R4-1812312

# Chengdu, China, 8th – 12th Oct 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for test cases on inter-frequency RSRQ measurement accuracy (section A.4.7.2, A.5.7.2, A.6.7.2 and A.7.7.2) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT |  |  |  |  |  |  | Date: |  | 2018-08-18 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Test cases for contention-based and non-contention-based random access for FR1 is not completed yet. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the following listed random access test cases for FR1: - Contention based random access test in FR1 for PSCell in EN-DC; - Non-contention based random access test in FR1 for PSCell in EN-DC; - Contention based random access test in FR1 for NR standalone; - Non-contention based random access test in FR1 for NR standalone. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirement for contention-based and non-contention based random access for FR1 can’t be tested. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex A.4.7.2, A.5.7.2, A.6.7.2 and A.7.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### >>>>>Start of Change 1<<<<<

#### A.4.7.2	SS-RSRQ

<<Omit expected changes from other CRs>>

##### A.4.7.2.x	Inter-frequency case with FR1 FDD target cell

###### A.4.7.2.x.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in Clause 10.1.9.1.1 and 10.1.9.1.2.

###### A.4.7.2.x.2	Test Parameters

In this test case the two cells (i.e., Cell 2 alnd Cell 3) are on different carrier frequencies and measurement gaps are provided. Both absolute accuracy and relative accuracy requirements of SS-RSRQ inter-frequency measurement are tested by using test parameters in Table A.4.7.2.x.2-1. In all test cases, Cell 2 is the PSCell and Cell 3 is target cell on FR1 FDD band. Cell 1 is the E-UTRA cell which specific test parameters for this test case are specified in Table A.3.7.2.1-1.

Table A.4.7.2.x.2-1: SS-RSRQ Inter frequency test parameters for FR1 FDD target cell

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 2 | Cell 3 | Cell 2 | Cell 3 | Cell 2 | Cell 3 |
| Frequency Range |  |  | FR1 |  | FR1 |  | FR1 |  |
| BWchannel |  | MHz | 10 | 10 | 10 | 10 | 10 | 10 |
| SCS |  | kHz | 15 | 15 | 15 | 15 | 15 | 15 |
| Gap Pttern Id |  |  | 0 | - | 0 | - | 0 | - |
| PDSCH Reference measurement channel |  |  | SR.1.xFDD | - | SR.1.x FDD | - | SR.1.x FDD | - |
| OCNG Patterns |  |  | OP.1 | OP.1 | OP.1 | OP.1 | OP.1 | OP.1 |
| SMTC Configuration |  |  | SMTC.1 FR1 |  | SMTC.1 FR1 |  | SMTC.1 FR1 |  |
| EPRE ratio of PSS to SSS |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to SSS |  |  |  |  |  |  |  |  |
| Note2 | NR_FDD_FR1_A | dBm/15kHz | TBD |  | TBD |  | TBD |  |
|  | NR_FDD_FR1_B |  |  |  |  |  | TBD |  |
|  | NR_FDD_FR1_E |  |  |  |  |  | TBD |  |
|  | NR_FDD_FR1_G |  |  |  |  |  | TBD |  |
|  | NR_FDD_FR1_H |  |  |  |  |  | TBD |  |
|  | NR_FDD_FR1_I |  |  |  |  |  | TBD |  |
|  | NR_FDD_FR1_P |  |  |  |  |  | TBD |  |
|  |  | dB | TBD |  | TBD |  | TBD |  |
| SS-RSRPNote3 | NR_FDD_FR1_A | dBm/15kHz | TBD |  | TBD |  | TBD |  |
|  | NR_FDD_FR1_B |  |  |  |  |  | TBD |  |
|  | NR_FDD_FR1_E |  |  |  |  |  | TBD |  |
|  | NR_FDD_FR1_G |  |  |  |  |  | TBD |  |
|  | NR_FDD_FR1_H |  |  |  |  |  | TBD |  |
|  | NR_FDD_FR1_I |  |  |  |  |  | TBD |  |
|  | NR_FDD_FR1_P |  |  |  |  |  | TBD |  |
| SS-RSRQNote3 | NR_FDD_FR1_A | dB | TBD |  | TBD |  | TBD |  |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  |
|  | NR_FDD_FR1_E |  |  |  |  |  |  |  |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  |
|  | NR_FDD_FR1_I |  |  |  |  |  |  |  |
|  | NR_FDD_FR1_P |  |  |  |  |  |  |  |
| IoNote3 | NR_FDD_FR1_A | dBm/9.36 MHz | TBD |  | TBD |  | TBD |  |
|  | NR_FDD_FR1_B |  |  |  |  |  | TBD |  |
|  | NR_FDD_FR1_E |  |  |  |  |  | TBD |  |
|  | NR_FDD_FR1_G |  |  |  |  |  | TBD |  |
|  | NR_FDD_FR1_H |  |  |  |  |  | TBD |  |
|  | NR_FDD_FR1_I |  |  |  |  |  | TBD |  |
|  | NR_FDD_FR1_P |  |  |  |  |  | TBD |  |
|  |  | dB | TBD | TBD | TBD | TBD | TBD | TBD |
| Propagation condition |  | - | AWGN |  | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRQ, SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRQ and SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in Section 3.5.2. |  |  |  |  |  |  |  |  |

###### A.4.7.2.x.3	Test Requirements

The SS-RSRQ measurement accuracy shall fulfil the requirements in section 10.1.9.1.1 and 10.1.9.1.2.

##### A.4.7.2.y	Inter-frequency case with FR1 TDD target cell

###### A.4.7.2.y.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in Clause 10.1.9.1.1 and 10.1.9.1.2.

###### A.4.7.2.y.2	Test Parameters

In this test case the two cells (i.e., Cell 2 alnd Cell 3) are on different carrier frequencies and measurement gaps are provided. Both absolute accuracy and relative accuracy requirements of SS-RSRQ inter-frequency measurement are tested by using either test setup in Table A.4.7.2.y.2-1 for SCS = 15kHz or test setup in Table A.4.7.2.y.2-2 for SCS = 30kHz, depending on UE capability. In all test cases, Cell 2 is the PSCell on either FR1 FDD or FR1 TDD band, and Cell 3 is target cell on FR1 TDD band. Cell 1 is the E-UTRA cell which specific test parameters for this test case are specified in Table A.3.7.2.1-1.

Table A.4.7.2.y.2-1: SS-RSRQ Inter frequency test parameters for FR1 TDD target cell with SCS = 15kHz

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 2 | Cell 3 | Cell 2 | Cell 3 | Cell 2 | Cell 3 |
| Frequency Range |  |  | FR1 |  | FR1 |  | FR1 |  |
| BWchannel |  | MHz | 10 | 10 | 10 | 10 | 10 | 10 |
| SCS |  | kHz | 15 | 15 | 15 | 15 | 15 | 15 |
| Gap Pttern Id |  |  | 0 | - | 0 | - | 0 | - |
| PDSCH Reference measurement channel |  |  | SR.1.xTDD | SR.1.xTDD | SR.1.xTDD | SR.1.xTDD | SR.1.xTDD | SR.1.xTDD |
| OCNG Patterns |  |  | OP.1 | OP.1 | OP.1 | OP.1 | OP.1 | OP.1 |
| SMTC Configuration |  |  | SMTC.1 FR1 |  | SMTC.1 FR1 |  | SMTC.1 FR1 |  |
| EPRE ratio of PSS to SSS |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to SSS |  |  |  |  |  |  |  |  |
| Note2 | NR_TDD_FR1_A | dBm/15kHz | TBD |  | TBD |  | TBD |  |
|  | NR_TDD_FR1_C |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR1_D |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR1_E |  |  |  |  |  | TBD |  |
|  |  | dB | TBD |  | TBD |  | TBD |  |
| SS-RSRPNote3 | NR_TDD_FR1_A | dBm/15kHz | TBD |  | TBD |  | TBD |  |
|  | NR_TDD_FR1_C |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR1_D |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR1_E |  |  |  |  |  | TBD |  |
| SS-RSRQNote3 | NR_TDD_FR1_A | dB | TBD |  | TBD |  | TBD |  |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  |
|  | NR_TDD_FR1_D |  |  |  |  |  |  |  |
|  | NR_TDD_FR1_E |  |  |  |  |  |  |  |
| IoNote3 | NR_TDD_FR1_A | dBm/9.36 MHz | TBD |  | TBD |  | TBD |  |
|  | NR_TDD_FR1_C |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR1_D |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR1_E |  |  |  |  |  | TBD |  |
|  |  | dB | TBD | TBD | TBD | TBD | TBD | TBD |
| Propagation condition |  | - | AWGN |  | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRQ, SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRQ and SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in Section 3.5.2. |  |  |  |  |  |  |  |  |

Table A.4.7.2.y.2-2: SS-RSRQ Inter frequency test parameters for FR1 TDD target cell with SSB SCS = 30kHz

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 2 | Cell 3 | Cell 2 | Cell 3 | Cell 2 | Cell 3 |
| Frequency Range |  |  | FR1 |  | FR1 |  | FR1 |  |
| BWchannel |  | MHz | 40 | 40 | 40 | 40 | 40 | 40 |
| SCS |  | kHz | 30 | 30 | 30 | 30 | 30 | 30 |
| Gap Pttern Id |  |  | 0 | - | 0 | - | 0 | - |
| PDSCH Reference measurement channel |  |  | SR.1.xTDD | SR.1.xTDD | SR.1.xTDD | SR.1.xTDD | SR.1.xTDD | SR.1.xTDD |
| OCNG Patterns |  |  | OP.1 | OP.1 | OP.1 | OP.1 | OP.1 | OP.1 |
| SMTC Configuration |  |  | SMTC.2 FR1 |  | SMTC.2 FR1 |  | SMTC.2 FR1 |  |
| EPRE ratio of PSS to SSS |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to SSS |  |  |  |  |  |  |  |  |
| Note2 | NR_TDD_FR1_A | dBm/30kHz | TBD |  | TBD |  | TBD |  |
|  | NR_TDD_FR1_C |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR1_D |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR1_E |  |  |  |  |  | TBD |  |
|  |  | dB | TBD |  | TBD |  | TBD |  |
| SS-RSRPNote3 | NR_TDD_FR1_A | dBm/30kHz | TBD |  | TBD |  | TBD |  |
|  | NR_TDD_FR1_C |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR1_D |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR1_E |  |  |  |  |  | TBD |  |
| SS-RSRQNote3 | NR_TDD_FR1_A | dB | TBD |  | TBD |  | TBD |  |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  |
|  | NR_TDD_FR1_D |  |  |  |  |  |  |  |
|  | NR_TDD_FR1_E |  |  |  |  |  |  |  |
| IoNote3 | NR_TDD_FR1_A | dBm/38.16 MHz | TBD |  | TBD |  | TBD |  |
|  | NR_TDD_FR1_C |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR1_D |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR1_E |  |  |  |  |  | TBD |  |
|  |  | dB | TBD | TBD | TBD | TBD | TBD | TBD |
| Propagation condition |  | - | AWGN |  | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRQ, SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRQ and SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in Section 3.5.2. |  |  |  |  |  |  |  |  |

###### A.4.7.2.y.3	Test Requirements

The SS-RSRQ measurement accuracy shall fulfil the requirements in section 10.1.9.1.1 and 10.1.9.1.2.

### >>>>>End of Change 1<<<<<

### >>>>>Start of Change 2<<<<<

#### A.5.7.2	SS-RSRQ

<<Omit expected changes from other CRs>>

##### A.5.7.2.y	Inter-frequency case with FR2 TDD target cell

###### A.5.7.2.y.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in Clause 10.1.9.1.1 and 10.1.9.1.2.

###### A.5.7.2.y.2	Test Parameters

In this test case the two cells (i.e., Cell 2 alnd Cell 3) are on different carrier frequencies and measurement gaps are provided. Both absolute accuracy and relative accuracy requirements of SS-RSRQ inter-frequency measurement are tested by using test setup in Table A.5.7.2.y.2-1 for SCS = 120kHz. In all test cases, Cell 2 is the PSCell on FR2 TDD band, and Cell 3 is target cell on FR2 TDD band. Cell 1 is the E-UTRA cell which specific test parameters for this test case are specified in Table A.3.7.2.1-1.

Table A.5.7.2.y.2-1: SS-RSRQ Inter frequency test parameters for FR2 TDD target cell with SCS = 120kHz

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 2 | Cell 3 | Cell 2 | Cell 3 | Cell 2 | Cell 3 |
| Frequency Range |  |  | FR2 |  | FR2 |  | FR2 |  |
| BWchannel |  | MHz | 100 | 100 | 100 | 100 | 100 | 100 |
| SCS |  | kHz | 120 | 120 | 120 | 120 | 120 | 120 |
| Gap Pttern Id |  |  | 0 | - | 0 | - | 0 | - |
| PDSCH Reference measurement channel |  |  | SR.1.xTDD | SR.1.xTDD | SR.1.xTDD | SR.1.xTDD | SR.1.xTDD | SR.1.xTDD |
| OCNG Patterns |  |  | OP.1 | OP.1 | OP.1 | OP.1 | OP.1 | OP.1 |
| SMTC Configuration |  |  | SMTC.1 FR2 |  | SMTC.1 FR2 |  | SMTC.1 FR2 |  |
| EPRE ratio of PSS to SSS |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to SSS |  |  |  |  |  |  |  |  |
| Note2 | NR_TDD_FR2_A | dBm/120kHz | TBD |  | TBD |  | TBD |  |
|  | NR_TDD_FR2_B |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_F |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_G |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_T |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_Y |  |  |  |  |  | TBD |  |
|  |  | dB | TBD |  | TBD |  | TBD |  |
| SS-RSRPNote3 | NR_TDD_FR2_A | dBm/120kHz | TBD |  | TBD |  | TBD |  |
|  | NR_TDD_FR2_B |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_F |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_G |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_T |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_Y |  |  |  |  |  | TBD |  |
| SS-RSRQNote3 | NR_TDD_FR2_A | dB | TBD |  | TBD |  | TBD |  |
|  | NR_TDD_FR2_B |  |  |  |  |  |  |  |
|  | NR_TDD_FR2_F |  |  |  |  |  |  |  |
|  | NR_TDD_FR2_G |  |  |  |  |  |  |  |
|  | NR_TDD_FR2_T |  |  |  |  |  |  |  |
|  | NR_TDD_FR2_Y |  |  |  |  |  |  |  |
| IoNote3 | NR_TDD_FR2_A | dBm/95.04 MHz | TBD |  | TBD |  | TBD |  |
|  | NR_TDD_FR2_B |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_F |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_G |  |  |  |  |  |  |  |
|  | NR_TDD_FR2_T |  |  |  |  |  |  |  |
|  | NR_TDD_FR2_Y |  |  |  |  |  | TBD |  |
|  |  | dB | TBD | TBD | TBD | TBD | TBD | TBD |
| Propagation condition |  | - | AWGN |  | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRQ, SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRQ and SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in Section 3.5.2. |  |  |  |  |  |  |  |  |

###### A.5.7.2.y.3	Test Requirements

The SS-RSRQ measurement accuracy shall fulfil the requirements in section 10.1.9.1.1 and 10.1.9.1.2.

### >>>>>End of Change 2<<<<<

### >>>>>Start of Change 3<<<<<

#### A.6.7.2	SS-RSRQ

<<Omit expected changes from other CRs>>

##### A.6.7.2.x	Inter-frequency case with FR1 FDD target cell

###### A.6.7.2.x.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in Clause 10.1.9.1.1 and 10.1.9.1.2.

###### A.6.7.2.x.2	Test Parameters

In this test case the two cells (i.e., Cell 1 alnd Cell 2) are on different carrier frequencies and measurement gaps are provided. Both absolute accuracy and relative accuracy requirements of SS-RSRQ inter-frequency measurement are tested by using test parameters in Table A.6.7.2.x.2-1. In all test cases, Cell 1 is the PCell and Cell 2 is target cell on FR1 FDD band.

Table A.6.7.2.x.2-1: SS-RSRQ Inter frequency test parameters for FR1 FDD target cell

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Frequency Range |  |  | FR1 |  | FR1 |  | FR1 |  |
| BWchannel |  | MHz | 10 | 10 | 10 | 10 | 10 | 10 |
| SCS |  | kHz | 15 | 15 | 15 | 15 | 15 | 15 |
| Gap Pttern Id |  |  | 0 | - | 0 | - | 0 | - |
| PDSCH Reference measurement channel |  |  | SR.1.xFDD | - | SR.1.x FDD | - | SR.1.x FDD | - |
| OCNG Patterns |  |  | OP.1 | OP.1 | OP.1 | OP.1 | OP.1 | OP.1 |
| SMTC Configuration |  |  | SMTC.1 FR1 |  | SMTC.1 FR1 |  | SMTC.1 FR1 |  |
| EPRE ratio of PSS to SSS |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to SSS |  |  |  |  |  |  |  |  |
| Note2 | NR_FDD_FR1_A | dBm/15kHz | TBD |  | TBD |  | TBD |  |
|  | NR_FDD_FR1_B |  |  |  |  |  | TBD |  |
|  | NR_FDD_FR1_E |  |  |  |  |  | TBD |  |
|  | NR_FDD_FR1_G |  |  |  |  |  | TBD |  |
|  | NR_FDD_FR1_H |  |  |  |  |  | TBD |  |
|  | NR_FDD_FR1_I |  |  |  |  |  | TBD |  |
|  | NR_FDD_FR1_P |  |  |  |  |  | TBD |  |
|  |  | dB | TBD |  | TBD |  | TBD |  |
| SS-RSRPNote3 | NR_FDD_FR1_A | dBm/15kHz | TBD |  | TBD |  | TBD |  |
|  | NR_FDD_FR1_B |  |  |  |  |  | TBD |  |
|  | NR_FDD_FR1_E |  |  |  |  |  | TBD |  |
|  | NR_FDD_FR1_G |  |  |  |  |  | TBD |  |
|  | NR_FDD_FR1_H |  |  |  |  |  | TBD |  |
|  | NR_FDD_FR1_I |  |  |  |  |  | TBD |  |
|  | NR_FDD_FR1_P |  |  |  |  |  | TBD |  |
| SS-RSRQNote3 | NR_FDD_FR1_A | dB | TBD |  | TBD |  | TBD |  |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  |
|  | NR_FDD_FR1_E |  |  |  |  |  |  |  |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  |
|  | NR_FDD_FR1_I |  |  |  |  |  |  |  |
|  | NR_FDD_FR1_P |  |  |  |  |  |  |  |
| IoNote3 | NR_FDD_FR1_A | dBm/9.36 MHz | TBD |  | TBD |  | TBD |  |
|  | NR_FDD_FR1_B |  |  |  |  |  | TBD |  |
|  | NR_FDD_FR1_E |  |  |  |  |  | TBD |  |
|  | NR_FDD_FR1_G |  |  |  |  |  | TBD |  |
|  | NR_FDD_FR1_H |  |  |  |  |  | TBD |  |
|  | NR_FDD_FR1_I |  |  |  |  |  | TBD |  |
|  | NR_FDD_FR1_P |  |  |  |  |  | TBD |  |
|  |  | dB | TBD | TBD | TBD | TBD | TBD | TBD |
| Propagation condition |  | - | AWGN |  | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRQ, SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRQ and SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in Section 3.5.2. |  |  |  |  |  |  |  |  |

###### A.6.7.2.x.3	Test Requirements

The SS-RSRQ measurement accuracy shall fulfil the requirements in section 10.1.9.1.1 and 10.1.9.1.2.

##### A.6.7.2.y	Inter-frequency case with FR1 TDD target cell

###### A.6.7.2.y.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in Clause 10.1.9.1.1 and 10.1.9.1.2.

###### A.6.7.2.y.2	Test Parameters

In this test case the two cells (i.e., Cell 1 alnd Cell 2) are on different carrier frequencies and measurement gaps are provided. Both absolute accuracy and relative accuracy requirements of SS-RSRQ inter-frequency measurement are tested by using either test setup in Table A.6.7.2.y.2-1 for SCS = 15kHz or test setup in Table A.6.7.2.y.2-2 for SCS = 30kHz, depending on UE capability. In all test cases, Cell 1 is the PCell on either FR1 FDD or FR1 TDD band, and Cell 2 is target cell on FR1 TDD band.

Table A.6.7.2.y.2-1: SS-RSRQ Inter frequency test parameters for FR1 TDD target cell with SCS = 15kHz

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 2 | Cell 3 | Cell 2 | Cell 3 | Cell 2 | Cell 3 |
| Frequency Range |  |  | FR1 |  | FR1 |  | FR1 |  |
| BWchannel |  | MHz | 10 | 10 | 10 | 10 | 10 | 10 |
| SCS |  | kHz | 15 | 15 | 15 | 15 | 15 | 15 |
| Gap Pttern Id |  |  | 0 | - | 0 | - | 0 | - |
| PDSCH Reference measurement channel |  |  | SR.1.xTDD | SR.1.xTDD | SR.1.xTDD | SR.1.xTDD | SR.1.xTDD | SR.1.xTDD |
| OCNG Patterns |  |  | OP.1 | OP.1 | OP.1 | OP.1 | OP.1 | OP.1 |
| SMTC Configuration |  |  | SMTC.1 FR1 |  | SMTC.1 FR1 |  | SMTC.1 FR1 |  |
| EPRE ratio of PSS to SSS |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to SSS |  |  |  |  |  |  |  |  |
| Note2 | NR_TDD_FR1_A | dBm/15kHz | TBD |  | TBD |  | TBD |  |
|  | NR_TDD_FR1_C |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR1_D |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR1_E |  |  |  |  |  | TBD |  |
|  |  | dB | TBD |  | TBD |  | TBD |  |
| SS-RSRPNote3 | NR_TDD_FR1_A | dBm/15kHz | TBD |  | TBD |  | TBD |  |
|  | NR_TDD_FR1_C |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR1_D |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR1_E |  |  |  |  |  | TBD |  |
| SS-RSRQNote3 | NR_TDD_FR1_A | dB | TBD |  | TBD |  | TBD |  |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  |
|  | NR_TDD_FR1_D |  |  |  |  |  |  |  |
|  | NR_TDD_FR1_E |  |  |  |  |  |  |  |
| IoNote3 | NR_TDD_FR1_A | dBm/9.36 MHz | TBD |  | TBD |  | TBD |  |
|  | NR_TDD_FR1_C |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR1_D |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR1_E |  |  |  |  |  | TBD |  |
|  |  | dB | TBD | TBD | TBD | TBD | TBD | TBD |
| Propagation condition |  | - | AWGN |  | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRQ, SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRQ and SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in Section 3.5.2. |  |  |  |  |  |  |  |  |

Table A.6.7.2.y.2-2: SS-RSRQ Inter frequency test parameters for FR1 TDD target cell with SSB SCS = 30kHz

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 2 | Cell 3 | Cell 2 | Cell 3 | Cell 2 | Cell 3 |
| Frequency Range |  |  | FR1 |  | FR1 |  | FR1 |  |
| BWchannel |  | MHz | 40 | 40 | 40 | 40 | 40 | 40 |
| SCS |  | kHz | 30 | 30 | 30 | 30 | 30 | 30 |
| Gap Pttern Id |  |  | 0 | - | 0 | - | 0 | - |
| PDSCH Reference measurement channel |  |  | SR.1.xTDD | SR.1.xTDD | SR.1.xTDD | SR.1.xTDD | SR.1.xTDD | SR.1.xTDD |
| OCNG Patterns |  |  | OP.1 | OP.1 | OP.1 | OP.1 | OP.1 | OP.1 |
| SMTC Configuration |  |  | SMTC.2 FR1 |  | SMTC.2 FR1 |  | SMTC.2 FR1 |  |
| EPRE ratio of PSS to SSS |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to SSS |  |  |  |  |  |  |  |  |
| Note2 | NR_TDD_FR1_A | dBm/30kHz | TBD |  | TBD |  | TBD |  |
|  | NR_TDD_FR1_C |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR1_D |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR1_E |  |  |  |  |  | TBD |  |
|  |  | dB | TBD |  | TBD |  | TBD |  |
| SS-RSRPNote3 | NR_TDD_FR1_A | dBm/30kHz | TBD |  | TBD |  | TBD |  |
|  | NR_TDD_FR1_C |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR1_D |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR1_E |  |  |  |  |  | TBD |  |
| SS-RSRQNote3 | NR_TDD_FR1_A | dB | TBD |  | TBD |  | TBD |  |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  |
|  | NR_TDD_FR1_D |  |  |  |  |  |  |  |
|  | NR_TDD_FR1_E |  |  |  |  |  |  |  |
| IoNote3 | NR_TDD_FR1_A | dBm/38.16 MHz | TBD |  | TBD |  | TBD |  |
|  | NR_TDD_FR1_C |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR1_D |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR1_E |  |  |  |  |  | TBD |  |
|  |  | dB | TBD | TBD | TBD | TBD | TBD | TBD |
| Propagation condition |  | - | AWGN |  | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRQ, SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRQ and SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in Section 3.5.2. |  |  |  |  |  |  |  |  |

###### A.6.7.2.y.3	Test Requirements

The SS-RSRQ measurement accuracy shall fulfil the requirements in section 10.1.9.1.1 and 10.1.9.1.2.

### >>>>>End of Change 3<<<<<

### >>>>>Start of Change 4<<<<<

#### A.7.7.2	SS-RSRQ

<<Omit expected changes from other CRs>>

##### A.7.7.2.y	Inter-frequency case with FR2 TDD target cell

###### A.7.7.2.y.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in Clause 10.1.9.1.1 and 10.1.9.1.2.

###### A.7.7.2.y.2	Test Parameters

In this test case the two cells (i.e., Cell 1 alnd Cell 2) are on different carrier frequencies and measurement gaps are provided. Both absolute accuracy and relative accuracy requirements of SS-RSRQ inter-frequency measurement are tested by using test setup in Table A.7.7.2.y.2-1 for SCS = 120kHz. In all test cases, Cell 1 is the PCell on FR2 TDD band, and Cell 2 is target cell on FR2 TDD band.

Table A.7.7.2.y.2-1: SS-RSRQ Inter frequency test parameters for FR2 TDD target cell with SCS = 120kHz

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Frequency Range |  |  | FR2 |  | FR2 |  | FR2 |  |
| BWchannel |  | MHz | 100 | 100 | 100 | 100 | 100 | 100 |
| SCS |  | kHz | 120 | 120 | 120 | 120 | 120 | 120 |
| Gap Pttern Id |  |  | 0 | - | 0 | - | 0 | - |
| PDSCH Reference measurement channel |  |  | SR.1.xTDD | SR.1.xTDD | SR.1.xTDD | SR.1.xTDD | SR.1.xTDD | SR.1.xTDD |
| OCNG Patterns |  |  | OP.1 | OP.1 | OP.1 | OP.1 | OP.1 | OP.1 |
| SMTC Configuration |  |  | SMTC.1 FR2 |  | SMTC.1 FR2 |  | SMTC.1 FR2 |  |
| EPRE ratio of PSS to SSS |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to SSS |  |  |  |  |  |  |  |  |
| Note2 | NR_TDD_FR2_A | dBm/120kHz | TBD |  | TBD |  | TBD |  |
|  | NR_TDD_FR2_B |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_F |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_G |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_T |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_Y |  |  |  |  |  | TBD |  |
|  |  | dB | TBD |  | TBD |  | TBD |  |
| SS-RSRPNote3 | NR_TDD_FR2_A | dBm/120kHz | TBD |  | TBD |  | TBD |  |
|  | NR_TDD_FR2_B |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_F |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_G |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_T |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_Y |  |  |  |  |  | TBD |  |
| SS-RSRQNote3 | NR_TDD_FR2_A | dB | TBD |  | TBD |  | TBD |  |
|  | NR_TDD_FR2_B |  |  |  |  |  |  |  |
|  | NR_TDD_FR2_F |  |  |  |  |  |  |  |
|  | NR_TDD_FR2_G |  |  |  |  |  |  |  |
|  | NR_TDD_FR2_T |  |  |  |  |  |  |  |
|  | NR_TDD_FR2_Y |  |  |  |  |  |  |  |
| IoNote3 | NR_TDD_FR2_A | dBm/95.04 MHz | TBD |  | TBD |  | TBD |  |
|  | NR_TDD_FR2_B |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_F |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_G |  |  |  |  |  |  |  |
|  | NR_TDD_FR2_T |  |  |  |  |  |  |  |
|  | NR_TDD_FR2_Y |  |  |  |  |  | TBD |  |
|  |  | dB | TBD | TBD | TBD | TBD | TBD | TBD |
| Propagation condition |  | - | AWGN |  | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRQ, SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRQ and SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in Section 3.5.2. |  |  |  |  |  |  |  |  |

###### A.7.7.2.y.3	Test Requirements

The SS-RSRQ measurement accuracy shall fulfil the requirements in section 10.1.9.1.1 and 10.1.9.1.2.

### >>>>>End of Change 4<<<<<