3GPP RAN WG4 Meeting #88bis 	R4-1812391

# Chengdu, China, October 8-12, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | DRAFT | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.101-2: Introducing multi-band applicability for PC3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-10-8 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The spherical coverage requirement for PC3, as captured in Version 15.3.0 of the specification, includes a note which defines the requirement’s applicability in the following way: “The requirements in this table are only applicable for UE which supports single band in FR2.” There are concerns among operators and device vendors that this definition precludes the applicability of the requirement to devices which support multiple FR2 bands. For operators who own spectrum which spans multiple 3GPP FR2 bands and for device vendors who target global device SKUs the lack of 3GPP requirements for such UEs may pose a serious risk from the certification perspective. The Rel-15 NR work item exception sheet contains the following objectives related to the multi-band applicability of FR2 minimum peak EIRP and EIRP spherical coverage requirements: • 6.2.1 UE MOP for PC1/PC2/PC3/PC4 o For PC3, spherical coverage and peak EIRP requirements for UEs which support multiple FR2 bands |  |  |  |  |  |  |  |  |
| Summary of change: |  | Based on the technical analysis provided in the accompanying discussion paper (R4-1812390), this CR proposes to define relaxations for peak EIRP and spherical coverage limits, labeled ΔTMBP and ΔTMBS, respectively. These relaxations are indexed by the bands which the device supports and are defined in a band-specific manner. For consistency, the CR also clarifies that the existing peak EIRP limits are applicable to UEs which support a single band in FR2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Spherical coverage requirements for multi-band UEs are not defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 38.521-2 |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<<<start of change>>>

#### 6.2.1.3	UE maximum output power for power class 3

The following requirements define the maximum output power radiated by the UE for any transmission bandwidth within the channel bandwidth for non-CA configuration, unless otherwise stated. The period of measurement shall be at least one sub frame (1ms). The values listed on the table below are for handheld UE, defined as minimum peak EIRP. The requirement is verified with the test metric of EIRP (Link=Beam peak search grids, Meas=Link angle).

Table 6.2.1.3-1: UE minimum peak EIRP for power class 3

| Operating band | Min peak EIRP (dBm) |
| --- | --- |
| n257 | 22.4 |
| n258 | 22.4 |
| n260 | 20.6 |
| n261 | 22.4 |
| NOTE 1: Minimum peak EIRP is defined as the lower limit without tolerance NOTE 2: The requirements in this table are only applicable for UE which supports single band in FR2 |  |

The maximum output power values for TRP and EIRP are found on the Table6.2.1.3-2. The max allowed EIRP is derived from regulatory requirements [8]. The requirements are verified with the test metrics of TRP (Link=TX beam peak direction) in beam locked mode and EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 6.2.1-2: UE maximum output power limits for power class 3

| Operating band | Max TRP (dBm) | Max EIRP (dBm) |
| --- | --- | --- |
| n257 | 23 | 43 |
| n258 | 23 | 43 |
| n260 | 23 | 43 |
| n261 | 23 | 43 |

The minimum EIRP at the 50th percentile of the distribution of radiated power measured over the full sphere around the UE is defined as the spherical coverage requirement and is found in Table 6.2.1.3-3 below. The requirement is verified with the test metric of EIRP (Link=Beam peak search grids, Meas=Link angle).

Table 6.2.1.3-3: UE spherical coverage for power class 3

| Operating band | Min EIRP at 50%-tile CDF (dBm) |
| --- | --- |
| n257 | 11.5 |
| n258 | 11.5 |
| n260 | 8 |
| n261 | 11.5 |
| NOTE 1: Minimum EIRP at 50 %-tile CDF is defined as the lower limit without tolerance NOTE 2: The requirements in this table are only applicable for UE which supports single band in FR2 |  |

Peak EIRP and spherical coverage performance for UEs which support multiple bands is expected to be optimized over the entire supported range of frequencies and is relaxed from the single-band requirements in Table 6.2.1.3-1 by the factor ΔTMBP (multi-band relaxation of minimum peak EIRP) and in Table 6.2.1.3-3 by the factor ΔTMBS (multi-band relaxation of spherical coverage), which are defined in Table 6.2.1.3-4 below.

Table 6.2.1.3-4: UE multi-band relaxations for power class 3

| Supported bands | Applicable to band | ΔTMBP (dB) | ΔTMBS (dB) |
| --- | --- | --- | --- |
| n260, n261 | n260 | [0] | [1.6] |
| n260, n261 | n261 | [0] | [0] |
| n257, n258, n260, n261 | n257 | [2.1] | [1.9] |
| n257, n258, n260, n261 | n258 | [2.1] | [2.0] |
| n257, n258, n260, n261 | n260 | [1.5] | [1.8] |
| n257, n258, n260, n261 | n261 | [2.0] | [1.2] |
| NOTE 1: The requirements in this table are applicable to UEs which support only the indicated bands |  |  |  |

<<<end of change>>>