3GPP TSG-RAN4 Meeting #88bis 	R4-1812456

# Chengdu, China, 8 – 12 Oct, 2018

Title:	Draft LS on frame boundary offset in OTDOA reference cell configuration

Response to:

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	RAN WG 4

To:	RAN WG 2

CC:

Contact Person:

#### Name:	Qiming Li

E-mail Address:	qiming.li@intel.com

Attachments:	N/A

1. Overall Description:

Inter-RAT RSTD measurement is supported for NR UE. UE may need to request measurement gap prior to performing RSTD measurement. Currently in TS38.355 the SFN offset between the serving NR cell and the assistance data reference cell can be provided to the UE. However, RAN4 believes only SFN offset is not enough for the UE to request measurement gaps. UE also needs to know the frame boundary offset between the NR serving cell and the assistance data reference cell, in order to calculate the gap offset, which shall be indicated when requesting measurement gaps.

2. Actions:

To RAN WG2:

RAN4 respectfully asks RAN2 to introduce frame boundary offset between the NR serving cell and the assistance data reference cell in OTDOA reference cell configuration.

3. Date of Next RAN4 Meetings:

3GPP RAN4 #89 		12 - 16 Nov 2018, 		Spokane, US

3GPP RAN4 #90		25 Feb - 1 Mar 2019, 		Athens, GR