3GPP TSG-RAN WG4 Meeting RAN4#88bis                                               R4-18Xxxxxx

Chengdu ,China, 8 -12 Oct, 2018

Agenda Item:   7.13.1.4

Source:        Samsung

Title:          Simulation results for fading CQI test cases

Document for:  Discussion

# Introduction

In last RAN4 meeting, list of test cases has been captured in WF [1]:

- Static CQI test cases - Test 1: FR1 FDD 10MHz &15kHz , 2*2, 2*4 - Test 2: FR1 TDD 40MHz&30kHz, 2*2, 2*4 - Test 3: FR2 100MHz& 120kHz, 2*2 - Wideband fading CQI test cases - Test 1: FR1 FDD 10MHz &15kHz , 2*2, 2*4 - Test 2: FR1 TDD 40MHz&30kHz, 2*2, 2*4 - Test 3: FR2 100MHz& 120kHz, 2*2 - Wideband PMI test cases - Test 1: FR1 FDD 10MHz &15kHz , 8*2, 8*4 - Test 2: FR1 TDD 40MHz&30kHz, 8*2, 8*4 - Test 3: FR2 100MHz& 120kHz, 2*2 - 4Tx PMI test cases (Wideband) - Test 1: FR1 FDD 10MHz &15kHz , 4*2, 4*4 - Test 2: FR1 TDD 40MHz&30kHz, 4*2, 4*4 - Sub-band CQI test cases (2nd priority) - Test 1: FR1 FDD 10MHz&15kHz, 2*2,2*4 - Test 2: FR1 TDD 40MHz&30kHz, 2*2,2*4 - RI test - Test 1: FR1 FDD 10MHz&15kHz, 2*2,4*4 - Test 2: FR1 TDD 40MHz&30kHz, 2*2, 4*4 - Test 3: FR2 100MHz&120kHz, 2*2

In this contribution, initial simulation results for fading CQI test cases provided.

# Discussion

## Simulation assumption

| Parameter | Unit | Fading CQI test (FR1 FDD) | Fading CQI test (FR1 TDD) | Fading CQI test (FR2) |
| --- | --- | --- | --- | --- |
| System parameter |  |  |  |  |
| Bandwidth/SCS | MHz/kHz | 10MHz /15kHz | 40MHz/30kHz | 100MHz/120kHz |
| Transmission Scheme |  | 1 | 1 | 1 |
| Duplex Mode |  | FDD | TDD | TDD |
| DLUL configuration |  | N.A | FFS | FFS |
| Propagation channel |  | TDL_A 30ns, 5Hz | TDL_A 30ns, 10Hz | FFS |
| Correlation and antenna configuration |  | 2Rx 2*2 4Rx: 2*4 [ULA High] | 2Rx 2*2 4Rx: 2*4 [ULA High] | 2Rx 2*2 |
| CSI-RS resource setting (periodic) |  |  |  |  |
| resourceConfigType |  | Periodic | Periodic | Periodic |
| CSI-RS/CSI-IM periodicity and slot offset |  | 5/0 | 5/0 | 5/0 |
| NZP CSI-RS ports |  | 2 port {3000,3001} | 2 port {3000,3001} | 2 port {3000,3001} |
| NZP CSI-RS configuration CDM type/CSIRS resource mapping/CSI-RS-Density |  | [FD-CDM2/density 1] | [FD-CDM2/density 1] | [FD-CDM2/density 1] |
| CSI-IM configuration |  | FFS | FFS | FFS |
| CSI report setting (periodic) |  |  |  |  |
| reportConfigType |  | periodic | periodic | periodic |
| Reporting periodicity and slot offset |  | FFS | FFS | FFS |
| Codebook Restriction |  | 000001 | 000001 | 000001 |
| Reporting granularity |  | Wide-band | Wide-band | Wide-band |
| Physical channel for reporting |  | PUSCH | PUSCH | PUSCH |
| PUCCH reporting format |  | FFS | FFS | FFS |
| CQI table |  | Table 2 with 256QAM | Table 2 with 256QAM | Table 1 without 256QAM |

## Simulation results

Simulation results for FR1 FDD

Table 1 Wideband fading CQI Result for FR1 FDD with 2*2

| SNR | Percentage of Reported CQI Index not in {median CQI1 -1, median CQI1 +1} (%) | BLER with following CQI | TP ratio with following CQI and Median CQI |
| --- | --- | --- | --- |
| 0dB | 20 | 0.0782 | 1.4564722 |
| 2dB | 22.2 | 0.0448 | 1.4087655 |
| 4dB | 30.45 | 0.0432 | 1.3735969 |
| 6dB | 34.75 | 0.064 | 1.3764034 |
| 8dB | 38.35 | 0.071 | 1.6047312 |
| 10dB | 36.5 | 0.0874 | 1.6770812 |
| 12dB | 30.85 | 0.0904 | 1.267249 |
| 14dB | 39.4 | 0.1098 | 1.3116845 |
| 16dB | 52.75 | 0.111 | 1.6475566 |
| 18dB | 38.15 | 0.1444 | 1.1579671 |
| 20dB | 31.55 | 0.1562 | 1.0926407 |

Table 2 Wideband fading CQI Result for FR1 FDD with 2*4

| SNR | Percentage of Reported CQI Index not in {median CQI1 -1, median CQI1 +1} (%) | BLER with following CQI | TP ratio with following CQI and Median CQI |
| --- | --- | --- | --- |
| 0dB | 1.65 | 0.1364 | 1.2423303 |
| 2dB | 3.35 | 0.1936 | 1.3678829 |
| 4dB | 8.3 | 0.1076 | 1.3053777 |
| 6dB | 11.1 | 0.0836 | 1.3462253 |
| 8dB | 10.3 | 0.0674 | 1.2057328 |
| 10dB | 11.1 | 0.0672 | 1.2996199 |
| 12dB | 18.4 | 0.0716 | 1.1795128 |
| 14dB | 19.4 | 0.0884 | 1.4158032 |
| 16dB | 22.2 | 0.1086 | 1.2200458 |
| 18dB | 15.85 | 0.1268 | 1.339773 |
| 20dB | 5.8 | 0.1776 | 1.1814386 |

Simulation results for FR1 TDD

Table 3 Wideband fading CQI Result for FR1 TDD with 2*2

| SNR | Percentage of Reported CQI Index not in {median CQI1 -1, median CQI1 +1} (%) | BLER with following CQI | TP ratio with following CQI and Median CQI |
| --- | --- | --- | --- |
| 0dB | 6 | 0.0886 | 1.7368664 |
| 2dB | 25.24 | 0.0458 | 1.728553 |
| 4dB | 28.72 | 0.094 | 1.8649223 |
| 6dB | 32.32 | 0.102 | 1.2038135 |
| 8dB | 25.6 | 0.1054 | 1.5552951 |
| 10dB | 24.92 | 0.1062 | 1.8378367 |
| 12dB | 31.28 | 0.1162 | 1.5658273 |
| 14dB | 42.76 | 0.122 | 1.2486694 |
| 16dB | 40.72 | 0.116 | 1.8089683 |
| 18dB | 26.92 | 0.1234 | 1.9877104 |
| 20dB | 18.44 | 0.1254 | 2.6255797 |

Table 4 Wideband fading CQI Result for FR1 TDD with 2*4

| SNR | Percentage of Reported CQI Index not in {median CQI1 -1, median CQI1 +1} (%) | BLER with following CQI | TP ratio with following CQI and Median CQI |
| --- | --- | --- | --- |
| 0dB | 3.94 | 0.2126 | 1.268603 |
| 2dB | 6.72 | 0.1652 | 1.2076052 |
| 4dB | 9.24 | 0.184 | 1.3819289 |
| 6dB | 12.12 | 0.1558 | 1.2590668 |
| 8dB | 10 | 0.1104 | 1.2192195 |
| 10dB | 9.88 | 0.1008 | 1.3436281 |
| 12dB | 14 | 0.0904 | 1.2499806 |
| 14dB | 16.16 | 0.0994 | 1.3311359 |
| 16dB | 21.04 | 0.1036 | 1.3319707 |
| 18dB | 14.24 | 0.0926 | 1.2919964 |
| 20dB | 4.2 | 0.1212 | 1.2087425 |

Simulation results for FR2 TDD

Table 5 Wideband fading CQI Result for FR2 TDD with 2*2

| SNR | Percentage of Reported CQI Index not in {median CQI1 -1, median CQI1 +1} (%) | BLER with following CQI | TP ratio with following CQI and Median CQI |
| --- | --- | --- | --- |
| 0dB | 7.86 | 0.2358 | 1.0668308 |
| 2dB | 10.6 | 0.2012 | 1.4186375 |
| 4dB | 23.84 | 0.122 | 1.1767849 |
| 6dB | 19 | 0.1948 | 1.1556555 |
| 8dB | 7.72 | 0.2218 | 1.5025668 |
| 10dB | 14 | 0.1632 | 1.7545439 |
| 12dB | 14.64 | 0.1332 | 1.6632596 |
| 14dB | 8.4 | 0.1012 | 1.6985785 |
| 16dB | 2.04 | 0.0802 | 1.1087633 |
| 18dB | 3.28 | 0.0824 | 1.181332 |
| 20dB | 7.86 | 0.0434 | 1.3596719 |

# Conclusion

In this contribution, we provided simulation results for fading CQI test. Below proposals given for test SNR points and requirements:

Proposal1: For FR1 2Rx (FDD and TDD):

- Test point: 6/7dB  +12/13 dB

- Test requirements:

  - Aifa>20%

  - BLER>2%

  - Gamma>1.05

Proposal2: For FR1 4Rx (FDD and TDD):

- Test point: 3/4dB  +9/10 dB

- Test requirements:

  - Aifa>2%

  - BLER>2%

  - Gamma>1.05

Proposal3: For FR2 2Rx (TDD):

- Test point: 6/7dB  +12/13 dB

- Test requirements:

  - Aifa>2%

  - BLER>2%

  - Gamma>1.05

# Reference

[1] R4-1811693, “Way forward on NR UE CSI requirements”, Samsung