3GPP TSG-RAN4 Meeting #88BIS 	R4-1812486

# Chengdu, China, 8 – 12 Oct., 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for Intra-frequency SS-RSRQ Accuarcy Test Cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT |  |  |  |  |  |  | Date: |  | 2018-09-20 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There are no intra-frequency SS-RSRQ accuracy test cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add intra-frequency SS-RSRQ accuracy test cases for EN-DC FR1/FR2 and NR standalone FR1/FR2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Intra-frequency SS-RSRQ accuracy requirements can not be verified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.7.2, A.5.7.2, A.6.7.2, A.7.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

A.4.7	Measurement Performance requirements

----- << Start of Change 1>> -----

A.4.7.2	SS-RSRQ

A.4.7.2.1	FDD Intra-frequency case

A.4.7.2.1.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in Clause 10.1.7.1.1.

A.4.7.2.1.2	Test Parameters

In this test case all cells are on the same carrier frequency. The absolute accuracy of SS-RSRQ intra-frequency measurement is test by using the parameters in Table A.4.7.2.1.2-1. In all test cases, Cell 2 is the PSCell and Cell 3 is the target cell. Cell 1 is the E-UTRA cell which specific test parameters for this test case are specified in Table A.3.7.2.1-1.

Table A.4.7.2.1.2-1: SS-RSRQ FDD Intra frequency test parameters for SCS=15kHz

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 2 | Cell 3 | Cell 2 | Cell 3 | Cell 2 | Cell 3 |
| Frequency Range |  |  | FR1 |  | FR1 |  | FR1 |  |
| BWchannel |  | MHz | 10 |  | 10 |  | 10 |  |
| PDSCH Reference measurement channel |  |  | SR.1.xFDD | - | SR.1.x FDD | - | SR.1.x FDD | - |
| OCNG Patterns |  |  | OP.x | OP.y | OP.x | OP.y | OP.x | OP.y |
| SMTC Configuration |  |  | SMTC.1 FR1 |  | SMTC.1 FR1 |  | SMTC.1 FR1 |  |
| SS-RSSI-Measurement |  |  | N/A |  | N/A |  | N/A |  |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |
| PBCH_DMRS_RA |  |  |  |  |  |  |  |  |
| PBCH_DMRS_RB |  |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |  |
| Note2 | NR_FDD_FR1_A | dBm/15kHz | -84.94 |  | -103.85 |  | -117 |  |
|  | NR_FDD_FR1_B |  |  |  |  |  | -116 |  |
|  | NR_FDD_FR1_E |  |  |  |  |  | -115 |  |
|  | NR_FDD_FR1_G |  |  |  |  |  | -114 |  |
|  | NR_FDD_FR1_H |  |  |  |  |  | -113 |  |
|  |  | dB | -1.76 |  | -4.7 |  | -5.46 |  |
| SS-RSRPNote3 | NR_FDD_FR1_A | dBm/15kHz | -81.94 |  | -106.75 |  | -121 |  |
|  | NR_FDD_FR1_B |  |  |  |  |  | -120 |  |
|  | NR_FDD_FR1_E |  |  |  |  |  | -119 |  |
|  | NR_FDD_FR1_G |  |  |  |  |  | -118 |  |
|  | NR_FDD_FR1_H |  |  |  |  |  | -117 |  |
| SS-RSRQNote3 | NR_FDD_FR1_A | dB | -14.77 |  | -16.76 |  | -17.34 |  |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  |
|  | NR_FDD_FR1_E |  |  |  |  |  |  |  |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  |
| IoNote3 | NR_FDD_FR1_A | dBm/9.36 MHz | -50 |  | -73 |  | -86.5 |  |
|  | NR_FDD_FR1_B |  |  |  |  |  | -85.5 |  |
|  | NR_FDD_FR1_E |  |  |  |  |  | -84.5 |  |
|  | NR_FDD_FR1_G |  |  |  |  |  | -83.5 |  |
|  | NR_FDD_FR1_H |  |  |  |  |  | -82.5 |  |
|  |  | dB | 3 | 3 | -2.9 | -2.9 | -4 | -4 |
| Propagation condition |  | - | AWGN |  | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRQ, SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRQ and SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in Section 3.5.2. |  |  |  |  |  |  |  |  |

A.4.7.2.1.3	Test Requirements

The SS-RSRQ measurement accuracy shall fulfil the requirements in section 10.1.7.1.1.

A.4.7.2.2	TDD Intra-frequency case

A.4.7.2.2.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in Clause 10.1.7.1.1.

A.4.7.2.2.2	Test Parameters

In this test case all cells are on the same carrier frequency. The absolute accuracy of SS-RSRQ intra-frequency measurement is test by using the parameters in Table A.4.7.2.2.2-1 for SCS=15kHz and Table A.4.7.2.2.2-2 for SCS=30kHz. In all test cases, Cell 2 is the PSCell and Cell 3 is the target cell. Cell 1 is the E-UTRA cell which specific test parameters for this test case are specified in Table A.3.7.2.1-1.

Table A.4.7.2.2.2-1: SS-RSRQ TDD Intra frequency test parameters for SCS=15kHz

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 2 | Cell 3 | Cell 2 | Cell 3 | Cell 2 | Cell 3 |
| Frequency Range |  |  | FR1 |  | FR1 |  | FR1 |  |
| BWchannel |  | MHz | 10 |  | 10 |  | 10 |  |
| PDSCH Reference measurement channel |  |  | SR.1.xTDD | - | SR.1.x TDD | - | SR.1.x TDD | - |
| OCNG Patterns |  |  | OP.x | OP.y | OP.x | OP.y | OP.x | OP.y |
| SMTC Configuration |  |  | SMTC.1 FR1 |  | SMTC.1 FR1 |  | SMTC.1 FR1 |  |
| SS-RSSI-Measurement |  |  | N/A |  | N/A |  | N/A |  |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |
| PBCH_DMRS_RA |  |  |  |  |  |  |  |  |
| PBCH_DMRS_RB |  |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |  |
| Note2 | NR_TDD_FR1_A | dBm/15kHz | -84.94 |  | -100.85 |  | -115 |  |
|  | NR_TDD_FR1_C |  |  |  |  |  | -114 |  |
|  | NR_TDD_FR1_D |  |  |  |  |  | -113 |  |
|  | NR_TDD_FR1_E |  |  |  |  |  | -112 |  |
|  |  | dB | -1.76 |  | -4.7 |  | -5.46 |  |
| SS-RSRPNote3 | NR_TDD_FR1_A | dBm/15kHz | -81.94 |  | -103.75 |  | -119 |  |
|  | NR_TDD_FR1_C |  |  |  |  |  | -118 |  |
|  | NR_TDD_FR1_D |  |  |  |  |  | -117 |  |
|  | NR_TDD_FR1_E |  |  |  |  |  | -116 |  |
| SS-RSRQNote3 | NR_TDD_FR1_A | dB | -14.77 |  | -16.76 |  | -17.34 |  |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  |
|  | NR_TDD_FR1_D |  |  |  |  |  |  |  |
|  | NR_TDD_FR1_E |  |  |  |  |  |  |  |
| IoNote3 | NR_TDD_FR1_A | dBm/9.36 MHz | -50 |  | -69.83 |  | -84.5 |  |
|  | NR_TDD_FR1_C |  |  |  |  |  | -83.5 |  |
|  | NR_TDD_FR1_D |  |  |  |  |  | -82.5 |  |
|  | NR_TDD_FR1_E |  |  |  |  |  | -81.5 |  |
|  |  | dB | 3 | 3 | -2.9 | -2.9 | -4 | -4 |
| Propagation condition |  | - | AWGN |  | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRQ, SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRQ and SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in Section 3.5.2. |  |  |  |  |  |  |  |  |

Table A.4.7.2.2.2-2: SS-RSRQ TDD Intra frequency test parameters for SCS=30kHz

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 2 | Cell 3 | Cell 2 | Cell 3 | Cell 2 | Cell 3 |
| Frequency Range |  |  | FR1 |  | FR1 |  | FR1 |  |
| BWchannel |  | MHz | 40 |  | 40 |  | 40 |  |
| PDSCH Reference measurement channel |  |  | SR.1.xTDD | - | SR.1.x TDD | - | SR.1.x TDD | - |
| OCNG Patterns |  |  | OP.x | OP.y | OP.x | OP.y | OP.x | OP.y |
| SMTC Configuration |  |  | SMTC.2 FR1 |  | SMTC.2 FR1 |  | SMTC.2 FR1 |  |
| SS-RSSI-Measurement |  |  | N/A |  | N/A |  | N/A |  |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |
| PBCH_DMRS_RA |  |  |  |  |  |  |  |  |
| PBCH_DMRS_RB |  |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |  |
| Note2 | NR_TDD_FR1_A | dBm/30kHz | -88.05 |  | -104.85 |  | -115 |  |
|  | NR_TDD_FR1_C |  |  |  |  |  | -114 |  |
|  | NR_TDD_FR1_D |  |  |  |  |  | -113 |  |
|  | NR_TDD_FR1_E |  |  |  |  |  | -112 |  |
|  |  | dB | -1.76 |  | -4.7 |  | -5.46 |  |
| SS-RSRPNote3 | NR_TDD_FR1_A | dBm/30kHz | -85.05 |  | -107.75 |  | -119 |  |
|  | NR_TDD_FR1_C |  |  |  |  |  | -118 |  |
|  | NR_TDD_FR1_D |  |  |  |  |  | -117 |  |
|  | NR_TDD_FR1_E |  |  |  |  |  | -116 |  |
| SS-RSRQNote3 | NR_TDD_FR1_A | dB | -14.77 |  | -16.76 |  | -17.34 |  |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  |
|  | NR_TDD_FR1_D |  |  |  |  |  |  |  |
|  | NR_TDD_FR1_E |  |  |  |  |  |  |  |
| IoNote3 | NR_TDD_FR1_A | dBm/38.16 MHz | -50 |  | -70.7 |  | -81.41 |  |
|  | NR_TDD_FR1_C |  |  |  |  |  | -80.41 |  |
|  | NR_TDD_FR1_D |  |  |  |  |  | -79.41 |  |
|  | NR_TDD_FR1_E |  |  |  |  |  | -78.41 |  |
|  |  | dB | 3 | 3 | -2.9 | -2.9 | -4 | -4 |
| Propagation condition |  | - | AWGN |  | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRQ, SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRQ and SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in Section 3.5.2. |  |  |  |  |  |  |  |  |

A.4.7.2.2.3	Test Requirements

The SS-RSRQ measurement accuracy shall fulfil the requirements in section 10.1.7.1.1.

----- << End of Change 1>> -----

A.5.7	Measurement Performance requirements

----- << Start of Change 2>> -----

A.5.7.2	SS-RSRQ

A.5.7.2.1	TDD Intra-frequency case

A.5.7.2.1.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in Clause 10.1.8.1.1.

A.5.7.2.1.2	Test Parameters

In this test case all cells are on the same carrier frequency. The absolute accuracy of SS-RSRQ intra-frequency measurement is test by using the parameters in Table A.5.7.2.1.2-1. In all test cases, Cell 2 is the PSCell and Cell 3 is the target cell. Cell 1 is the E-UTRA cell which specific test parameters for this test case are specified in Table A.3.7.2.2-1.

Table A.5.7.2.1.2-1: SS-RSRQ TDD Intra frequency test parameters for SCS=120kHz

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 2 | Cell 3 | Cell 2 | Cell 3 | Cell 2 | Cell 3 |
| Frequency Range |  |  | FR2 |  | FR2 |  | FR2 |  |
| BWchannel |  | MHz | 100 |  | 100 |  | 100 |  |
| PDSCH Reference measurement channel |  |  | SR.1.xTDD | - | SR.1.x TDD | - | SR.1.x TDD | - |
| OCNG Patterns |  |  | OP.x | OP.y | OP.x | OP.y | OP.x | OP.y |
| SMTC Configuration |  |  | SMTC.1 FR2 |  | SMTC.1 FR2 |  | SMTC.1 FR2 |  |
| SS-RSSI-Measurement |  |  | N/A |  | N/A |  | N/A |  |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |
| PBCH_DMRS_RA |  |  |  |  |  |  |  |  |
| PBCH_DMRS_RB |  |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |  |
| Note2 | NR_TDD_FR2_A | dBm/120kHz | TBD |  | TBD |  | TBD |  |
|  | NR_TDD_FR2_B |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_F |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_G |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_T |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_Y |  |  |  |  |  | TBD |  |
|  |  | dB | TBD |  | TBD |  | TBD |  |
| SS-RSRPNote3 | NR_TDD_FR2_A | dBm/120kHz | TBD |  | TBD |  | TBD |  |
|  | NR_TDD_FR2_B |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_F |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_G |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_T |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_Y |  |  |  |  |  | TBD |  |
| SS-RSRQNote3 | NR_TDD_FR2_A | dB | TBD |  | TBD |  | TBD |  |
|  | NR_TDD_FR2_B |  |  |  |  |  |  |  |
|  | NR_TDD_FR2_F |  |  |  |  |  |  |  |
|  | NR_TDD_FR2_G |  |  |  |  |  |  |  |
|  | NR_TDD_FR2_T |  |  |  |  |  |  |  |
|  | NR_TDD_FR2_Y |  |  |  |  |  |  |  |
| IoNote3 | NR_TDD_FR2_A | dBm/95.04 MHz | TBD |  | TBD |  | TBD |  |
|  | NR_TDD_FR2_B |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_F |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_G |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_T |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_Y |  |  |  |  |  | TBD |  |
|  |  | dB | 3 | 3 | -2.9 | -2.9 | -4 | -4 |
| Propagation condition |  | - | AWGN |  | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRQ, SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRQ and SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in Section 3.5.2. |  |  |  |  |  |  |  |  |

A.5.7.2.1.3	Test Requirements

The SS-RSRQ measurement accuracy shall fulfil the requirements in section 10.1.8.1.1.

----- << End of Change 2>> -----

A.6.7	Measurement Performance requirements

----- << Start of Change 3>> -----

A.6.7.2	SS-RSRQ

A.6.7.2.1	FDD Intra-frequency case

A.6.7.2.1.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in Clause 10.1.7.1.1.

A.6.7.2.1.2	Test Parameters

In this test case all cells are on the same carrier frequency. The absolute accuracy of SS-RSRQ intra-frequency measurement is test by using the parameters in Table A.6.7.2.1.2-1. In all test cases, Cell 1 is the PCell and Cell 2 is the target cell.

Table A.6.7.2.1.2-1: SS-RSRQ FDD Intra frequency test parameters for SCS=15kHz

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Frequency Range |  |  | FR1 |  | FR1 |  | FR1 |  |
| BWchannel |  | MHz | 10 |  | 10 |  | 10 |  |
| PDSCH Reference measurement channel |  |  | SR.1.xFDD | - | SR.1.x FDD | - | SR.1.x FDD | - |
| OCNG Patterns |  |  | OP.x | OP.y | OP.x | OP.y | OP.x | OP.y |
| SMTC Configuration |  |  | SMTC.1 FR1 |  | SMTC.1 FR1 |  | SMTC.1 FR1 |  |
| SS-RSSI-Measurement |  |  | N/A |  | N/A |  | N/A |  |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |
| PBCH_DMRS_RA |  |  |  |  |  |  |  |  |
| PBCH_DMRS_RB |  |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |  |
| Note2 | NR_FDD_FR1_A | dBm/15kHz | -84.94 |  | -103.85 |  | -117 |  |
|  | NR_FDD_FR1_B |  |  |  |  |  | -116 |  |
|  | NR_FDD_FR1_E |  |  |  |  |  | -115 |  |
|  | NR_FDD_FR1_G |  |  |  |  |  | -114 |  |
|  | NR_FDD_FR1_H |  |  |  |  |  | -113 |  |
|  |  | dB | -1.76 |  | -4.7 |  | -5.46 |  |
| SS-RSRPNote3 | NR_FDD_FR1_A | dBm/15kHz | -81.94 |  | -106.75 |  | -121 |  |
|  | NR_FDD_FR1_B |  |  |  |  |  | -120 |  |
|  | NR_FDD_FR1_E |  |  |  |  |  | -119 |  |
|  | NR_FDD_FR1_G |  |  |  |  |  | -118 |  |
|  | NR_FDD_FR1_H |  |  |  |  |  | -117 |  |
| SS-RSRQNote3 | NR_FDD_FR1_A | dB | -14.77 |  | -16.76 |  | -17.34 |  |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  |
|  | NR_FDD_FR1_E |  |  |  |  |  |  |  |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  |
| IoNote3 | NR_FDD_FR1_A | dBm/9.36 MHz | -50 |  | -73 |  | -86.5 |  |
|  | NR_FDD_FR1_B |  |  |  |  |  | -85.5 |  |
|  | NR_FDD_FR1_E |  |  |  |  |  | -84.5 |  |
|  | NR_FDD_FR1_G |  |  |  |  |  | -83.5 |  |
|  | NR_FDD_FR1_H |  |  |  |  |  | -82.5 |  |
|  |  | dB | 3 | 3 | -2.9 | -2.9 | -4 | -4 |
| Propagation condition |  | - | AWGN |  | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRQ, SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRQ and SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in Section 3.5.2. |  |  |  |  |  |  |  |  |

A.6.7.2.1.3	Test Requirements

The SS-RSRQ measurement accuracy shall fulfil the requirements in section 10.1.7.1.1.

A.6.7.2.2	TDD Intra-frequency case

A.6.7.2.2.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in Clause 10.1.7.1.1.

A.6.7.2.2.2	Test Parameters

In this test case all cells are on the same carrier frequency. The absolute accuracy of SS-RSRQ intra-frequency measurement is test by using the parameters in Table A.6.7.2.2.2-1 for SCS=15kHz and Table A.6.7.2.2.2-2 for SCS=30kHz. In all test cases, Cell 1 is the PCell and Cell 2 is the target cell.

Table A.6.7.2.2.2-1: SS-RSRQ TDD Intra frequency test parameters for SCS=15kHz

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Frequency Range |  |  | FR1 |  | FR1 |  | FR1 |  |
| BWchannel |  | MHz | 10 |  | 10 |  | 10 |  |
| PDSCH Reference measurement channel |  |  | SR.1.xTDD | - | SR.1.x TDD | - | SR.1.x TDD | - |
| OCNG Patterns |  |  | OP.x | OP.y | OP.x | OP.y | OP.x | OP.y |
| SMTC Configuration |  |  | SMTC.1 FR1 |  | SMTC.1 FR1 |  | SMTC.1 FR1 |  |
| SS-RSSI-Measurement |  |  | N/A |  | N/A |  | N/A |  |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |
| PBCH_DMRS_RA |  |  |  |  |  |  |  |  |
| PBCH_DMRS_RB |  |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |  |
| Note2 | NR_TDD_FR1_A | dBm/15kHz | -84.94 |  | -100.85 |  | -115 |  |
|  | NR_TDD_FR1_C |  |  |  |  |  | -114 |  |
|  | NR_TDD_FR1_D |  |  |  |  |  | -113 |  |
|  | NR_TDD_FR1_E |  |  |  |  |  | -112 |  |
|  |  | dB | -1.76 |  | -4.7 |  | -5.46 |  |
| SS-RSRPNote3 | NR_TDD_FR1_A | dBm/15kHz | -81.94 |  | -103.75 |  | -119 |  |
|  | NR_TDD_FR1_C |  |  |  |  |  | -118 |  |
|  | NR_TDD_FR1_D |  |  |  |  |  | -117 |  |
|  | NR_TDD_FR1_E |  |  |  |  |  | -116 |  |
| SS-RSRQNote3 | NR_TDD_FR1_A | dB | -14.77 |  | -16.76 |  | -17.34 |  |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  |
|  | NR_TDD_FR1_D |  |  |  |  |  |  |  |
|  | NR_TDD_FR1_E |  |  |  |  |  |  |  |
| IoNote3 | NR_TDD_FR1_A | dBm/9.36 MHz | -50 |  | -69.83 |  | -84.5 |  |
|  | NR_TDD_FR1_C |  |  |  |  |  | -83.5 |  |
|  | NR_TDD_FR1_D |  |  |  |  |  | -82.5 |  |
|  | NR_TDD_FR1_E |  |  |  |  |  | -81.5 |  |
|  |  | dB | 3 | 3 | -2.9 | -2.9 | -4 | -4 |
| Propagation condition |  | - | AWGN |  | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRQ, SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRQ and SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in Section 3.5.2. |  |  |  |  |  |  |  |  |

Table A.6.7.2.2.2-2: SS-RSRQ TDD Intra frequency test parameters for SCS=30kHz

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Frequency Range |  |  | FR1 |  | FR1 |  | FR1 |  |
| BWchannel |  | MHz | 40 |  | 40 |  | 40 |  |
| PDSCH Reference measurement channel |  |  | SR.1.xTDD | - | SR.1.x TDD | - | SR.1.x TDD | - |
| OCNG Patterns |  |  | OP.x | OP.y | OP.x | OP.y | OP.x | OP.y |
| SMTC Configuration |  |  | SMTC.2 FR1 |  | SMTC.2 FR1 |  | SMTC.2 FR1 |  |
| SS-RSSI-Measurement |  |  | N/A |  | N/A |  | N/A |  |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |
| PBCH_DMRS_RA |  |  |  |  |  |  |  |  |
| PBCH_DMRS_RB |  |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |  |
| Note2 | NR_TDD_FR1_C | dBm/30kHz | -88.05 |  | -104.85 |  | -114 |  |
|  | NR_TDD_FR1_D |  |  |  |  |  | -113 |  |
|  | NR_TDD_FR1_E |  |  |  |  |  | -112 |  |
|  |  | dB | -1.76 |  | -4.7 |  | -5.46 |  |
| SS-RSRPNote3 | NR_TDD_FR1_C | dBm/30kHz | -85.05 |  | -107.75 |  | -118 |  |
|  | NR_TDD_FR1_D |  |  |  |  |  | -117 |  |
|  | NR_TDD_FR1_E |  |  |  |  |  | -116 |  |
| SS-RSRQNote3 | NR_TDD_FR1_C | dB | -14.77 |  | -16.76 |  | -17.34 |  |
|  | NR_TDD_FR1_D |  |  |  |  |  |  |  |
|  | NR_TDD_FR1_E |  |  |  |  |  |  |  |
| IoNote3 | NR_TDD_FR1_C | dBm/38.16 MHz | -50 |  | -70.7 |  | -80.41 |  |
|  | NR_TDD_FR1_D |  |  |  |  |  | -79.41 |  |
|  | NR_TDD_FR1_E |  |  |  |  |  | -78.41 |  |
|  |  | dB | 3 | 3 | -2.9 | -2.9 | -4 | -4 |
| Propagation condition |  | - | AWGN |  | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRQ, SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRQ and SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in Section 3.5.2. |  |  |  |  |  |  |  |  |

A.6.7.2.2.3	Test Requirements

The SS-RSRQ measurement accuracy shall fulfil the requirements in section 10.1.7.1.1.

----- << End of Change 3>> -----

A.7.7	Measurement Performance requirements

----- << Start of Change 4>> -----

A.7.7.2	SS-RSRQ

A.7.7.2.1	TDD Intra-frequency case

A.7.7.2.1.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in Clause 10.1.8.1.1.

A.7.7.2.1.2	Test Parameters

In this test case all cells are on the same carrier frequency. The absolute accuracy of SS-RSRQ intra-frequency measurement is test by using the parameters in Table A.7.7.2.1.2-1. In all test cases, Cell 1 is the PCell and Cell 2 the target cell.

Table A.7.7.2.1.2-1: SS-RSRQ TDD Intra frequency test parameters for SCS=120kHz

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Frequency Range |  |  | FR2 |  | FR2 |  | FR2 |  |
| BWchannel |  | MHz | 100 |  | 100 |  | 100 |  |
| PDSCH Reference measurement channel |  |  | SR.1.xTDD | - | SR.1.x TDD | - | SR.1.x TDD | - |
| OCNG Patterns |  |  | OP.x | OP.y | OP.x | OP.y | OP.x | OP.y |
| SMTC Configuration |  |  | SMTC.1 FR2 |  | SMTC.1 FR2 |  | SMTC.1 FR2 |  |
| SS-RSSI-Measurement |  |  | N/A |  | N/A |  | N/A |  |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |
| PBCH_DMRS_RA |  |  |  |  |  |  |  |  |
| PBCH_DMRS_RB |  |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |  |
| Note2 | NR_TDD_FR2_A | dBm/120kHz | TBD |  | TBD |  | TBD |  |
|  | NR_TDD_FR2_B |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_F |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_G |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_T |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_Y |  |  |  |  |  | TBD |  |
|  |  | dB | TBD |  | TBD |  | TBD |  |
| SS-RSRPNote3 | NR_TDD_FR2_A | dBm/120kHz | TBD |  | TBD |  | TBD |  |
|  | NR_TDD_FR2_B |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_F |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_G |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_T |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_Y |  |  |  |  |  | TBD |  |
| SS-RSRQNote3 | NR_TDD_FR2_A | dB | TBD |  | TBD |  | TBD |  |
|  | NR_TDD_FR2_B |  |  |  |  |  |  |  |
|  | NR_TDD_FR2_F |  |  |  |  |  |  |  |
|  | NR_TDD_FR2_G |  |  |  |  |  |  |  |
|  | NR_TDD_FR2_T |  |  |  |  |  |  |  |
|  | NR_TDD_FR2_Y |  |  |  |  |  |  |  |
| IoNote3 | NR_TDD_FR2_A | dBm/95.04 MHz | TBD |  | TBD |  | TBD |  |
|  | NR_TDD_FR2_B |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_F |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_G |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_T |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_Y |  |  |  |  |  | TBD |  |
|  |  | dB | 3 | 3 | -2.9 | -2.9 | -4 | -4 |
| Propagation condition |  | - | AWGN |  | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRQ, SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRQ and SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in Section 3.5.2. |  |  |  |  |  |  |  |  |

A.7.7.2.1.3	Test Requirements

The SS-RSRQ measurement accuracy shall fulfil the requirements in section 10.1.8.1.1.

----- << End of Change 4>> -----