# 3GPP TSG-RAN WG4 Meeting #88bis                                                            R4-1812543

# Chengdu, China, 8th - 12th Oct, 2018

Source: 	ZTE Corporation

Title: 	TP to TS38.141-2: OTA Transmitter ON/OFF power (Section 6.5.1)

Agenda Item:	7.9.4.1

Document for:	Approval

1	Introduction

- In contribution [1], some corrections on the measurement time for transmitter OFF power for TS38.104 are given, and the intention for this TP for TS38.141-2[2] is to align with the corrections in [1]. In addition, the step of multi-band testing is deleted for BS type 2-O since it is no need to consider multi-band operation for FR2.

2	Reference

[1] R4-1812537, Draft CR to TS 38.104: Transmitter OFF power for CA (Section 6.4.1), ZTE Corporation

[2] TS38.141-2, v1.0.0

--------------Start of TP  -------------

6.5	OTA transmit ON/OFF power

6.5.1	OTA transmitter OFF power

6.5.1.1	Definition and applicability

OTA transmitter OFF power requirements apply only to TDD operation of NR BS.

OTA transmitter OFF power is defined as the mean power measured over 70/Nµs filtered with a square filter of bandwidth equal to the transmission bandwidth configuration of the BS (BWConfig) centred on the assigned channel frequency during the transmitter OFF period. N = SCS/15, where SCS is Sub Carrier Spacing in kHz.

For BS supporting intra-band contiguous CA, the transmitter OFF power is defined as the mean power measured over 70/N µs filtered with a square filter of bandwidth equal to the Aggregated BS Channel Bandwidth BWChannel_CA centred on (Fedge_high+Fedge_low)/2 during the transmitter OFF period. N = SCS/15, where SCS is the smallest Sub Carrier Spacing in kHz used in the Aggregated BS Channel Bandwidth.

For BS type 1-O, the transmitter OFF power is defined as the output power at the co-location test antenna conducted output(s). For BS type 2-O the transmitter OFF power is defined as TRP.

For multi-band co-location test antenna conducted output(s), the requirement is only applicable during the transmitter OFF period in all supported operating bands.

For single band co-location test antenna conducted output(s) supporting transmission in multiple operating bands, the requirement is applicable per supported operating band.

For multi-band RIBs, the requirement is only applicable during the transmitter OFF period in all supported operating bands.

For single band RIBs supporting transmission in multiple operating bands, the requirement is applicable per supported operating band.

6.5.1.2	Minimum requirement

The minimum requirement for BS type 1-O is in TS 38.104 [2], subclause 9.5.2.2.

The minimum requirement for BS type 2-O is in TS 38.104 [2], subclause 9.5.2.3.

6.5.1.3	Test purpose

The purpose of this test is to verify the OTA transmitter OFF power is within the limits of the minimum requirements.

6.5.1.4	Method of test

6.5.1.4.1	Initial conditions

Test environment: Normal; see annex B.2.

RF channels to be tested: M; see subclause 4.9.1.

Base Station RF Bandwidth positions to be tested:

-	MRFBW in single band operation, see subclause 4.9.1;

-	BRFBW_T'RFBW and B'RFBW_TRFBW in multi-band operation; see subclause 4.9.1.

6.5.1.4.2	Procedure

6.5.1.4.2.1	General procedure

OTA test requires correct use of an appropriate test facility which has been calibrated and is capable of performing measurements within the measurement uncertainties in subclause 4.1.2.

1)	Place the BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (Dx.y) of the BS with the test system.

3)	Set the BS in the direction of the declared beam peak direction of the beam direction pair, for the beam to be tested.

6.5.1.4.2.2	BS type 1-O

4)	Place the co-location test antenna as specified in subclause 4.12.

5)	Configure the beam peak direction of the BS according to the declared beam direction pair.

6)	Set the BS to transmit according to the applicable test configuration in subclause 4.8 using the corresponding test models or set of physical channels in subclause 4.9.3.

7)	Measure the mean power spectrum density measured over 70/N μs filtered with a square filter of bandwidth equal to the RF bandwidth of the NR BS centred on the central frequency of the RF bandwidth. 70/N μs average window centre is set from 35/N μs after end of one transmitter ON period + 10 μs to 35/N μs before start of next transmitter ON period - 10 μs. N = SCS/15, where SCS is Sub Carrier Spacing in kHz.

8)	For an NR BS supporting contiguous CA, measure the mean power spectral density over 70/N μs filtered with a square filter of bandwidth equal to the Aggregated Channel Bandwidth BWChannel_CA centred on (Fedge_high+Fedge_low)/2. 70/N μs average window centre is set from 35/N μs after end of one transmitter ON period + 10 μs to 35/N μs before start of next transmitter ON period - 10 μs. N = SCS/15, where SCS is the smallest Sub Carrier Spacing in kHz used in the Aggregated BS Channel Bandwidth.

In addition, for a multi-band RIB, the following steps shall apply:

9)	For a multi-band RIB and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

6.5.1.4.2.2	BS type 2-O

[4)	Configure the beam peak direction of the NR BS according to the declared beam direction pair.

5)	Set the NR BS to transmit according to the applicable test configuration in subclause 4.8 using the corresponding test models or set of physical channels in subclause 4.9.

In addition, for an NR BS declared to be capable of multi-carrier and/or CA operation use the applicable test signal configuration and corresponding power setting specified in subclause 4.7.3.

6)	Set the NR BS in the direction of the appropriated TRP measurement grid (see annex xx).

7)	Measure EIRP by either a) or b) below. EIRP is measured over 70/N μs filtered with a square filter of bandwidth equal to the RF bandwidth of the NR BS centred on the central frequency of the RF bandwidth. 70/N μs average window centre is set from 35/N μs after end of one transmitter ON period + 3μs to 35/N μs before start of next transmitter ON period - 3μs. N = SCS/15, where SCS is Sub Carrier Spacing in kHz.

a)	If the test facility only supports single polarization, then measure EIRP with the test facility's test antenna/probe polarization matched to the NR BS.

b)	If the test facility supports dual polarization then measure total EIRP for two orthogonal polarizations (denoted p1 and p2) and calculate total radiated transmit power for particular beam direction pair as EIRP = EIRPp1 + EIRPp2.

8)	For an NR BS supporting contiguous CA, measure EIRP by either a) or b) below. EIRP is measured over 70/N μs filtered with a square filter of bandwidth equal to the Aggregated Channel Bandwidth BWChannel_CA centred on (Fedge_high+Fedge_low)/2. 70/N μs average window centre is set from 35/N μs after end of one transmitter ON period + 3 μs to 35/N μs before start of next transmitter ON period - 3 μs. N = SCS/15, where SCS is the smallest Sub Carrier Spacing in kHz used in the Aggregated BS Channel Bandwidth.

a)	If the test facility only supports single polarization, then measure EIRP with the test facility's test antenna/probe polarization matched to the NR BS.

b)	If the test facility supports dual polarization then measure total EIRP for two orthogonal polarizations (denoted p1 and p2) and calculate total radiated transmit power for particular beam direction pair as EIRP = EIRPp1 + EIRPp2.

9)	Repeat step 6-8 for all directions in the appropriated TRP measurement grid needed for full TRP estimation (see annex xx).

10)	Calculate TRP using the EIRP measurements.

]

6.5.1.5	Test requirements

6.5.1.5.1	BS type 1-O

The measured mean power spectral density according to subclause 6.5.1.4.2 shall be less than -102.6 dBm/MHz for carrier frequency f ≤ 3.0 GHz.

The measured mean power spectral density according to subclause 6.5.1.4.2 shall be less than -102.4 dBm/MHz for carrier frequency 3.0 GHz < f ≤ 6.0 GHz.

For multi-band RIB, the requirement is only applicable during the transmitter OFF period in all supported operating bands.

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in subclause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.1.

6.5.1.5.2	BS type 2-O

The measured mean power spectral density according to subclause 9.5.1.4.2 shall be less than [-36+XX] dBm/MHz for carrier frequency f ≤ XX GHz.

The measured mean power spectral density according to subclause 9.5.1.4.2 shall be less than [-36+XX] dBm/MHz for carrier frequency XX GHz < f ≤ XX GHz.

The measured mean power spectral density according to subclause 9.5.1.4.2 shall be less than [-36+XX] dBm/MHz for carrier frequency XX GHz < f ≤ XX GHz.

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in subclause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.1.

--------------End of TP ------------