3GPP TR 36.716-01-01 V0.1.0 (2018-10)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Radio Access Networks;

LTE Intra-band CA for xDL/yUL including contiguous and non-contiguous spectrum, (x>=y)

(Release 16)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

<keyword[, keyword]>

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2016, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	5

2	References	5

3	Definitions, symbols and abbreviations	5

3.1	Definitions	5

3.2	Symbols	6

3.3	Abbreviations	6

4	Background	6

4.1	TR Maintenance	6

5	Intra-Band Contiguous Carrier Aggregation: Specific Band Combination Part	6

5.1	CA_3DL_41D_3UL_41D_BCS0	6

5.1.1	Channel bandwidths per operating band	6

5.1.2	UE maximum output power	6

5.1.3	Spurious emission band UE co-existence	7

5.1.4	UE maximum output power with additional requirements	7

5.1.5	Additional Spectrum emission mask for CA_41D	8

6	Intra-Band Non-Contiguous Carrier Aggregation: Specific Band Combination Part	10

6.1	CA_xDL_a-a_yUL_b-b_BCSz	10

6.1.1	Channel bandwidths per operating band for CA	10

6.1.2	UE co-existence studies	10

6.1.3	∆TIB and ∆RIB values	10

6.1.4	REFSENS requirements	10

Annex A: Change history	10

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document is a technical report for Intra-band Carrier Aggregation Rel-16 for xDL/yUL including contiguous and non-contiguous spectrum under Rel-16 time frame. The purpose is to gather the relevant background information and studies in order to address Intra-band Carrier Aggregation requirements for the Rel-16 band combinations in Table 1-1 and Table 1-2.

Table 1-1: Release 15 Intra-band carrier contiguous aggregation combinations

| CA combination | REL-indep. from |
| --- | --- |
| CA_3DL_41D_3UL_41D_BCS0 | REL-12 |
| CA_4DL_41E_3UL_41D_BCS0 | REL-12 |
| CA_5DL_41F_3UL_41D_BCS0 | REL-12 |
| CA_2DL_48C_2UL_48C_BCS0 | REL-12 |

Table 1-2: Release 15 Intra-band carrier non-contiguous aggregation combinations

| CA combination | REL-indep. from |
| --- | --- |
| CA_3DL_48A-48C_2UL_48C_BCS0 | REL-12 |
| CA_4DL_48C-48C_2UL_48C_BCS0 | REL-12 |

This TR contains a band specific combination part. The actual requirements are added to the corresponding technical specifications.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]		RP-180806, “New WID on Rel16 LTE Intra-band CA for x DL/y UL including contiguous and non-contiguous spectrum(x>=y)”, RAN#80, Ericsson

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

# 4	Background

The present document is a technical report for Intra-band Carrier Aggregation under Rel-16 timeframe. The document covers each band combination specific issues (i.e. one sub-clause defined per band combination)

## 4.1	TR Maintenance

A single company is responsible for introducing all approved TPs in the current TR, i.e. TR editor. However, it is the responsibility of the contact person of each band combination to ensure that the TPs related to the band combination have been implemented.

# 5	Intra-Band Contiguous Carrier Aggregation: Specific Band Combination Part

## 5.1	CA_3DL_41D_3UL_41D_BCS0

### 5.1.1	Channel bandwidths per operating band

Table 5.1.1-1 defines the supported E-UTRA CA configurations and bandwidth combination set for LTE-advanced 3UL intra-band contiguous CA in Band 41.

Table 5.1.1-1: E-UTRA CA configurations and bandwidth combination sets for CA_41D

|  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Uplink CA configurations (NOTE 3) | Component carriers in order of increasing carrier frequency |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] Channel bandwidths for carrier [MHz] |  |  |
| CA_41D | CA_41D | 10 | 20 | 15 | 60 | 0 |
|  |  | 10 | 15, 20 | 20 |  |  |
|  |  | 15 | 20 | 10, 15 |  |  |
|  |  | 15 | 10, 15, 20 | 20 |  |  |
|  |  | 20 | 15, 20 | 10 |  |  |
|  |  | 20 | 10, 15, 20 | 15, 20 |  |  |

### 5.1.2	UE maximum output power

The UE maximum output power for CA_41D is proposed to reuse requirement for bandwidth class C.

Table 5.1.2-1 CA UE Power Class for intra-band contiguous CA_41D

| E-UTRA CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_41D |  |  |  |  | 23 | +2/-22 |  |  |
| NOTE 2: If all transmitted resource blocks (Figure 5.6A-1) over all component carriers are confined within FUL_low and FUL_low + 4 MHz or/and FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |  |  |  |  |  |  |

### 5.1.3	Spurious emission band UE co-existence

Table 5.1.3-1 shows the spurious emission band UE co-existence for CA_41D.

Table 5.1.3-1: Spurious emission band UE co-existence for CA_41D

| E-UTRA CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_41D | E-UTRA Band 1, 2, 3, 4, 5, 8, 10, 12, 13 , 14, 17, 24, 25, 26, 27, 28, 29, 30, 34, 39, 40, 42, 44, 65, 66, 70 | FDL_low | - | FDL_high | -50 | 1 |  |

### 5.1.4	UE maximum output power with additional requirements

Simulations were completed to determine A-MPR for 3CC in B41D. It was used a validated PA model with PA calibration point MPR0 with QPSK 18RB single CC waveform to meet minimum general requirements for E-UTRA ACLR, UTRA1 ACLR, UTRA2 ACLR and General Spurious emissions. Carrier Leakage and Image suppression were both at -25dBc with Counter IM3 at -60dBc.

Supporting simulation results are shown in the appendix.

Proposal to use A-MPR for contiguous allocations as shown in Table 5.1.4-1

Table 5.1.4-1: Contiguous Allocation A-MPR for CA_NS_04 (power class 3)

| CA Bandwidth Class D | RBStart | LCRB [RBs] | RBstart + LCRB [RBs] | A-MPR for QPSK [dB] | A-MPR for 16 QAM, 64 QAM and 256 QAM [dB] |
| --- | --- | --- | --- | --- | --- |
| 50 RB / 75 RB/ 100 RB | 0 – 64 and 161 – 224 | >0 | N/A | ≤4dB | ≤4.5dB |
|  | 65 – 160 | N/A | >161 | ≤3dB | ≤3.5dB |
| 50 RB / 100 RB / 100 RB | 0 – 72 and 178 – 249 | >0 | N/A | ≤4dB | ≤4.5dB |
|  | 73 – 177 | N/A | >178 | ≤3dB | ≤3.5dB |
| 75 RB / 75 RB / 100 RB | 0 – 72 and 178 – 249 | >0 | N/A | ≤4dB | ≤4.5dB |
|  | 73 – 177 | N/A | >178 | ≤3dB | ≤3.5dB |
| 75 RB / 100 RB / 100 RB | 0 – 78 and 197 – 274 | >0 | N/A | ≤3.5dB | ≤4dB |
|  | 79 - 196 | N/A | >197 | ≤2.5dB | ≤3dB |
| 100 RB / 100 RB / 100 RB | 0 – 96 and 204 – 300 | >0 | N/A | ≤3.5dB | ≤4dB |
|  | 97– 203 | N/A | >204 | ≤2.5dB | ≤3dB |
| NOTE 1: RBstart indicates the lowest RB index of transmitted resource blocks NOTE 2: LCRB is the length of a contiguous resource block allocation NOTE 3: For intra-subframe frequency hopping which intersects regions, notes 1 and 2 apply on a per slot basis. For intra-slot or intra-subslot frequency hopping which intersects regions, notes 1 and 2 apply on a per Tno_hopping basis.. NOTE 4: For intra-subframe frequency hopping which intersects regions, the larger A-MPR value may be applied for both slots in the subframe. For intra-slot frequency hopping which intersects regions, the larger A-MPR value may be applied for the slot. For intra-subslot frequency hopping which intersects regions, the larger A-MPR value may be applied for the subslot. |  |  |  |  |  |

Non-contiguous Allocations

For 3CC, there will be extra back-off required because of the triple beat product, even though the PSD is lower for each allocation in each CC. A simple study shows that the product is ~1.7dB higher than the equivalent 2 tone IMD product in the 2CC case for the same power in the 2CC and 3CC case. The back off required, assuming 3:1 will be ~0.57dB. A simple simulation study shows this effect in Figure 1, and a more comprehensive simulation was completed and shown in the appendix. As a result, the specification is bumped up by 0.5dB for non-contiguous allocations.

Figure 5.1.4-2 is comparing Power Back-off for the same non-contiguous allocation ratio for 2CC Vs 3CC (left). Comprehensive 3CC non-contiguous simulation with proposed spec (right).

Figure 5.1.4-2

Proposal to use A-MPR algorithm below for non-contiguous allocations.

If the UE is configured to CA_41D or any uplink inter-band CA configuration containing CA_41D and it receives IE CA_NS_04 the allowed maximum output power reduction applied to transmissions on two serving cells assigned to Band 41 with non-contiguous resource allocation is defined as follows for UE power class 3

A-MPR = CEIL {MA, 0.5}

Where MA is defined as follows:

MA 	= 	11.5,	 				0≤ A < 0.05

= -55.0A + 14.25,		0.05≤ A < 0.15

= -4.0A + 6.60, 			0.15≤ A < 0.40

= -0.833A + 5.333,		0.40 ≤ A ≤ 1

Where A = NRB_alloc / NRB_agg.

### 5.1.5	Additional Spectrum emission mask for CA_41D

The FCC emission requirements defines emission boundaries as a function of emission bandwidth.  For the purpose of deriving 3GPP specifications, the occupied bandwidth has been taken as equivalent to the FCC definition of emission bandwidth.  The existing specification for CA_NS_04 emissions only considers uplink carrier aggregation class C with two component carriers.  Therefore, the emission requirement is symmetric; that is, the emission requirement for BWA+BWB is the same as it is for BWA+BWB.  However, when extending the same emission table to bandwidth class D with three component carriers, the symmetry does not hold.  For example, the emission requirement for BWA+BWB+BWC is not necessarily the same as that for BWA+BWC+BWB.  The reason is that the channel spacing between carriers, and therefore the overall occupied bandwidth, is not necessarily the same.  In this example, the spacing between AB and BC is different than the spacing between AC and CB (or BC).  Thus, we define the CA_NS_04 SEM table for CA_41D as shown below.

It is observed that the requirement within the first MHz of the channel edge is progressively more challenging with increasing bandwidth.  The FCC requirement in the first MHz allows for the use of a measurement bandwidth that is 2% of the emission bandwidth requiring an attenuation of 40+10logP while 3GPP specifications have typically used 30 kHz measurement bandwidth.  Given the larger channel bandwidths for uplink bandwidth class D, it is proposed to increase the measurement bandwidth to 300 kHz to more closely reflect a 2% emission bandwidth for the bandwidths of interest.  Note that this formulation still complies with the -13 dBm requirement in 1% emission bandwidth for the range from 2495 – 2496 MHz as required by FCC rules, for the CA_41D bandwidths of interest.

Table 5.1.5-1: Additional SEM requirements for CA_41D and CA_NS_04

| Spectrum emission limit [dBm]/BWChannel_CA |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 75+50+100 or 100+50+75 RB (44.15 MHz) | 50+75+100 or 100+75+50 RB (44.6 MHz) | 75+100+50 or 50+100+75 RB (44.75 MHz) | 100+50+100 RB (48.8 MHz) | 75+100+75 or 100+100+50 or 50+100+100 RB (49.7 MHz) | 75+75+100 or 100+75+75 RB (49.85 MHz) | Measurement bandwidth |
| 0-1 | -14.5 | -14.5 | -14.5 | -15 | -15 | -15 | 300 kHz |
| 1-5 | -10 | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| 5-42.15 | -13 | -13 | -13 | -13 | -13 | -13 | 1 MHz |
| 42.15-42.6 | -25 | -13 | -13 | -13 | -13 | -13 | 1 MHz |
| 42.6-42.75 | -25 | -25 | -13 | -13 | -13 | -13 | 1 MHz |
| 42.75-46.8 | -25 | -25 | -25 | -13 | -13 | -13 | 1 MHz |
| 46.8-47.7 | -25 | -25 | -25 | -25 | -13 | -13 | 1 MHz |
| 47.7-47.85 | -25 | -25 | -25 | -25 | -25 | -13 | 1 MHz |
| 47.85-49.15 | -25 | -25 | -25 | -25 | -25 | -25 | 1 MHz |
| 49.15-49.6 |  | -25 | -25 | -25 | -25 | -25 | 1 MHz |
| 49.6-49.75 |  |  | -25 | -25 | -25 | -25 | 1 MHz |
| 49.75-53.8 |  |  |  | -25 | -25 | -25 | 1 MHz |
| 53.8-54.7 |  |  |  |  | -25 | -25 | 1 MHz |
| 54.7-54.85 |  |  |  |  |  | -25 | 1 MHz |
| Spectrum emission limit [dBm]/BWChannel_CA |  |  |  |  |  |  |  |
| ΔfOOB (MHz) | 100+75+100 RB (54.2 MHz) | 75+100+100 or 100+100+75 RB (54.65 MHz) | 100+100+100 RB (59.6 MHz) |  |  |  | Measurement bandwidth |
| 0-1 | -15.5 | -15.5 | -16 |  |  |  | 300 kHz |
| 1-5 | -10 | -10 | -10 |  |  |  | 1 MHz |
| 5-52.2 | -13 | -13 | -13 |  |  |  | 1 MHz |
| 52.2-52.65 | -25 | -13 | -13 |  |  |  | 1 MHz |
| 52.65-57.6 | -25 | -25 | -13 |  |  |  | 1 MHz |
| 57.6-59.2 | -25 | -25 | -25 |  |  |  | 1 MHz |
| 59.2-59.65 |  | -25 | -25 |  |  |  | 1 MHz |
| 59.65-64.6 |  |  | -25 |  |  |  | 1 MHz |

Additionally, a spurious emission requirement is also applicable when CA_NS_04 is signaled.  This requirement also applies for CA_41D.

Table 5.1.5-2: Additional spurious emissions requirements for CA_NS_04

| Frequency band (MHz) | Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 2490.5 ≤ f < 2495 | -13 | 1 MHz |
| 0 < f < 2490.5 | -25 | 1 MHz |

# 6	Intra-Band Non-Contiguous Carrier Aggregation: Specific Band Combination Part

## 6.1	CA_xDL_a-a_yUL_b-b_BCSz

### 6.1.1	Channel bandwidths per operating band for CA

<Text will be added.>

### 6.1.2	UE co-existence studies

<Text will be added.>

### 6.1.3	∆TIB and ∆RIB values

<Text will be added if it’s necessary>

### 6.1.4	REFSENS requirements

<Text will be added if it’s necessary>

# Annex A: Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| 2018-08 | 3GPP RAN4#88 | R4-1810380 |  |  | Initial TR skeleton |  | 0.0.1 |
| 2018-10 | 3GPP RAN4#88bis | R4-1812778 |  |  | Transferred CA_3DL_41D_3UL_41D_BCS0 chapters from Rel-15 TR 36.715-00-00 Implemented TP´s from RAN4 #88: R4-1810383, “Scope TP from RAN 80 for 36.716-01-01”, Ericsson R4-1810896, “A-MPR B41D Power Class 3”, Qualcomm Tech. Netherlands B.V | 0.0.1 | 0.1.0 |