3GPP TSG-RAN WG4 Meeting #88bis	R4-1812813

Chengdu, China, 8 – 12 October 2018

Agenda Item:	5.3.2

Source:	Ericsson

Title:	Test case for MSG3-based channel quality report

Document for:	Discussion

# 1	Introduction

RAN4#88 agreed with the requirement of MSG3-based channel quality reporting [1]. This contribution discusses the corresponding test case.

# 2	Discussion

## 2.1	MSG3-based channel quality reporting requirements

MSG3-based channel quality report functionality consists of the quality measurement and reporting on MSG3. As illustrated in Figure 1, in prior to the random access preamble transmission (MSG1), UE first estimates the CE level to decide which NPRACH resource is used for the preamble transmission (T1). If eNB can detect the preamble, eNB transmits the random access response (RAR or MSG2), consisting of NPDCCH and NPDSCH. After that UE transmits NPUSCH format 1 for RRC connection request (MSG3) where the NPUSCH transmission timing (k0) is specified by the eNB. According to the LS from RAN1, at the MSG3 transmission, UE optionally reports the NPDCCH repetition number with that UE can decode NPDCCH with BLER of 1%.

Figure 1	UE procedure for reporting channel quality in MSG3.

To report the channel quality, RAN2 has decided to introduce CQI-NPDCCH-NB and CQI-NPDCCH-Short-NB, depending on the message as shown in Table 1. Table 2 and Table 3 show the reported value for each information element [1].

Table 1	RRC message which can report channel quality.

| RRC message | Channel quality IE |
| --- | --- |
| RRCConnectionReestablishmentRequest-NB-r13 | CQI-NPDCCH-NB |
| RRCConnectionRequest-NB-r13 | CQI-NPDCCH-NB |
| RRCConnectionResumeRequest-NB-r13 | CQI-NPDCCH-NB |
| RRCConnectionReestablishmentRequest-NB-r14 | CQI-NPDCCH-Short-NB |

Table 2	Downlink channel quality measurement report mapping of CQI-NPDCCH-NB

| Reported value | NPDCCH repetition level |
| --- | --- |
| noMeasurement | No measurement reporting |
| candidateRep-A | 1 |
| candidateRep-B | 2 |
| candidateRep-C | 4 |
| candidateRep-D | 8 |
| candidateRep-E | 16 |
| candidateRep-F | 32 |
| candidateRep-G | 64 |
| candidateRep-H | 128 |
| candidateRep-I | 256 |
| candidateRep-J | 512 |
| candidateRep-K | 1024 |
| candidateRep-L | 2048 |

Table 3	Downlink channel quality measurement report mapping of CQI-NPDCCH-Short-NB

| Reported value | NPDCCH repetition level |
| --- | --- |
| noMeasurements | No measurement reporting |
| candidateRep-1 | Rmax/[8] (Note 1) |
| candidateRep-2 | Rmax |
| candidateRep-3 | [4]×Rmax (Note 2) |
| Note 1: When Rmax is less than 8, set candidateRep-1 to 1. Note 2: When Rmax is more than 512, set candidateRep-3 to 2048. |  |

RAN4 also agreed with the channel quality reporting accuracy as shown in Table 4. According this table, when Es/Iot ≥ -6dB, if UE report the repetition number R in CQI-NPDCCH-NB or CQI-NPDCCH-Short-NB, NPDCCH BLER with repetition number R shall be less than or equal to 1%, and NPDCCH BLER with repetition number [R/4] shall be more than 1%. On the other hand, when Es/Iot ≤ -6dB, if UE report the repetition number R in CQI-NPDCCH-NB or CQI-NPDCCH-Short-NB, NPDCCH BLER with repetition number R shall be less than or equal to 1%, and NPDCCH BLER with repetition number [R/8] shall be more than 1%.

Table 4	Channel quality reporting accuracy.

| NPDCCH Repetition | Pm-Dsg (%) | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  | E-UTRA operating band groups Note 2 | Minimum Io |  | Maximum Io |
|  |  | dB |  | dBm/15kHz | dBm/BWChannel | dBm/BWChannel |
| R NOTE 1 | ≤1 | -6 dB | NFDD_G | -122.9 | N/A | -70 |
| [R/4] NOTE 1 | >1 | -6 dB | NFDD_G | -122.9 | N/A | -70 |
| R NOTE 1 | ≤1 | -15 ≤ Ês/Iot ≤ -6 dB | NFDD_G | - 122.9 | N/A | -70 |
| [R/8] NOTE 1 | >1 | -15 ≤ Ês/Iot ≤ -6 dB | NFDD_G | - 122.9 | N/A | -70 |
| NOTE 1: R is the reported NPDCCH repetition level that UE has reported in CQI-NPDCCH-NB or CQI-NPDCCH-Short-NB. NOTE 2: Io is assumed to have constant EPRE across the bandwidth. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |

## 2.2	Test case design

The possible test parameters are shown in Table 5.

Table 5	Test parameters for MSG3-based channel quality report.

| Test number | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- |
| Ês/Iot | More than -6dB, e.g. -4dB | More than -6dB, e.g. -4dB | Less than -6dB, e.g., -12dB | Less than -6dB, e.g., -12dB |
| DCI format | N1 | N1 | N1 | N1 |
| Antenna configuration | 2x1 | 2x1 | 2x1 | 2x1 |
| Channel model | AWGN | AWGN | AWGN | AWGN |
| Aggregation level | 2 | 2 | 2 | 2 |
| DRX | OFF | OFF | OFF | OFF |
| Channel quality IE | CQI-NPDCCH-NB | CQI-NPDCCH-Short-NB | CQI-NPDCCH-NB | CQI-NPDCCH-Short-NB |

Procedure

- TE sets Es/Iot.

- TE receives CQI-NPDCCH-NB or CQI-NPDCCH-Short-NB depending on the RRC message

- TE transmits NPDCCH with repetition number R and verifies NPDCCH BLER does not exceed 1%.

- TE transmits NPDCCH with repetition number [R/4] for tests 1/2 or repetition number [R/8] for test 3/4 and verifies NPDCCH BLER is larger than 1%.

# 3	Conclusion

In this contribution we discuss our initial thought for test case for MSG3-based channel quality report. RAN4 need more discussion on the test procedure and test points.

# References

- R4-1810139, “Introduction of MSG3-based channel quality report for NB-IoT”, Ericsson.