3GPP TSG-RAN WG4 Meeting #88bis	R4-1812816

Chengdu, China, 8 – 12 October 2018

Agenda Item:	6.4.5.10

Source:	Ericsson

Title:	RMC for NB-IoT TDD RRM tests

Document for:	Discussion

# 1	Introduction

RAN4#88 agreed with the list for RRM test cases for Rel-15 NB-IoT [1]. Since NB-IoT supports TDD band in Rel-15, RAN4 needs to specify RMC for test cases. This contribution discusses the RMC table used for NB-IoT TDD RRM test cases.

# 2	RMC for NB-IoT TDD test

## 2.1	DL scheduling

### 2.1.1	TDD configuration

One of the difference of TDD from (HD-)FDD is we should specify the TDD UL/DL configurations and special subframe configuration. According to RAN1 specification [2], RAN1 has agreed to apply the same TDD configuration as LTE TDD configuration to NB-IoT TDD with the following restrictions:

- Uplink-downlink configuration 0 and 6 are not supported.

- UpPTS is not used for NPUSCH or NPRACH.

- DwPTS and UpPTS in special configuration 10 is not used for transmissions.

Another restriction is related to NPUSCH formats (Table 1). As it is referred to, NPUSCH with SCS=3.75kHz supports UL/DL configurations 1 and 4 only although SCS=15kHz supports UL/DL configurations 1, 2, 3, 4, and 5.

Table 1	Supported UL/DL configuration for NPUSCH format and SCS. (From TS36.211 Table 10.1.2.3-2).

| NPUSCH format |  | Supported uplink-downlink configurations |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1 | 3.75 kHz | 1, 4 | 1 | 16 | 7 |
|  | 15 kHz | 1, 2, 3, 4, 5 | 1 | 16 |  |
|  |  |  | 3 | 8 |  |
|  |  |  | 6 | 4 |  |
|  |  |  | 12 | 2 |  |
| 2 | 3.75 kHz | 1, 4 | 1 | 4 |  |
|  | 15 kHz | 1, 2, 3, 4, 5 | 1 | 4 |  |

When we review the existing TDD test cases for LTE, most test cases use TDD configuration 1 (DSUUD DSUUD) and special subframe configuration 6 (9DL : 3GP : 2UL). Since NB-IoT TDD allows to use these configurations, we discuss the DL/UL scheduling with the assumption TDD UL/DL configuration 1 and SSF configuration 6.

Proposal 1: Set TDD UL/DL configuration 1 and special subframe configuration 6 for NB-IoT TDD RRM test cases.

### 2.1.2	NPSS/NSSS/NPBCH/SIB1-NB scheduling

Table 1 shows the scheduling of NPSS/NSSS/NPBCH for NB-IoT FDD and TDD. Table 2 also shows the scheduling of SIB1-NB. For NB-IoT TDD, it is shown in the case UE will detect SIB1-NB from the scheduling information from NPSS/NSSS/NPBCH. Note RAN1 has specified another scheduling assisted by higher layer signaling [3].

Table 2	NPSS/NSSS/NPBCH scheduling

|  | NPBCH | NPSS | NSSS |
| --- | --- | --- | --- |
| TDD | SF#9 | SF#5 | SF#0 (even radio frame) |
| FDD | SF#0 | SF#5 | SF#9 (even radio frame) |

Table 3	SIB1-NB scheduling

|  | PDSCH Repetition number | Transmitted subframes |
| --- | --- | --- |
| FDD | 4 or 8 | SF#4 in even radio frame |
|  | 16 | SF#4 in even radio frame or odd radio frame, depending on cell ID |
| TDD | 4 or 8 | SF#0 in odd radio frame |
|  | 16 | SF#0 in odd radio frame, or SF#4 in even radio frame or odd radio frame, depending on cell ID |

### 2.1.3	DL scheduling

Considering the assumed TDD configuration and SIB1-NB scheduling, the DL scheduling of NB-IoT TDD RRM test is given as in Table 4. As shown in Table 3, NB-IoT TDD system can transmit SIB1-NB in subframe 0 or 4. For simplicity, we assume SIB1-NB is scheduled in SF#0.

Table 4	DL scheduling used for NB-IoT TDD RRM test cases.

| Frame | Even |  |  |  |  |  |  |  |  |  | Odd |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SF | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| DL/UL/SP | D | S | U | U | D | D | S | U | U | D | D | S | U | U | D | D | S | U | U | D |
| Channel | NSSS |  |  |  |  | NPSS |  |  |  | NPBCH | SIB1-NB |  |  |  |  | NPSS |  |  |  | NPBCH |

Proposal 2: SIB1-NB is scheduled in SF#0 in odd radio frames.

## 2.2	RMC for NB-IoT TDD RRM test cases

### 2.2.1	Test setup

When we review the existing RMC tables used for HD-FDD test cases, RMC supports 5MHz LTE system bandwidth and 10MHz LTE system bandwidth. The reason RAN4 specifies 5MHz is to support the bands not supporting 10MHz, such as B31. For NB-IoT TDD, according to TS36.101, only B41 is assumed as operating band, and in the future release, B42 and B43 could be added. If we look the supported system bandwidth, all the NB-IoT TDD bands support 10MHz system bandwidth. This means we don’t need to specify RMC with 5MHz LTE system bandwidth.

Proposal 3: RMC specifies with 10MHz LTE system bandwidth only.

Finally we need to discuss the RMC scheduling in special subframe. Since the purpose of RMC for RRM test is not to verify the NPDSCH/NPDCCH demodulation, we don’t need to transmit NPDCCH/NPDCCH in the special subframes.

Proposal 4: NPDCCH/PDSCH is transmitted in subframe 4.

Proposal 5: No NPDCCH/NPDSCH is scheduled in the special subframes.

### 2.2.2	Example of RMC table

Table 4 is an example of RMC table used for NB-IoT TDD RRM test cases. In this example, we reuse the same MCS used for HD-FDD test cases, i.e., QPSK 1/3.

Table 5	Example of RMC table for NB-IoT TDD RRM test cases.

| Parameter | Unit | Value |
| --- | --- | --- |
| NPDSCH Reference channel | - |  |
| LTE Carrier bandwidth | MHz | 10 |
| Allocated resource blocks | PRB | 1 |
| Number of transmitter antennas | - | 1 |
| Allocated subframes per Radio Frame | subframes | Note 1 |
| Modulation |  | QPSK |
| Target coding rate |  | 1/3 |
| Information Bit Payload |  |  |
| For Sub-Frame 4 | Bits | 40 Note 2 |
| For Sub-Frame 0, 5, 9 | Bits | 0 |
| For Sub-Frame 1, 6 (DwPTS) | Bits | 0 |
| Number of Code Blocks per Sub-Frame |  |  |
| For Sub-Frame 4 |  | 1 Note 2 |
| For Sub-Frame 0, 5, 9 |  | 0 |
| For Sub-Frame 1, 6 (DwPTS) |  | 0 |
| Maximum number of repetitions | - | Note 3 |
| NPDCCH start symbol | symbols | 3 |
| Cell ID | - | Note 4 |
| Note 1: Shall depend upon the NPDSCH scheduling. Note 2: Only apply for subframes scheduled with NPDSCH. Note 3: Maximum number of repetitions shall depend upon the test configuration. Note 4: Cell ID shall depend upon the test configuration. Note 5: SIB1-NB is transmitted in subframe 0 in odd radio frames. |  |  |

# 3	Conclusion

Proposal 1: Set TDD UL/DL configuration 1 and special subframe configuration 6 for NB-IoT TDD RRM test cases.

Proposal 2: SIB1-NB is scheduled in SF#0 in odd radio frames.

Proposal 3: RMC specifies with 10MHz LTE system bandwidth only.

Proposal 4: NPDCCH/PDSCH is transmitted in subframe 4.

Proposal 5: No NPDCCH/NPDSCH is scheduled in the special subframes.

# References

- R4-1811705, “List for RRM test cases for Rel-15 NB-IoT”, Ericsson.

- 3GPP TS36.211.

- 3GPP TS36.213.