3GPP TSG-RAN WG4 Meeting #88bis	R4-1812846

Chengdu, China, 8 – 12 October 2018

Agenda Item:	7.12.4

Source:	Ericsson

Title:	TDD configuration for RRM performance requirements

Document for:	Discussion

# 1	Introduction

RAN4#88 agreed with the RMC for RRM tests together with TDD configuration [1]. This contribution discusses the issue on the agreed configuration for EN-DC test scenario.

# 2	Discussion

## 2.1	TDD configuration used for EN-DC setup and RMC

RAN4#88 agreed with the test setup for LTE parameters for EN-DC tests and it is included in the latest TS38.133 [1]. Since EN-DC assumes LTE as the control plane, this setup also need LTE configuration. The tables below are the copy from the latest specification, and it is observed that TDD uplink-downlink configuration is set to 1 and TDD special subframe configuration is set to 6. This configuration is commonly used in the existing LTE RRM test cases.

Table A.3.7.2.1-1: E-UTRAN cell specific test parameters for tests with all NR cells in FR1

| Parameter | Unit | E-UTRAN Cell1 |
| --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 |
| Duplex mode |  | FDD or TDD |
| TDD special subframe configurationNote1 |  | 6 |
| TDD uplink-downlink configurationNote1 |  | 1 |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |
| OCNG PatternsNote2 |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |
| PBCH_RA | dB | 0 |
| PBCH_RB | dB |  |
| PSS_RA | dB |  |
| SSS_RA | dB |  |
| PCFICH_RB | dB |  |
| PHICH_RA | dB |  |
| PHICH_RB | dB |  |
| PDCCH_RA | dB |  |
| PDCCH_RB | dB |  |
| PDSCH_RA | dB |  |
| PDSCH_RB | dB |  |
| OCNG_RANote3 | dB |  |
| OCNG_RBNote3 | dB |  |
| NocNote4 | dBm/15 kHz | -104 |
| Ês/Noc | dB | 17 |
| Ês/Iot | dB | 17 |
| RSRP Note5 | dBm/15 kHz | -87 |
| SCH_RP Note5 | dBm/15 kHz | -87 |
| Io Note5 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN |
| Antenna Configuration |  | 1x2 |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211. Note 2: DL RMCs and OCNG patterns are specified in sections A 3.1 and A 3.2 of TS 36.133 respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |

Table A.3.7.2.2-1: E-UTRAN cell specific test parameters for tests with one or more NR cells in FR2

| Parameter | Unit | E-UTRAN Cell1 |
| --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 |
| Duplex mode |  | FDD or TDD |
| TDD special subframe configurationNote1 |  | 6 |
| TDD uplink-downlink configurationNote1 |  | 1 |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |
| OCNG PatternsNote2 |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |
| PBCH_RA | dB | 0 |
| PBCH_RB | dB |  |
| PSS_RA | dB |  |
| SSS_RA | dB |  |
| PCFICH_RB | dB |  |
| PHICH_RA | dB |  |
| PHICH_RB | dB |  |
| PDCCH_RA | dB |  |
| PDCCH_RB | dB |  |
| PDSCH_RA | dB |  |
| PDSCH_RB | dB |  |
| OCNG_RANote3 | dB |  |
| OCNG_RBNote3 | dB |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211. Note 2: DL RMCs and OCNG patterns are specified in sections A 3.1 and A 3.2 of TS 36.133 respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: The E-UTRA signal is required only to ensure the E-UTRA link to the DUT in the EN-DC operation. The Test System shall provide a stable and noise-free E-UTRA signal without need of precise propagation modelling, path loss and polarization control. Further details of the E-UTRA signal configuration are not defined as part of the cell specific test parameters, since the E-UTRA link is not under performance verification and is not expected to influence the NR FR2 requirement. |  |  |

On the other hand, RAN4#88 agreed with RMC table used for NR RRM tests. In this table, the TDD configuration is set to ‘3D1S1U’ with S=9DL:3GP:2UL for SCS=15/30/120kHz. (See tables below)

Table A.3.1.1.2-1: PDSCH Reference Measurement Channels for SCS=15kHz

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | SR.1.1 TDD |  |  |  |  |  |  |
| Channel bandwidth | MHz | 10 |  |  |  |  |  |  |
| Number of transmitter antennas |  | 1 |  |  |  |  |  |  |
| Allocated resource blocks for PDSCH Note 1 |  | 24 |  |  |  |  |  |  |
| Uplink-Downlink Configuration |  | 3D1S1U |  |  |  |  |  |  |
| Flexible Slot ‘S’ Configuration |  | 9DL:3GP:2UL |  |  |  |  |  |  |
| Allocated slots per Radio Frame |  | TBD |  |  |  |  |  |  |
| MCS index |  | [4] |  |  |  |  |  |  |
| Modulation |  | QPSK |  |  |  |  |  |  |
| Target Coding Rate |  | 1/3 |  |  |  |  |  |  |
| Number of control symbols |  | 2 |  |  |  |  |  |  |
| Information Bit Payload |  |  |  |  |  |  |  |  |
| For slots with RMSI Note 2 | Bits | TBD |  |  |  |  |  |  |
| Number of Code Blocks per slot |  | 1 |  |  |  |  |  |  |
| Binary Channel Bits Per slot |  |  |  |  |  |  |  |  |
| For slots with RMSI Note 2 | Bits | TBD |  |  |  |  |  |  |
| Note 1: Allocation is located in the middle of active DL BWP. Note 2: PDSCH is scheduled on the slots with RMSI. Note 3: If necessary the information bit payload size can be adjusted to facilitate the test implementation. The payload sizes are defined in 3GPP TS 38.213. |  |  |  |  |  |  |  |  |

Table A.3.1.1.1-2: PDSCH Reference Measurement Channels for SCS=30kHz

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | SR.2.1 TDD |  |  |  |  |  |  |
| Channel bandwidth | MHz | 40 |  |  |  |  |  |  |
| Number of transmitter antennas |  | 1 |  |  |  |  |  |  |
| Allocated resource blocks for PDSCH Note 1 |  | 24 |  |  |  |  |  |  |
| Uplink-Downlink Configuration |  | 3D1S1U |  |  |  |  |  |  |
| Flexible Slot ‘S’ Configuration |  | 9DL:3GP:2UL |  |  |  |  |  |  |
| Allocated slots per Radio Frame |  | TBD |  |  |  |  |  |  |
| MCS index |  | [4] |  |  |  |  |  |  |
| Modulation |  | QPSK |  |  |  |  |  |  |
| Target Coding Rate |  | 1/3 |  |  |  |  |  |  |
| Number of control symbols |  | 2 |  |  |  |  |  |  |
| Information Bit Payload |  |  |  |  |  |  |  |  |
| For slots with RMSI Note 2 | Bits | TBD |  |  |  |  |  |  |
| Number of Code Blocks per slot |  | 1 |  |  |  |  |  |  |
| Binary Channel Bits Per slot |  |  |  |  |  |  |  |  |
| For slots with RMSI Note 2 | Bits | TBD |  |  |  |  |  |  |
| Note 1: Allocation is located in the middle of active DL BWP. Note 2: PDSCH is scheduled on the slots with RMSI. Note 3: If necessary the information bit payload size can be adjusted to facilitate the test implementation. The payload sizes are defined in 3GPP TS 38.213. |  |  |  |  |  |  |  |  |

Table A.3.1.1.1-3: PDSCH Reference Measurement Channels for SCS=120kHz

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | SR.3.1 TDD |  |  |  |  |  |  |
| Channel bandwidth | MHz | 100 |  |  |  |  |  |  |
| Number of transmitter antennas |  | 1 |  |  |  |  |  |  |
| Allocated resource blocks for PDSCH Note 1 |  | 24 |  |  |  |  |  |  |
| Uplink-Downlink Configuration |  | 3D1S1U |  |  |  |  |  |  |
| Flexible Slot ‘S’ Configuration |  | 9DL:3GP:2UL |  |  |  |  |  |  |
| Allocated slots per Radio Frame |  | TBD |  |  |  |  |  |  |
| MCS index |  | [4] |  |  |  |  |  |  |
| Modulation |  | QPSK |  |  |  |  |  |  |
| Target Coding Rate |  | 1/3 |  |  |  |  |  |  |
| Number of control symbols |  | 2 |  |  |  |  |  |  |
| Information Bit Payload |  |  |  |  |  |  |  |  |
| For slots with RMSI Note 2 | Bits | TBD |  |  |  |  |  |  |
| Number of Code Blocks per slot |  | 1 |  |  |  |  |  |  |
| Binary Channel Bits Per slot |  |  |  |  |  |  |  |  |
| For slots with RMSI Note 2 | Bits | TBD |  |  |  |  |  |  |
| Note 1: Allocation is located in the middle of active DL BWP. Note 2: PDSCH is scheduled on the slots with RMSI. Note 3: If necessary the information bit payload size can be adjusted to facilitate the test implementation. The payload sizes are defined in 3GPP TS 38.213. |  |  |  |  |  |  |  |  |

## 2.2	Issue with the EN-DC tests

The EN-DC RRM tests with FR1 is performed by connecting the signal cables from eNB/gNB simulators to the UE antenna ports. Depending on the UE implementation and LTE-NR band combination, both signal cables (one from LTE, another from NR) are connected to one Rx port. Under this condition, if TDD configuration is different between LTE and NR, it happens cross-link interference and if affects to RRM tests such as handover test or measurement test.

As we discussed so far, there is UL/DL configuration mismatch between the LTE for EN-DC test and RMC for NR. LTE part assumes ‘DSUUD DSUUD’ for SCS=15kHz, and NR part assumes ‘DDDSU DDDSU’ for SCS=15kHz. With this configuration, eNB assumes UL slot but gNB assumes DL slot, for example, in NR slot 2 or LTE subframe 2. This means UE measuring NR DL signal in slot 2 may be affected by LTE UL signal.

## 2.3	Proposal of TDD configuration

When RAN4 discussed the TDD configuration for NR RMC, one of the candidate configuration was the same as LTE configuration 1, i.e., ‘DSUUD DUSSD’. However as of RAN4#88, the possible UL-DL transmission periodicity is 0.5ms, 0.625ms, 1ms, 1.25ms, 2ms, 2.5ms, 5ms, or 10ms. This means it was not possible to configure the exactly same configuration as LTE UL/DL configuration 1 because NR configuration is ‘DDSUU DDSUU’; the TE should shift the frame boundary by 1ms.

However RAN1/RAN2 has discussed the additional periodicities to set the same TDD configuration as LTE, and RAN plenary #81 agreed with two additional periodicity: 3ms and 4ms [2].

TDD-UL-DL-Pattern ::= SEQUENCE { dl-UL-TransmissionPeriodicity ENUMERATED {ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms5, ms10}, nrofDownlinkSlots INTEGER (0..maxNrofSlots), nrofDownlinkSymbols INTEGER (0..maxNrofSymbols-1), nrofUplinkSlots INTEGER (0..maxNrofSlots), nrofUplinkSymbols INTEGER (0..maxNrofSymbols-1), ..., [[ dl-UL-TransmissionPeriodicity-v15xy ENUMERATED {ms3, ms4} OPTIONAL -- Need R ]] }

Since LTE has not many possible TDD UL/DL configurations, LTE tests uses UL/DL configuration 1 in many cases. We propose to keep the TDD configuration for LTE setup, and adapt the NR TDD configuration to LTE RMC. According to our analysis this new parameter makes it possible to configure the LTE TDD configuration 1 with the same frame timing.

For FR1 SCS=15kHz, we propose to configure two TDD UL-DL patterns, and set the pattern 1 to ‘DSUU’ with periodicity of 4ms and set the pattern 2 to ‘D’ with periodicity of 1ms. For special slot, we can set S=9DL:3GP:2UL.

For FR1 SCS=30kHz, we propose to configure two TDD UL-DL patterns, and set the pattern 1 to ‘DDDSUUUU’ with periodicity of 4ms and set the pattern 2 to ‘DD’ with periodicity of 1ms. For special slot, we can set S=4DL:6GP:4UL.

Note for EN-DC tests in FR2, LTE signal comes from the conduced cable and NR signal comes from OTA. This means LTE and NR can configure different TDD configuration and therefore we propose to keep TDD configuration for NR RMC in FR2 is ‘DDDSU’ with periodicity of 0.625ms.

# 3	Conclusion

Proposal:

For SCS=15kHz, set the TDD pattern 1 to ‘DSUU’ with 4ms periodicity and TDD pattern 2 to ‘D’ with 1ms periodicity. Special slot sets to: S=9DL:3GP:2UL

For SCS=30kHz, set the TDD pattern 1 to ‘DDDSUUUU’ with 4ms periodicity and TDD pattern 2 to ‘DD’ with 1ms periodicity. Special slot sets to: S=4DL:6GP:4UL

| Reference channel |  | SR.1.1 TDD | SR.2.1 TDD | SR.3.1 TDD |
| --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 40 | 100 |
| Number of transmitter antennas |  | 1 | 1 | 1 |
| Allocated resource blocks for PDSCH Note 1 |  | 24 | 24 | 24 |
| TDD UL/DL pattern 1 |  |  |  |  |
| dl-UL-TransmissionPeriodicity | ms | 4 | 4 | 0.625 |
| nrofDownlinkSlots |  | 1 | 3 | 3 |
| nrofDownlinkSymbols |  | 9 | 4 | 9 |
| nrofUplinkSlots |  | 2 | 4 | 1 |
| nrofUplinkSymbols |  | 2 | 4 | 2 |
| TDD UL/DL pattern 2 |  |  |  |  |
| dl-UL-TransmissionPeriodicity | ms | 1 | 1 | N/A |
| nrofDownlinkSlots |  | 1 | 2 | N/A |
| nrofDownlinkSymbols |  | 0 | 0 | N/A |
| nrofUplinkSlots |  | 0 | 0 | N/A |
| nrofUplinkSymbols |  | 0 | 0 | N/A |
| Allocated slots per Radio Frame |  | TBD | TBD | TBD |
| MCS index |  | [4] | [4] | [4] |
| Modulation |  | QPSK | QPSK | QPSK |
| Target Coding Rate |  | 1/3 | 1/3 | 1/3 |
| Number of control symbols |  | 2 | 2 | 2 |
| Information Bit Payload |  |  |  |  |
| For slots with RMSI Note 2 | Bits | TBD | TBD | TBD |
| Number of Code Blocks per slot |  | 1 | 1 | 1 |
| Binary Channel Bits Per slot |  |  |  |  |
| For slots with RMSI Note 2 | Bits | TBD | TBD | TBD |

# References

- R4-1811923, “CR to TS 38.133: Implementation of endorsed draft CRs from RAN4-AH-1807 and RAN4 #88”, Intel.

- RP-181942, “NR related CRs, part 5”, RAN2.