3GPP TSG-RAN WG4 Meeting #88bis	R4-1812913

## Chengdu, China, 8 - 12 October, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.817-02 | CR | 0015 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TR 38.817-02: EIRP in extreme conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AASenh_BS_LTE_UTRA-Perf |  |  |  |  |  |  | Date: |  | 2018-09-12 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Conformance testing aspects for EIRP in extreme conditions are missing from the TR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Measurement uncertainty background for EIRP in extreme conditions is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformance testing aspects for EIRP in extreme conditions remain missing from the TR. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

------------------------------ Modified section ------------------------------

### 12.2.X	EIRP accuracy – extreme conditions

#### 12.2.X.1 BS type 1-O

For BS type 1-O is MU analysis is documented 3GPP TR 37.843 [X] subclause 10.2.9.

#### 12.2.X.2 BS type 2-O

#### 12.2.X.2.1	General

#### 12.2.X.2.2	Compact antenna test range (CATR)

##### 12.2.X.2.2.1	General

##### 12.2.X.2.2.2	Calibration

##### 12.2.X.2.2.3	 Procedure

##### 12.2.X.2.2.4	  MU assessment

###### 12.2.X.2.2.5	MU Budget

###### 12.2.X.2.2.6	MU Value

Table 12.2.X.2.2.6-1: Compact antenna test range uncertainty assessment for extreme EIRP accuracy measurement

| UID | Uncertainty Source | Uncertainty value | Uncertainty value | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] | Standard uncertainty ui [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 24.25<f | 37<f |  |  |  | 24.25<f | 37<f |
|  |  | <29.5GHz | <40GHz |  |  |  | <29.5GHz | <40GHz |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Misalignment DUT & pointing error | 0.2 | 0.2 | Exp. normal | 2 | 1 | 0.1 | 0.1 |
| 2 | RF power measurement equipment (e.g. spectrum analyzer, power meter) | 0.5 | 0.7 | Gaussian | 1 | 1 | 0.5 | 0.7 |
| 3 | Standing wave between DUT and test range antenna | 0.03 | 0.03 | U-shaped | 1.41 | 1 | 0.02 | 0.02 |
| 4 | RF leakage, test range antenna cable connector terminated. | 0.03 | 0.03 | Normal | 1 | 1 | 0.03 | 0.03 |
| 5 | QZ ripple with DUT (extreme) | 0.6 | 0.6 | Normal | 1 | 1 | 0.6 | 0.6 |
| X4 | Frequency flatness | 0.25 | 0.25 | Normal | 1 | 1 | 0.25 | 0.25 |
| X1 | Radome loss variation | 0.15 | 0.15 | Rectangular | 1.73 | 1 | 0.09 | 0.09 |
| X2 | Wet radome loss variation | 1 | 1 | U-shaped | 1.41 | 1 | 0.71 | 0.71 |
| X3 | Change in absorber behavior | 2.25 | 2.25 | U-shaped | 1.41 | 1 | 1.60 | 1.60 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 6 | Network Analyzer | 0.3 | 0.3 | Normal | 1 | 1 | 0.3 | 0.3 |
| 7 | Uncertainty of return loss (S11) measurement of SGH and test receiver (VNA) ports | 0.43 | 0.57 | U-shaped | √2 | 1 | 0.3 | 0.4 |
| 8 | Insertion loss variation in receiver chain | 0 | 0 | Rectangular | √3 | 1 | 0 | 0 |
| 9 | RF leakage, test range antenna cable connector terminated. | 0.0012 | 0.0012 | Normal | 1 | 1 | 0.0012 | 0.0012 |
| 10 | Influence of the calibration antenna feed cable | 0.21 | 0.29 | U-shaped | √2 | 1 | 0.15 | 0.2 |
| 11 | SGH Calibration uncertainty | 0.52 | 0.52 | Rectangular | √3 | 1 | 0.3 | 0.3 |
| 12 | Misalignment positioning system | 0 | 0 | Exp. normal | 2 | 1 | 0 | 0 |
| 13 | Misalignment SGH and pointing error | 0 | 0 | Exp. normal | 2 | 1 | 0 | 0 |
| 14 | Rotary joints | 0 | 0 | U-shaped | √2 | 1 | 0 | 0 |
| 15 | Standing wave between SGH and test range antenna | 0.09 | 0.09 | U-shaped | √2 | 1 | 0.06 | 0.06 |
| 16 | QZ ripple with SGH | 0.009 | 0.009 | Normal | 1 | 1 | 0.009 | 0.009 |
| 17 | Switching uncertainty | 0.1 | 0.1 | Rectangular | √3 | 1 | 0.25 | 0.25 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 2.01 | 2.09 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 3.93 | 4.09 |

#### 12.2.X.X	Summary

------------------------------ End of modified sections ------------------------------