3GPP TSG-RAN WG4 Meeting #88bis                     	R4-1813084

Chengdu, China, Oct 8th –12th, 2018

Agenda item:            7.13.1.2

Source:	MediaTek Inc.

Title:	NR PDSCH simulation result

Document for:          Discussion

# 1	Introduction

In this paper, we present the simulation result of PDSCH.

# 2	Discussion

Based on the simulation assumption in [1], we provide the simulation result of PDSCH performance. Table A.1, Table A.2 and Table A.3 (in Appendix) are the test case list for FR1 FDD, FR1 TDD and FR2 TDD respectively. Table A.4 contains other detailed parameter setting in the simulation.

Table 2.1 is the 70% Tput SNR values of FR1 FDD tests and Table 2.2 is the 70% Tput SNR values of FR1 TDD tests. Figure A.2.1~Figure A.2.28 (in Appendix) are the corresponding performance curves.

Table 2.1 70% Tput SNR of FR1 FDD tests

| Case Number | 70% Tput SNR – 2RX | 70% Tput SNR – 4RX |
| --- | --- | --- |
| FR1-FDD1 | -1.57 | -4.58 |
| FR1-FDD2 |  |  |
| FR1-FDD3 | 22.2 | 18.42 |
| FR1-FDD4 | 13.13 | 7.37 |
| FR1 FDD5 | 17.13 | 11.05 |
| FR1-FDD6 | N/A | 9.39 |
| FR1-FDD7 | N/A | 13.5 |
| FR1-FDD8 | 17.9/ 15.58(R-ML) | N/A |
| FR1-FDD9 | N/A | 26.5/ 19.65(R-ML) |
| FR1-FDD10 | 6.18 | 2.68 |

Table 2.2 70% Tput SNR of FR1 TDD tests

| Case Number | 70% Tput SNR – 2RX | 70% Tput SNR – 4RX |
| --- | --- | --- |
| FR1-TDD1 | -1.71 | -4.66 |
| FR1-TDD2 |  |  |
| FR1-TDD3 | 22.82 | 18.67 |
| FR1-TDD4 | 13.1 | 7.48 |
| FR1-TDD5 | 18.13 | 11.4 |
| FR1-TDD6 | N/A | 9.58 |
| FR1-TDD7 | N/A | 13.77 |
| FR1-TDD8 | 18.45/ 16.2(R-ML) | N/A |
| FR1-TDD9 | N/A | 26.67/ 20.39(R-ML) |
| FR1-TDD10 | 6.31 | 2.81 |
| FR1-TDD11 | -1.92 | -4.88 |
| FR1-TDD12 | -1.89 | -4.86 |

Table 2.3 70% Tput SNR of FR2 TDD tests

| Case Number | 70% Tput SNR |
| --- | --- |
| FR2-1 | -0.21 |
| FR2-2 |  |
| FR2-3 | 13.68 |
| FR2-4 | 19.75/18.11(R-ML) |
| FR2-5 | 6.72 |
| FR2-6 | 11.87 |

Observation 1: The performance difference between MMSE-IRC and R-ML is quite large in 4RX with media correlation channel. To guarantee the system performance, the performance requirement of R-ML receiver should be included in the test case.

# 3	Conclusions

In the contribution, we provide our simulation on PDSCH. We have the following observations:

Observation 1: The performance difference between MMSE-IRC and R-ML is quite large in 4RX with media correlation channel. To guarantee the system performance, the performance requirement of R-ML receiver should be included in the test case.

# 4	Reference

# [1] R4-1811723, “Simulation assumptions for NR PDSCH demodulation performance requirements”, Intel

Appendix

Table A.1 PDSCH FR1 FDD test cases

| Case Number | CHBW/ SCS | MIMO | MCS | Number of layer | Number of PDSCH PRBs | Channel Model | Receiver | Max number of HARQ process |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FR1-FDD1 | 10MHz/15kHz | 2Tx 2Rx ULA Low 2Tx 4Rx ULA Low | QPSK, MCS 4 | 1 Layer | 52 | TDL-B 100ns, 400 Hz | MMSE-IRC | [4] |
| FR1- FDD 2 | 10MHz/15kHz | 2Tx 2Rx ULA Low 2Tx 4Rx ULA Low | QPSK, MCS 4 | 1 Layer | 6 (in the middle of CBW) | TDL-C 300ns, 100 Hz | MMSE-IRC | [4] |
| FR1- FDD 3 | 10MHz/15kHz | 2Tx 2Rx ULA Low 2Tx 4Rx ULA Low | 256QAM, MCS 24 | 1 Layer | 52 | TDL-A 30ns, 10Hz | MMSE-IRC | [4] |
| FR1- FDD 4 | 10MHz/15kHz | 2Tx 2Rx ULA Low 2Tx 4Rx ULA Low | 16QAM, MCS 13 | 2 Layers | 52 | TDL-C 300ns, 100 Hz | MMSE-IRC | [4] |
| FR1- FDD 5 | 10MHz/15kHz | 2Tx 2Rx ULA Low 2Tx 4Rx ULA Low | 64QAM, MCS 19 | 2 Layers | 52 | TDL-A 30ns, 10Hz | MMSE-IRC | [4] |
| FR1-FDD6 | 10MHz/15kHz | 4Tx 4Rx ULA Low | 16QAM, MCS 13 | 3 Layers | 52 | TDL-A 30ns, 10Hz | MMSE-IRC | [4] |
| FR1-FDD7 | 10MHz/15kHz | 4Tx 4Rx ULA Low | 16QAM, MCS 13 | 4 Layers | 52 | TDL-A 30ns, 10Hz | MMSE-IRC | [4] |
| FR1-FDD8 | 10MHz/15kHz | 2Tx 2Rx ULA Med | 16QAM, MCS 13 | 2 Layers | 52 | TDL-A 30ns, 10Hz | MMSE-IRC R-ML | [4] |
| FR1-FDD9 | 10MHz/15kHz | 4Tx 4RX ULA Med A | 16QAM, MCS 13 | 3 Layers | 52 | TDL-A 30ns, 10Hz | MMSE-IRC R-ML | [4] |
| FR1- FDD10 | 10MHz/15kHz | 2Tx 2Rx ULA Low 2Tx 4Rx ULA Low | 16QAM, MCS 13 | 1 Layer | 52 | TDL-C 300ns, 100 Hz | MMSE-IRC | 8 |

Table A.2 PDSCH FR1 TDD test cases

| Case Number | CHBW/ SCS | MIMO | MCS | Number of layer | Number of PDSCH PRBs | Channel Model | Receiver | TDD Configuration | Max number of HARQ process |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FR1- TDD1 | 40MHz/30kHz | 2Tx 2Rx ULA Low 2Tx 4Rx ULA Low | QPSK MCS 4 | 1 Layer | 106 | TDL-B 100ns, 400 Hz | MMSE-IRC | 1 | 8 |
| FR1- TDD2 | 40MHz/30kHz | 2Tx 2Rx ULA Low 2Tx 4Rx ULA Low | QPSK MCS 4 | 1 Layer | 6 (in the middle of CBW) | TDL-C 300ns, 100 Hz | MMSE-IRC | 1 | 8 |
| FR1- TDD3 | 40MHz/30kHz | 2Tx 2Rx ULA Low 2Tx 4Rx ULA Low | 256QAM MCS 24 | 1 Layer | 106 | TDL-A 30ns, 10Hz | MMSE-IRC | 1 | 8 |
| FR1- TDD4 | 40MHz/30kHz | 2Tx 2Rx ULA Low 2Tx 4Rx ULA Low | 16QAM MCS 13 | 2 Layers | 106 | TDL-C 300ns, 100 Hz | MMSE-IRC | 1 | 8 |
| FR1- TDD5 | 40MHz/30kHz | 2Tx 2Rx ULA Low 2Tx 4Rx ULA Low | 64QAM MCS 19 | 2 Layers | 106 | TDL-A 30ns, 10Hz | MMSE-IRC | 1 | 8 |
| FR1- TDD6 | 40MHz/30kHz | 4Tx 4Rx ULA Low | 16QAM MCS 13 | 3 Layers | 106 | TDL-A 30ns, 10Hz | MMSE-IRC | 1 | 8 |
| FR1- TDD7 | 40MHz/30kHz | 4Tx 4Rx ULA Low | 16QAM MCS 13 | 4 Layers | 106 | TDL-A 30ns, 10Hz | MMSE-IRC | 1 | 8 |
| FR1- TDD8 | 40MHz/30kHz | 2Tx 2Rx ULA Med | 16QAM MCS 13 | 2 Layers | 106 | TDL-A 30ns, 10Hz | MMSE-IRC R-ML | 1 | 8 |
| FR1- TDD9 | 40MHz/30kHz | 4Tx 4RX ULA Med A | 16QAM MCS 13 | 3 Layers | 106 | TDL-A 30ns, 10Hz | MMSE-IRC R-ML | 1 | 8 |
| FR1- TDD10 | 40MHz/30kHz | 2Tx 2Rx ULA Low 2Tx 4Rx ULA Low | 16QAM MCS 13 | 1 Layer | 106 | TDL-C 300ns, 100 Hz | MMSE-IRC | 1 | 16 |
| FR1- TDD11 | 40MHz/30kHz | 2Tx 2Rx ULA Low 2Tx 4Rx ULA Low | [QPSK MCS 4] | [1 Layer] | 106 | TDL-A 30ns, 10Hz | MMSE-IRC | 2 | 8 |
| FR1- TDD12 | 40MHz/30kHz | 2Tx 2Rx ULA Low 2Tx 4Rx ULA Low | [QPSK MCS 4] | [1 Layer] | 106 | TDL-A 30ns, 10Hz | MMSE-IRC | 3 | 8 |

Table A.3 PDSCH FR2 TDD test cases

| Case Number | CHBW/ SCS | MIMO | MCS | Number of layer | Number of PDSCH PRBs | Channel Model | Receiver | TDD Configuration | Max number of HARQ process |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FR2-1 | 100MHz/120kHz | 2Tx 2Rx Low | QPSK MCS 4 | 1 Layer | 66 | TDL-A 30ns, 300Hz | MMSE-IRC | 1 | 8 |
| FR2-2 | 100MHz/120kHz | 2Tx 2Rx Low | QPSK MCS 4 | 2 Layers | 6 (in the middle of CBW) | TDL-A 30ns, 75Hz | MMSE-IRC | 2 | 10 |
| FR2-3 | 100MHz/120kHz | 2Tx 2Rx Low | 16QAM MCS 13 | 2 Layers | 66 | TDL-A 30ns, 300Hz | MMSE-IRC | 1 | 8 |
| FR2-4 | 100MHz/120kHz | 2Tx 2Rx ULA Med | 16QAM MCS 13 | 2 Layers | 66 | TDL-A 30ns, 75Hz | MMSE-IRC R-ML | 2 | 10 |
| FR2-5 | 100MHz/120kHz | 2Tx 2Rx Low | 16QAM MCS 13 | 1 Layer | 66 | TDL-A 30ns, 300Hz | MMSE-IRC | 1 | 16 |
| FR2-6 | 100MHz/120kHz | 2Tx 2Rx Low | 64QAM MCS 19 | 1 Layer | 66 | TDL-A 30ns, 75Hz | MMSE-IRC | 1 | 8 |

Table A.4 PDSCH simulation parameter setting

| Items | Setting | Notes |
| --- | --- | --- |
| TDD UL/DL configurations | - FR1 TDD - Configuration 1: 7D1S2U, S = 6D:4G:4U - Configuration 2: DDDSUDDSUU, S1(D,GP,U)=[10, 4, 0], S2(D, GP, U) = [10,4,0] - Configuration 3: DDDSU, S=10D:2G:2U - FR2 TDD - Configuration 1: DDDSU, S=10D:2G:2U - Configuration 2: DDSU, S=11D+3G |  |
| DMRS | - Type 1 single symbol front loaded - 1 additional DMRS for Low (5/10Hz) and Medium (100 Hz) Speed scenarios - OFDM symbols with DMRS: #2 and #9 - 2 additional DMRSs for High (400 Hz) Speed scenarios - OFDM symbols with DMRS: #2, #6 and #9 - FDM is applied between DMRS and data for test cases with 1 Layer and 2 Layers |  |
| Impairments model | - FR1 - TX EVM = 6% for QPSK/16QAM/64QAM - TX EVM = 3% for 256QAM - FR2 - TX EVM = 6% for QPSK/16QAM/64QAM - Phase noise is explicitly modelled for Rx. Use phase noise model #2 (UE side) as defined in TR 38.803 for FR2 demodulation performance tests. - CF = 29 GHz |  |

PDSCH simulation performance curve: X-label (SNR) / Y-label (Tput ratio)

Figure A.2.1 PDSCH Tput – Test case FR1-FDD1

Figure A.2.2 PDSCH Tput – Test case FR1-FDD2

Figure A.2.3 PDSCH Tput – Test case FR1-FDD3

Figure A.2.4 PDSCH Tput – Test case FR1-FDD4

Figure A.2.5 PDSCH Tput – Test case FR1-FDD5

Figure A.2.6 PDSCH Tput – Test case FR1-FDD6

Figure A.2.7 PDSCH Tput – Test case FR1-FDD7

Figure A.2.8 PDSCH Tput – Test case FR1-FDD8

Figure A.2.9 PDSCH Tput – Test case FR1-FDD9

Figure A.2.10 PDSCH Tput – Test case FR1-FDD10

Figure A.2.11 PDSCH Tput – Test case FR1-TDD1

Figure A.2.12 PDSCH Tput – Test case FR1-TDD2

Figure A.2.13 PDSCH Tput – Test case FR1-TDD3

Figure A.2.14 PDSCH Tput – Test case FR1-TDD4

Figure A.2.15 PDSCH Tput – Test case FR1-TDD5

Figure A.2.16 PDSCH Tput – Test case FR1-TDD6

Figure A.2.17 PDSCH Tput – Test case FR1-TDD7

Figure A.2.18 PDSCH Tput – Test case FR1-TDD8

Figure A.2.19 PDSCH Tput – Test case FR1-TDD9

Figure A.2.20 PDSCH Tput – Test case FR1-TDD10

Figure A.2.21 PDSCH Tput – Test case FR1-TDD11

Figure A.2.22 PDSCH Tput – Test case FR1-TDD12

Figure A.2.23 PDSCH Tput – Test case FR2-1

Figure A.2.24 PDSCH Tput – Test case FR2-2

Figure A.2.25 PDSCH Tput – Test case FR2-3

Figure A.2.26 PDSCH Tput – Test case FR2-4

Figure A.2.27 PDSCH Tput – Test case FR2-5

Figure A.2.28 PDSCH Tput – Test case FR2-6