3GPP TSG-RAN WG4 Meeting #88bis                 	R4-1813220

Chengdu, China, 8 – 12 October 2018

Source:	Nokia, Nokia Shanghai Bell

Title:	NR PUCCH simulation results

Agenda item:	7.13.2.3

Document for:	Discussion

Introduction

In the last RAN4 meeting, a WF was agreed for NR PUCCH demodulation in [1]. In this contribution, we provide simulation results for the formats and test cases listed in the WF.

Initial simulation assumptions

The simulation results in the following section are based on the simulation assumptions that were agreed in the WF:

Simulation results

In Table 1 we provide simulation results for all the discussed formats and parameter combinations.

Table 1: Simulation results for NR PUCCH.

| PUCCH format | SCS | Number of bits | Number of OFDM symbols | Number of PRBs | Note | SINR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| Format 0 | 15KHz, 10MHz | 1 | 1 | 1 | ACK miss | 6.8 |
|  |  | 1 | 2 | 1 | ACK miss | 3.3 |
|  | 30KHz, 40MHz | 1 | 1 | 1 | ACK miss | 7.9 |
|  |  | 1 | 2 | 1 | ACK miss | 2.5 |
| Format 1 | 15KHz, 10MHz | 2 | 14 | 1 | ACK miss | -6.2 |
|  |  |  |  |  | NACK to ACK | -5.1 |
|  | 30KHz, 40MHz |  |  |  | ACK miss | -6.5 |
|  |  |  |  |  | NACK to ACK | -5.5 |
| Format 2 | 15KHz, 10MHz | 4 | 1 | 4 | ACK miss | 1.3 |
|  |  |  |  |  | NACK to ACK |  |
|  |  | 22 | 2 | 9 | ACK miss | -4 |
|  |  |  |  |  | NACK to ACK |  |
|  | 30KHz, 40MHz | 4 | 1 | 4 | ACK miss | 2.8 |
|  |  |  |  |  | NACK to ACK |  |
|  |  | 22 | 2 | 9 | ACK miss | -4.5 |
|  |  |  |  |  | NACK to ACK |  |
| Format 3 | 15KHz, 10MHz | 16 | 14 | 1 | Without additional DMRS | -1.5 |
|  |  |  |  |  | With additional DMRS | -2 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -0.8 |
|  | 30KHz, 40MHz | 16 | 14 | 1 | Without additional DMRS | -2 |
|  |  |  |  |  | With additional DMRS | -2.4 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -5.7 |
| Format 4 | 15KHz, 10MHz | 22 | 14 | 1 | Without additional DMRS | -0.7 |
|  |  |  |  |  | With additional DMRS | -0.8 |
|  | 30KHz, 40MHz |  |  |  | Without additional DMRS | -5.6 |
|  |  |  |  |  | With additional DMRS | -6.2 |

Conclusion

In this contribution we have provided simulation results for NR PUCCH.

- References

[1] R4-1811727, Way forward on NR PUCCH demodulation performance in Rel-15, Huawei, 3GPP TSG-RAN WG4 Meeting #88 Gothenburg, Sweden, 20 - 24 Aug 2018.