3GPP TSG RAN WG4 Meeting #88Bis	R4-1813283

Chengdu, China, October 8 – 12, 2018

Agenda Item:	7.9.2.3

Source:	Huawei, HiSilicon

Title:	Reference Symbol for Test Model

Document for:	Approval

# Introduction

In RAN#88, there were agreements to define the DMRS in the test models [1][2]. However, the details of the DMRS need further discussion. This contribution proposes DMRS locations for the downlink and the parameters needed for the DMRS sequence generation.

# Discussion

## Common factors

Among the design parameters, the cell id NIDcell is a common factor.

Proposal 1: The cell id shall be [1].

## PBCH

The initialization for the scrambling sequence generator is based on the cell id NIDcell and the SS block index (iSSB=iSSBmod4+4nHF) for L=4 and nHF=0 for first half frame and nHF=1 for the second half frame and for iSSB=iSSBmod8 for L=8 and 64. The values based on the proposed SS block design are listed in Table 1.

Table 1. Parameter values for the PBCH DMRS

| SS block SCS | L | iSSB | nHF |
| --- | --- | --- | --- |
| Case 1, 15 kHz | 4 | 0, 1 | 0 |
| Case 2, 30 kHz | 4 | 0, 1, 2, 3 | 0 |
| Case 3, 30 kHz | 4 | 0, 1, 2, 3 | 0 |
| Case 4, 120 kHz | 4 | 0, 1, 2, 3 | 0 |

For the PBCH, the DMRS locations are specified in Table 7.4.3.1-1 of [3]. Specifically, every fourth RE of the second, third, and fourth symbol of the SS block contains a DMRS, as indicated in Table 2

Table 2. RE locations of DMRS for PBCH, v = NIDcell mod 4

| Symbol |  |
| --- | --- |
| 1 | 4w+v, w=0, …, 59 |
| 2 | 4w+v, w=0, …, 11, 48, …, 59 |
| 3 | 4w+v, w=0, …, 59 |

## PDCCH

The initialization for the scrambling sequence generator is based on the slot number and either the cell id or UE configuration.

For testing purposes, the cell id should be used for NID.

The PDCCH DMRS are located in REs 1, 5, and 9 of each RB carrying the PDCCH.

## PDSCH

The initialization for the scrambling sequence generator is based on the slot number, symbol number within the slot, NIDnSCID, and the parameter nSCID.

For testing purposes, the cell id should be used, i.e,  and that nSCID=0.

The position of the DMRS l is based on several factors: l=l+l'.

Proposal 2: One antenna port shall be used (Antenna port 1000).

It was agreed to use configuration type 1 for DMRS, with every other RE containing DMRS. To determine the other parameters for the DMRS, it is assumed that the PDSCH is configured using dedicated signaling.

For the IE PDSCH-TimeDomainResourceAllocationList, the field mappingType is set to typeA.

For the IE DMRS-DownlinkConfig, the fields are

dmrs-Type: field absent so that type 1 is used.

dmrs-AdditionalPosition: pos3

maxLength: field absent so maxLength=1 is used

scramblingID0: field absent

scramblingID1: field absent

phaseTrackingRS: field absent for FR1

In addition, dmrs-TypeA-Position = 2 which implies l0=2.

With this configuration, the locations of the DMRS for FDD and TDD are given in Table 3

Table 3. DMRS

|  | S | L | l |
| --- | --- | --- | --- |
| FDD | 2 | 12 | 2, 5, 8, 11 |
| Full slots in TDD | 2 | 12 | 2, 5, 8, 11 |
| 10 DL symbol TDD slot | 2 | 8 | 2, 6, 9 |
| 6 DL symbol TDD slot | 2 | 4 | 2 |
| 12 DL symbol TDD slot | 2 | 10 | 2, 5, 8, 11 |

Since the DMRS density for a 6 DL symbol TDD slot is 1, no measurements should be taken.

Proposal 3: For a TDD slot with only 6 DL symbols, no measurements should be taken.

# Conclusion

Proposal 1: The cell id shall be [1].

Proposal 2: One antenna port shall be used (Antenna port 1000).

Proposal 3: For a TDD slot with only 6 DL symbols, no measurements should be taken.

The TP shall capture the values.

# References

R4-1811625, “TP to TS 38.141-1: NR Test Models”, Nokia, Nokia Shanghai Bell, RAN4#88, Aug. 20-24, 2018.

R4-1811626, “TP to TS 38.141-2: Section 6.1 NR Test Models”, Ericsson, RAN4#88, Aug. 20-24, 2018.

38.211

38.214