3GPP TSG-RAN WG4 Meeting #88-Bis		  												R4-1813539

Chengdu, China, 08 - 12 October 2018

Source:	Nokia, Nokia Shanghai Bell

Title:	TP to TS 38.141-1: Clause 4.9.2 Test models

Agenda item:	7.9.2.3

Document for:	Approval

# Introduction

This contribution provides text proposal to TS 38.141-1 clause 4.9.3 Test models. Last RAN4 #88 meeting TP to clause with Test models were agreed in [1].

However agreed TP [1] does not include all details that are needed to finalize Test model section in specification.

This TP provides following details:

- Addition of “FR1” to NR test models to distinguish NR test models for FR1 and FR2,

- Removal of brackets for first bullet in clause 4.9.2.2,

- Correction (shifting) of TDD pattern for test model to align in practice agreed NR TDD configuration with LTE configuration 2 and special sub-frame configuration 7,

- Extraction of common for all NR FR1 test models physicial channel parameters to main section, to avoid repetition of the same details for SSB, PDCCH, PDSCH channels.

- Boosted and measured RBG (resource block group)

# References

- R4-1811625	TP to TS 38.141-1: NR Test Models, Nokia, Nokia Shanghai Bell

<Start of TP>

### 4.9.2	Test models

#### 4.9.2.1	General

The following sections describe the NR FR1 test models needed for BS type 1-C and BS type 1-H.

#### 4.9.2.2 	NR FR1 test models

The set-up of physical channels for transmitter tests shall be according to one of the NR FR1 test models (NR-FR1-TM) below. A reference to the applicable test model is made within each test.

The following general parameters are used by all NR FR1 test models:

-	The test models are defined for a single antenna port (using p = 1000); 1 code word (q = 0), 1 layer, precoding is not used; unless specified otherwise

-	Duration is 1 radio frame (10 ms) for FDD and 2 radio frames for TDD (20 ms)

-	Normal CP

-	Virtual resource blocks of localized type

For NR TDD, test models are derived based on the uplink/downlink configuration as showing in the table 4.9.2.2-1.

Table 4.9.2.2-1: Configurations of NR TDD for BS type 1-C and BS type 1-H for test models

Table 4.9.2.2-1: Configurations of TDD gNB test models for NR FR1

| SCS [kHz] |  | Pattern 1 (periodicity 3 ms) |  |  |  |  | Pattern 2 (periodicity 2 ms) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Number of DL slots |  | Number of DL symbols in S slot | Number of UL symbols in S slot | Number of UL slots | Number of DL slots |  | Number of DL symbols in S slot | Number of UL symbols in S slot |  | Number of UL slots |
| 15 | 1 |  | 10 | 2 | 1 | 2 |  | 0 |  | 0 | 0 |
| 30 | 3 |  | 6 | 4 | 2 | 4 |  | 0 |  | 0 | 0 |
| 60 (Note) | 6 |  | 12 | 8 | 4 | 8 |  | 0 |  | 0 | 0 |
| Note: There are two S slots in Pattern 1. First S slot has 12 DL symbols followed by 2 flexible symbols; second S slot has 6 flexible symbols followed by 8 UL symbols. |  |  |  |  |  |  |  |  |

Common physical channel parameters for all NR FR1 test models are specify in the table 4.9.2.2-2. Specific physical channel parameters for NR FR1 test models are describe in sections 4.9.2.2.1 to 4.9.2.2.8.

Table 4.9.2.2-2: Physical channel parameters common for all NR FR1 test models

| SSB parameters |  |  |  |
| --- | --- | --- | --- |
| SSB periodicity | 10 ms (FDD) / 20 ms (TDD) |  |  |
| SSB mapping type | Case A/B/C |  |  |
| SSB bursts per radio frame | 1 |  |  |
| SS Blocks in SS burst | 1 |  |  |
| SS Block slot position in radio frame | 15 kHz | 30 kHz | 60 kHz (NOTE) |
|  | 1 | 0 | 0 |
| SS Block start symbol | 15 kHz | 30 kHz | 60 kHz |
|  | 2 | 8 | 4 |
| SS Block PRB offset | NRB2- 10 |  |  |
| SS Block subcarrier offset | 0 |  |  |
| Primary Synchronization Signal EPRE / Secondary Synchronization Signal EPRE [dB] | 0.000 |  |  |
| Secondary Synchronisation Signal EPRE / PBCH DM-RS [dB] | 0.000 |  |  |
| PDCCH |  |  |  |
| # of symbols used for control channel | 2 |  |  |
| Aggregation level (AL) | arg maxAL ϵ {1, 2,4,8,16}AL * 6≤ NRB |  |  |
| # of CCEs allocated to PDCCH | AL * 6 |  |  |
| # of RBs not allocated by PDCCH in the first symbol | NRB – AL * 6 |  |  |
| DM-RS configuration and density | Comb structure with same frequency density of ¼ (i.e., every 4th subcarrier) on all REGs |  |  |
| PDSCH |  |  |  |
| Nominal RBG size P configuration 1 as in table 5.1.2.2.1-1 in TS 38.214 | 2 when NRB ϵ <1; 36> 4 when NRB ϵ <37; 72> 8 when NRB <73; 144> 16 when NRB >= 145 |  |  |
| DM-RS configuration and density | Type 1, Comb2 (every other subcarrier) in symbols 3 and 11 |  |  |
| PDSCH EPRE to DM-RS EPRE [dB] | -3 |  |  |
| NOTE: For 60 kHz SCS data, SSB has SCS of 30 kHz. |  |  |  |

Where NRB is definied as a number of RB for given bandwidth and subcarrier spacing as in TS38.104, table 5.3.2-1.

Table 4.9.2.2-3: Number of RBG groups

| SCS [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 13 | 13 | 10 | 14 | 17 | 10 | 14 | 17 | N/A | N/A | N/A | N/A | N/A |
| 30 | 6 | 12 | 10 | 13 | 17 | 10 | 14 | 17 | 11 | 12 | 14 | 16 | 18 |
| 60 | N/A | 6 | 9 | 12 | 16 | 10 | 13 | 17 | 10 | 12 | 14 | 16 | 17 |

Table 4.9.2.2-4: Boosted (NR-FR1-TM1.2) or Measured (NR-FR1-TM3.2, NR-FR1-TM3.3) RBG numbers

DL Test Slots 0-9 (SCS 15 kHz, 30 kHz, 60 kHz)

for slot i = 0 to 9 RbgNumber(0, i)=0; for element n = 1 to 5 RbgNumber(n, i) = (RbgNumber(n-1, i) + i +1) mod (NumOfRbg - 2); end for end for

DL Test Slots 10-19 (SCS 30 kHz, 60 kHz)

for slot i = 10 to 19 for element n = 0 to 5 RbgNumber(n, i) = (RbgNumber(n, i-10) + 1) mod (numOfRbg - 2); end for end for

DL Test Slots 20-29 (SCS 60 kHz)

for slot i = 20 to 29 for element n = 0 to 5 RbgNumber(n, i) = (RbgNumber(n, i-20) + 2) mod (NumOfRbg - 2); end for end for

DL Test Slots 30-39 (SCS 60 kHz)

for slot i = 30 to 39 for element n = 0 to 5 RbgNumber(n, i) = (RbgNumber(n, i-30) + 3) mod (NumOfRbg - 2); end for end for

Table 4.9.2.2-5: Allocated RB numbers for NR-FR1-TM2 and NR-FR1-TM2a

| DL Test Slots 0-9 (SCS 15 kHz, 30 kHz, 60 kHz) |  |
| --- | --- |
| Slot 0 | RbNumber(0) = 0 |
| Slot 1 | RbNumber(1) = NRB-1 |
| Slots 2-9 | for slot i = 2 to 9 RbNumber(i) = (AllocSpacing * i) mod (NRB-1); end for |
| DL Test Slots 10-19 (SCS 30 kHz, 60 kHz) |  |
| for slot i = 10 to 19 RbNumber(i) = (AllocSpacing * i) mod (NRB-1); end for |  |
| DL Slots 20-39 (SCS 60 kHz) |  |
| for slot i = 20 to 39 RbNumber(i) = (AllocSpacing * i) mod (NRB-1); end for |  |

Where AllocSpacing is defined as:

AllocSpacing = NRB10*2μ

DL Test Slots are allocated to slots in radio frame according to mapping functions specified in table 4.9.2.2.5 for FDD and table 4.9.2.2-6 for TDD.

In case of the FDD, DL Test Slot i is mapped to slot in radio frame i. In case of the TDD, subsequent DL Test Slots are mapped to consecutive DL-only slots.

Table 4.9.2.2-5: Slot mapping function (FDD)

| SCS [kHz] | Slot in radio frame |
| --- | --- |
| 15 | DLOnlySlots = {0, 1, …, 9}; for i = 0 to 9 SlotInRadioFrame(Slots(i)) = DLTestSlot(i); end for |
| 30 | DLOnlySlots = {0, 1, …, 19}; for i = 0 to 19 SlotInRadioFrame(Slots(i)) = DLTestSlot(i); end for |
| 60 | DLOnlySlots = {0, 1, …, 39}; for i = 0 to 39 SlotInRadioFrame(Slots(i)) = DLTestSlot(i); end for |

Table 4.9.2.2-6: Slot mapping function (TDD)

| SCS [kHz] | Slot in radio frame |
| --- | --- |
| 15 | DLOnlySlots = {0, 3, 4, 5, 8, 9}; for i = 0 to 5 SlotInRadioFrame(Slots(i)) = DLTestSlot(i); end for |
| 30 | DLOnlySlots = {0, 1, 2, 6, 7, 8, 9, 10, 11, 12, 16, 17, 18, 19}; for i = 0 to 13 SlotInRadioFrame(Slots(i)) = DLTestSlot(i); end for |
| 60 | DLOnlySlots = {0, 1, 2, 3, 4, 5, 10, 11, 12, 13, 14, 15, 20, 21, 22, 23, 24, 25, 32, 33, 34, 35, 36, 37, 38, 39}; for i = 0 to 26 SlotInRadioFrame(Slots(i)) = DLTestSlot(i); end for |

#### 4.9.2.2.1	NR FR1 test model 1.1 (NR-FR1-TM1.1)

This model shall be used for tests on:

-	BS output power

-	Unwanted emissions

-	Occupied bandwidth

-	ACLR

-	Operating band unwanted emissions

-	Transmitter spurious emissions

-	Transmitter intermodulation

Table 4.9.2.2.1-1: Specific physical channel parameters of NR-FR1-TM1.1

| PDSCH |  |
| --- | --- |
| Downlink resource allocation (RAT) | Type 1 |
| # of QPSK PDSCH PRBs which are boosted | NRB |
| # of QPSK PDSCH PRBs which are deboosted | 0 |

#### 4.9.2.2.2	NR FR1 test model 1.2 (NR-FR1-TM1.2)

This model shall be used for tests on:

-	Unwanted emissions

-	ACLR

-	Operating band unwanted emissions

Table 4.9.2.2.2-1: Specific physical channel parameters of NR-FR1-TM1.2

| PDSCH |  |
| --- | --- |
| Downlink resource allocation (RAT) | Type 0 |
| # of QPSK PDSCH PRBs which are boosted | NRB2PP NRBx |
| Level of boosting [dB] | 3 |
| # of QPSK PDSCH PRBs which are deboosted | NRB-NRBx NRB-NRB2PP |
| Level of deboosting [dB] | 10*log10NRB-NRBxNRB-y*NRBx log10NRB-NRB2PPNRB-LbLINNRB2PP LbLIN = 2 (3 dB) |

where NRB is considering the maximum transmission bandwidth configuration seen in table 5.3.2-1 in TS 38.104.

#### 4.9.2.2.3	NR FR1 test model 2 (NR-FR1-TM2)

This model shall be used for tests on:

-	Total power dynamic range (lower OFDM symbol power limit at min power),

-	EVM of single 64QAM PRB allocation (at min power)

-	Frequency error (at min power)

Common physical channel parameters are defined in tables 4.9.2.2-2, 4.9.2.2-3 and 4.9.2.2-4. Specific physical channel parameters are defined in table 4.9.2.2.3-1. DL Test Slots are allocated to slots in radio frame according to mapping functions specified in table 4.9.2.2.5 for FDD and table 4.9.2.2-6 for TDD.

Table 4.9.2.2.3-1: Specific physical channel parameters of NR-FR1-TM2

| PDSCH |  |
| --- | --- |
| Downlink resource allocation (RAT) | Type 1 |
| # of PDSCH PRBs which are boosted | 1 |
| # of QPSK PDSCH PRBs which are deboosted | NRB - 1 |

#### 4.9.2.2.4	NR FR1 test model 2a (NR-FR1-TM2a)

This model shall be used for tests on:

-	Total power dynamic range (lower OFDM symbol power limit at min power),

-	EVM of single 256QAM PRB allocation (at min power)

-	Frequency error (at min power)

Common physical channel parameters are defined in tables 4.9.2.2-2, 4.9.2.2-3 and 4.9.2.2-4. Specific physical channel parameters are defined in table 4.9.2.2.3-1. DL Test Slots are allocated to slots in radio frame according to mapping functions specified in table 4.9.2.2.5 for FDD and table 4.9.2.2-6 for TDD with all 64QAM PDSCH PRBs replaced by 256QAM PDSCH PRBs.

#### 4.9.2.2.5	NR FR1 test model 3.1 (NR-FR1-TM3.1)

This model shall be used for tests on:

-	Output power dynamics

-	Total power dynamic range (upper OFDM symbol power limit at max power with all 64QAM PRBs allocated)

-	Transmitted signal quality

-	Frequency error

-	EVM for 64QAM modulation (at max power)

Common physical channel parameters are defined in tables are defined in tables 4.9.2.2-2, 4.9.2.2-3 and 4.9.2.2-4. Specific physical channel parameters are defined in table 4.9.2.2.1-1 with 64QAM PDSCH PRBs used.

#### 4.9.2.2.6	NR FR1 test model 3.1a (NR-FR1-TM3.1a)

This model shall be used for tests on:

-	Output power dynamics

-	Total power dynamic range (upper OFDM symbol power limit at max power with all 256QAM PRBs allocated)

-	Transmitted signal quality

-	Frequency error

-	EVM for 256QAM modulation (at max power)

Common physical channel parameters are defined in tables are defined in tables 4.9.2.2-2, 4.9.2.2-3 and 4.9.2.2-4. Specific physical channel parameters are defined in table 4.9.2.2.1-1 with 256QAM PDSCH PRBs used.

#### 4.9.2.2.7	NR FR1 test model 3.2 (NR-FR1-TM3.2)

This model shall be used for tests on:

-	Transmitted signal quality

-	Frequency error

-	EVM for 16QAM modulation

Table 4.9.2.2.7-1: Specific physical ch annel parameters of NR-FR1-TM3.2

| PDSCH |  |
| --- | --- |
| Downlink resource allocation (RAT) | Type 0 |
| # of PDSCH PRBs which are measured | NRB2PP |
| Power change of measured PRBs [dB] / [linear] | [Pc] / [PcLIN] -3 [dB] / 0.5 (16QAM) |
| # of PDSCH PRBs which power is adjusted in order to preserve power balance | NRB-NRB2PP |
| Level of adjustment [dB] | 10log10NRB-NRB2PPNRB-PcLINNRB2PP |

#### 4.9.2.2.8	NR FR1 test model 3.3 (NR-FR1-TM3.3)

This model shall be used for tests on:

-	Transmitted signal quality

-	Frequency error

-	EVM for QPSK modulation

Table 4.9.2.2.8-1: Specific physical ch annel parameters of NR-FR1-TM3.3

| PDSCH |  |
| --- | --- |
| Downlink resource allocation (RAT) | Type 0 |
| # of PDSCH PRBs which are measured | NRB2PP |
| Power change of measured PRBs [dB] / [linear] | [Pc] / [PcLIN] -3 [dB] / 0.5 (16QAM) -6 [dB] / 0.25 (QPSK) |
| # of PDSCH PRBs which power is adjusted in order to preserve power balance | NRB-NRB2PP |
| Level of adjustment [dB] | 10log10NRB-NRB2PPNRB-PcLINNRB2PP |

<End of modified section>