3GPP TSG-RAN WG4 Meeting #88bis	R4-1813687

Chengdu, China, 8 – 12 October 2018

Agenda Item:	7.12.5.3

Source:	Ericsson

Title:	PDCCH simulation results for NR RLM tests

Document for:	Discussion

# 1	Introduction

This contribution provides the PDCCH simulation results for NR RLM test cases.

# 2	Simulation results

Table 1 shows the simulation parameter for PDCCH we investigated, and Table 2 shows the simulation results. For Qin, we should the required SNR (dB) to achieve 2% of BLER. For Qout, we show the required SNR (dB) to achieve 10% of BLER with 4dB power boosting of PDCCH/PDCCH_DMRS.

From our simulation results the SNR separation between Qin and Qout is approximately 5.5dB for static condition and 7-8dB for fading condition.

Table 1	Simulation parameters for RLM test

| Parameters | FR1 SCS=15kHz | FR1 SCS=30kHz | FR2 SCS=120kHz |
| --- | --- | --- | --- |
| Channel BW | 10MHz | 40MHz | 100MHz |
| Format | 1_0 | 1_0 | 1_0 |
| CORESET RB | 24 | 24 | 24 |
| Payload size (W/O CRC) | 39 | 41 | 40 |
| AL | 4 for IS 8 for OOS | 4 for IS 8 for OOS | 4 for IS 8 for OOS |
| CCE-to-REG mapping | Interleaved | Interleaved | Interleaved |
| Interleaver size | 2 | 2 | 2 |
| REG bundling size | 6 | 6 | 6 |
| Propagation channel | AWGN, TDL-C 300ns, 100Hz | AWGN, TDL-C 300ns, 100Hz | AWGN, TDL-A 30ns, 75Hz |
| Antenna configuration | 2x2 2x4 | 2x2 2x4 | 2x2 |

Table 2	Simulation results for RLM tests

| Test number | SCS | Propagation channel | Antenna configuration | Qin (dB) | Qout (dB) | Diff of Qin and Qout (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 15kHz | AWGN | 2x2 | -5.9 | -11.48 | 5.58 |
| 2 |  |  | 2x4 | -7.33 | -13.01 | 5.68 |
| 3 |  | TDL-C 300ns, 100Hz | 2x2 | -1.75 | -10.27 | 8.52 |
| 4 |  |  | 2x4 | -5.53 | -12.93 | 7.4 |
| 5 | 30kHz | AWGN | 2x2 | -5.87 | -11.48 | 5.61 |
| 6 |  |  | 2x4 | -7.37 | -12.99 | 5.62 |
| 7 |  | TDL-C 300ns, 100Hz | 2x2 | -1.69 | -10.18 | 8.49 |
| 8 |  |  | 2x4 | -5.51 | -12.78 | 7.27 |
| 9 | 120kHz | AWGN | 2x2 | -5.87 | -11.49 | 5.62 |
| 10 |  | TDL-A 30ns, 75Hz | 2x2 | -1.5 | -10.08 | 8.58 |

# 3	Conclusion

This contribution provided our PDCCH simulation results for RLM tests. We propose to use our results to decide the test points.