3GPP TSG-RAN WG4 Meeting #90		R4-1900502
Athens, Greece, 25 Feb - 1 Mar 2019

Agenda item:	10.1.1

Source:	Keysight Technologies, GTS

Title:	RTS system setup for 4x4 MIMO OTA testing

Document for:	Discussion

# Introduction

During the last RAN4 meeting, [1] outlined the framework to extend the Radiated Two Stage (RTS) method for rank 4 (4Rx) requirements and for a frequency extension to 7.25GHz. In the meantime, we performed 4×4 RTS MIMO OTA tests on commercial devices with 4 independent downlink streams. Good isolation levels (larger than 15dB isolation between any wanted signal and cross-talk signal) and full throughput for UMa and UMi channel models were achieved with this RTS setup.

The Absolute Data Throughput Framework (ADTF) concept introduced in [2] allows for a given MIMO OTA testing method to emulate the specified network and channel propagation characteristics based on an absolute data throughput metric. The difference between the conducted tests which take into account the known reference antenna patterns and the radiated tests is a measure of the accuracy of the test methodology to emulate the channel model. The ADTF framework was extended in this paper to 4x4 MIMO and performed for LTE FDD Band 3 and LTE TDD Band 39. The maximum throughput with UMa and UMi channel models were be achieved for 4 layers and the differences between cable-conducted and radiated test results were small which proves the RTS setup for 4x4 MIMO is correct.

This proposal will introduce the test setup and parameter configurations for the tests performed. Due to the sensitivity of the absolute performance results, only relative differences between conducted and radiated test results are included in this contribution.

# RTS MIMO OTA System Setup for 4x4 Test

### System Setup

To support four receivers (for the 2 x 4 or 4 x 4 configuration) for the MIMO OTA RTS test methodology, at least two cross-polarized probe antennas located at different positions are required. Figure 1 shows the coordinate system and general probe configuration. The two dual-polarized probe antennas are placed on a circular arc around the DUT in the y-z plane with each dual-polarized probe antennas paced arbitrarily on the arc. The 4 RX DUT is placed in the centre of the arc and can be rotated in the x-y plane. This distributed axes system is able to perform the 3D antenna pattern measurements which can then be used to determine the transmission paths between the four probe antennas/polarizations and the four DUT antennas. The transmission matrix, H, is then described by a 4 x 4 matrix. Changing the probe antenna positions and/or rotating the DUT alters the transmission matrix. To perform the second stage RTS test, it is only necessary to find one transmission matrix which can achieve sufficient isolation after applying an inverted channel matrix using an internal or external channel emulator.

The system setup for the proposed 4x4 RTS MIMO OTA system is shown in Figure 2, which is based on very simple one-box MIMO OTA test solution. Four downlink signals from the Keysight UXM are fed to the probe antennas while the uplink signal is introduced by the communication antenna via the UXM Rx input port. Depending on chamber size and path losses, amplifiers may be needed for downlink and uplink. Both the MIMO fading channel model emulation and inverse matrix are implemented in the UXM without the need of external channel emulation.

Figure 1: Coordinate System Definition

Figure 2: 4x4 RTS MIMO OTA System setup

### Base Station Parameters

For the 4 x 4 MIMO OTA measurements, most BS parameters follow the suggestions in Table 7.1-1 and 7.1-2 from [2] for 2 x 2 MIMO OTA testing. Since the number of layers is extended to 4 (from 2), the maximum theoretical throughput is also doubled. The BS settings to reflect this change are illustrated in Figure 3, a screenshot of the UXM.

Figure 3: UXM screen capture of transmission mode settings

Different PDSCH power offsets relative to RS EPRE were also implemented: PA = -6, PB = 3; PA = -6, PB =1; PA = -3, PB = 1. It was found that the DUT exhibited different performance for different PA and PB combinations. Table 1 and Table 2 match the tables in [2], but the changes for the 4 x 4 configuration are highlighted in yellow.

Table 1: Settings for FDD eNodeB emulator

| eNodeB settings (Note 1) | Unit | Value |
| --- | --- | --- |
| Physical channel |  |  |
| Connection mode of UE |  | Connection established |
| DL MIMO mode |  | 4 x 4 open loop spatial multiplexing |
| Duplex mode |  | FDD |
| Schedule type |  | Reference Measurement Channel (RMC) |
| Reference Channel |  | R.35 |
| Bandwidth DL | MHz | 10 |
| Number of RBs DL |  | 50 |
| Start RB DL |  | 0 |
| Modulation DL |  | 64QAM |
| Maximum Theoretical Throughput | Mbps | 70.11 |
| TBS Idx DL |  | 18 (RMC defined, Note 2) |
| Bandwidth UL | MHz | 10 |
| Number of RBs UL |  | 50 |
| Start RB UL |  | 0 |
| Modulation UL |  | QPSK |
| TBS Idx UL |  | 6 (RMC defined) |
| Transmit power control | dBm | -10/10 MHz, open loop (Note 3) |
| PDSCH power offset relative to RS EPRE | dB | PA = -6, PB = 3; PA = -6, PB =1; PA = -3, PB = 1; |
| Number of HARQ transmissions |  | 1 (no HARQ re-transmissions) |
| AWGN |  | OFF |
| DL power level (RS EPRE) | dBm / 15 kHz | Set at eNodeB simulator with correction from calibration |
| Number of subframes for FOM measurement |  | 2000 minimum for static channel 20000 minimum for faded channel (Note 4) |

Table 2: Settings for TDD eNodeB emulator

| eNodeB settings | Unit | Value |
| --- | --- | --- |
| Physical channel |  |  |
| Connection mode of UE |  | Connection established |
| DL MIMO mode |  | 4 x4 open loop spatial multiplexing |
| Duplex mode |  | TDD |
| Schedule tyoe |  | Reference Measurement Channel (RMC) |
| Reference Channel |  | Table 7.1-3 |
| Up/Downlink Frame Configuration |  | 1 |
| Special Frame configuration |  | 7 |
| Bandwidth DL | MHz | 20 |
| Number of RBs DL |  | 100 |
| Start RB DL |  | 0 |
| Modulation DL |  | 64QAM |
| Maximum Theoretical Throughput | Mbps | 58.6 |
| TBS Idx DL |  | 16 |
| Bandwidth UL | MHz | 20 |
| Number of RBs UL |  | 100 |
| Start RB UL |  | 0 |
| Modulation UL |  | QPSK |
| TBS Idx UL |  | 6 |
| Transmit power control | dBm | -10/20 MHz, open loop (NOTE 2) |
| PDSCH power offset relative to RS EPRE | dB | PA = -6, PB = 3; PA = -6, PB =1; PA = -3, PB = 1; |
| Number of HARQ transmissions |  | 1 (no HARQ re-transmissions) |
| AWGN |  | OFF |
| DL power level (RS EPRE) | dBm / 15 kHz | Set at eNodeB simulator with correction from calibration |
| Number of subframes for FOM measurement |  | 2000 minimum for static channel 20000 minimum for faded channel |

### Channel model parameters

For the channel model configuration, both UMi and UMa channel fading models outlined in [2] were implemented. The key difference is the BS antenna implementation: The BS antennas assumption for the 2x2 implementation in [2] were, dual polarized equal power elements with a fixed 0λ separation, 45 degrees slanted. For the 4x4 implementation, two dual identical polarized equal power elements as for 2x2 were assumed with a fixed 2 lambda separation.

# ADTF Validation by Cable-conducted test vs Radiated test

The ADTF concept is defined in [2] was used to validate the 4x4 RTS MIMO OTA test setup after extending the concept to 4x4 MIMO. The accuracy in emulating the specified network and channel propagation is validated by looking at the difference in conducted and radiated test results, i.e., the P_MODE and S_Mode results [3] for the 70% KPI.

In the 1st stage, the DUT’s 4 Rx antenna patterns were measured in magnitude and phase using the Antenna Test Function (ATF) as shown in Figure 2.  The conducted ADTF stage test setup is illustrated in Figure 4; the UXM is performing various throughput tests with the DUT connected to the UXM via RF cables to the conducted ports while the internal channel emulator is emulating the antenna patterns measured in the 1st stage. In the radiated ADTF test, the DUT is placed inside the chamber and the test system software will identify the optimal probe antenna configuration, tune the inverse channel matrix and validate the inverse matrix can achieve >15dB isolation. Subsequently, the radiated throughput tests are performed while the internal channel emulator is emulating the antenna patterns measured in the 1st stage.

Figure 4: Cable-conducted 2nd stage test setup

The experiments were performed in LTE FDD Band 3 and LTE TDD Band 39. For all bands and UE evaluated, the maximum throughput for UMa and UMi channel models and rank 4 were achieved. The performance comparison was done for each individual angle (P_MODE) and the average results (S_MODE) over 12 rotation angles. The differences between cable-conducted and radiated test results were very small: the maximum difference on individual angle comparison (P_MODE) we observed was 0.82dB while for most cases the P_MODE and S_MODE results were within 0.5dB, which proves the presented 4x4 RTS MIMO OTA setup is correct.

# Conclusion

This proposal briefly presented the 4 x 4 RTS MIMO OTA test setup and configuration of important 4x4 parameters. The ADTF framework was used to outline the accuracy in emulating the specified network and channel propagation.

It is assumed that RTS 4x4 MIMO OTA testing is applicable to FR1 NR MIMO in a similar fashion as outlined here for LTE. Changes to the BS antenna assumptions and channel models/scenarios are straightforward since they are implemented in baseband, e.g., the extension to 3D required for NR by 38.901 is not an issue for the RTS method and the channel models can be emulated very accurately.

# References

- R4-1814834, Extension of RTS test method from 2Rx to 4Rx and up to 7.25 GHz, Keysight Technologies, GTS, 3GPP TSG-RAN WG4 Meeting #89, November 2018

- TR 37.977, Verification of radiated multi-antenna reception performance of User Equipment (UE), V14.6.0 (2018-06)

- TS 37.144, User Equipment (UE) and Mobile Station (MS) GSM, UTRA and E-UTRA over the air performance requirements, V15.0.0 (2018-06)