3GPP TSG-RAN WG4 Meeting #90 	R4-1901347

Athens, Greece, 25th Feb -  1st Mar 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-2 | CR | - | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to 38.141-2 – PUSCH requirements with CP-OFDM for FR2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-03-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduce the SNR requirement for PUSCH FR2 with CP-OFDM tests based on 38.141-2 v15.0.0. Clean up the tables and add missing note back from previous agreed TP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the SNR requirement for PUSCH FR2 with CP-OFDM tests based on 38.141-2 v15.0.0. Clean up the tables and add missing note back from previous agreed TP. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | New requirement for PUSCH can’t be verified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<start of the change>

##### 8.2.1.5.2	Test requirement for BS type 2-O

The throughput measured according to subclause 8. 2.1.4.2 shall not be below the limits for the SNR levels specified in table 8. 2.1.5.2-1 to 8.2.1.5.2-5.

Table 8.2.1.5.2-1: Test requirements for PUSCH, 50 MHz Channel Bandwidth, 60 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex TBD) | Fraction of maximum throughput | FRC (Annex A) | DMRS configuration | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-300 | 70 % | [TBD] | 1+0 | [TBD] |
|  |  | Normal | TDLA30-300 | 70 % | [TBD] | 1+0 | [TBD] |
|  |  | Normal | TDLA30-75 | 70 % | [TBD] | 1+0 | [TBD] |
| 2 |  | Normal | TDLA30-300 | 70 % | [TBD] | 1+0 | [TBD] |
|  |  | Normal | TDLA30-300 | 70 % | [TBD] | 1+0 | [TBD] |

Table 8.2.1.5.2-2: Test requirements for PUSCH, 100 MHz Channel Bandwidth, 60 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex TBD) | Fraction of maximum throughput | FRC (Annex A) | DMRS configuration | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-300 | 70 % | [TBD] | 1+0 | [TBD] |
|  |  | Normal | TDLA30-300 | 70 % | [TBD] | 1+0 | [TBD] |
|  |  | Normal | TDLA30-75 | 70 % | [TBD] | 1+0 | [TBD] |
| 2 |  | Normal | TDLA30-300 | 70 % | [TBD] | 1+0 | [TBD] |
| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex TBD) | Fraction of maximum throughput | FRC (Annex A) | DMRS configuration | SNR (dB) |
| 1 | 2 | Normal | TDLA30-300 | 70 % | [TBD] | 1+0 | [TBD] |
|  |  | Normal | TDLA30-300 | 70 % | [TBD] | 1+0 | [TBD] |
|  |  | Normal | TDLA30-75 | 70 % | [TBD] | 1+0 | [TBD] |
| 2 |  | Normal | TDLA30-300 | 70 % | [TBD] | 1+0 | [TBD] |

Table 8.2.1.5.2-3: Test requirements for PUSCH, 50 MHz Channel Bandwidth, 120 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex TBD) | Fraction of maximum throughput | FRC (Annex A) | DMRS configuration | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-300 | 70 % | [TBD] | 1+0 | [TBD] |
|  |  | Normal | TDLA30-300 | 70 % | [TBD] | 1+0 | [TBD] |
|  |  | Normal | TDLA30-75 | 70 % | [TBD] | 1+0 | [TBD] |
| 2 |  | Normal | TDLA30-300 | 70 % | [TBD] | 1+0 | [TBD] |
|  |  | Normal | TDLA30-300 | 70 % | [TBD] | 1+0 | [TBD] |

Table 8.2.1.5.2-4: Test requirements for PUSCH, 100 MHz Channel Bandwidth, 120 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex TBD) | Fraction of maximum throughput | FRC (Annex A) | DMRS configuration | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-300 | 70 % | [TBD] | 1+0 | [TBD] |
|  |  | Normal | TDLA30-300 | 70 % | [TBD] | 1+0 | [TBD] |
|  |  | Normal | TDLA30-75 | 70 % | [TBD] | 1+0 | [TBD] |
| 2 |  | Normal | TDLA30-300 | 70 % | [TBD] | 1+0 | [TBD] |

Table 8.2.1.5.2-5: Test requirements for PUSCH, 200 MHz Channel Bandwidth, 120 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex TBD) | Fraction of maximum throughput | FRC (Annex A) | DMRS configuration | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-300 | 70 % | [TBD] | 1+0 | [TBD] |
|  |  | Normal | TDLA30-300 | 70 % | [TBD] | 1+0 | [TBD] |
|  |  | Normal | TDLA30-75 | 70 % | [TBD] | 1+0 | [TBD] |
| 2 |  | Normal | TDLA30-300 | 70 % | [TBD] | 1+0 | [TBD] |
|  |  | Normal | TDLA30-300 | 70 % | [TBD] | 1+0 | [TBD] |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in Annex C.

<end of the change>