3GPP RAN WG4 Meeting #90 	R4-1902501

# Athens, Greece, February 25 – March 1, 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | DRAFT | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.101-2: Introduction of the requirement on beam correspondence |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple Inc., [Samsung], [Qualcomm], [Intel], … |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2019-02-25 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Beam correspondence is an important UL beam management tool in NR FR2 networks, and many system performance indicators seek to optimize UL beam management metrics, such as overhead for beam training, network access latency, and UL beam gain. During the RAN #82 meeting a number of agreements were reached on the beam correspondence feature in RP-182879, and the feature list was updated in RP-182866 with the following RAN decisions captured for FG 2-20: Mandatory with capability signaling - UE that fulfills the beam correspondence requirement without the uplink beam sweeping shall set the bit to 1 - UE that fulfills the beam correspondence requirement with the uplink beam sweeping shall set the bit to 0 FG 2-30 was further updated based on the agreements: Optional with capability signaling - Capability signaling shall be set to 1 if 2-20 is set to 0 - For the UE meeting the minimum peak EIRP and spherical coverage requirements without the uplink beam sweeping, this feature is optional A general requirement on beam correspondence is needed to reflect the RAN agreements. A requirement specific to PC3 is needed to reflect the RAN agreements which are specific to PC3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduces a reference to the UE capabilities specification (38.306), new definitions and symbols relevant to the definition of beam correspondence, a general UE requirement on beam correspondence, a requirement for PC3, and a definition of how this requirement is verified. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A UE requirement on beam correspondence would not be defined in the Rel-15 NR specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.1, 3.2, 6.6.1, 6.6.1.1 (new), 6.6.4, 6.6.4.1 (new), 6.6.4.2 (new), 6.6.4A (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 38.521-2 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<<<start of change 1>>>

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.101-1: “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”

[3]	3GPP TS 38.101-3: “NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios”

[4]	3GPP TR 38.810: “Study on test methods for New Radio”

[5]	3GPP TS 38.521-2: “NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone"

[6]	Recommendation ITU-R M.1545: "Measurement uncertainty as it applies to test limits for the terrestrial component of International Mobile Telecommunications-2000"

[7]	ITU-R Recommendation SM.329-10, "Unwanted emissions in the spurious domain"

[8]	47 CFR Part 30, “UPPER MICROWAVE FLEXIBLE USE SERVICE, §30.202   Power limits”, FCC.

[9]	3GPP TS 38.211: "NR; Physical channels and modulation".

[10]	3GPP TS 38.213: "NR; Physical layer procedures for control".

[11]	3GPP TS 38.215: "NR; Physical layer measurements".

[12]	3GPP TS 38.133: "NR; Requirements for support of radio resource management".

[13]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[14]	3GPP TS 38.306: “NR; User Equipment (UE) radio access capabilities”.

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Aggregated Channel Bandwidth: The RF bandwidth in which a UE transmits and receives multiple contiguously aggregated carriers.

Beam correspondence: the ability of the UE to select a suitable beam for UL transmission based on DL measurements with or without relying on UL beam sweeping.

Carrier aggregation: Aggregation of two or more component carriers in order to support wider transmission bandwidths.

Carrier aggregation band: A set of one or more operating bands across which multiple carriers are aggregated with a specific set of technical requirements.

Carrier aggregation bandwidth class: A class defined by the aggregated transmission bandwidth configuration and maximum number of component carriers supported by a UE.

Carrier aggregation configuration: A combination of CA operating band(s) and CA bandwidth class(es) supported by a UE.

NOTE:	Carriers aggregated in each band can be contiguous or non-contiguous.

EIRP(Link=Link angle, Meas=Link angle): measurement of the UE such that the link angle is aligned with the measurement angle. EIRP (indicator to be measured) can be replaced by EIS, Frequency, EVM, carrier Leakage, In-band eission and OBW. Beam peak search grids, TX beam peak direction, and RX beam peak direction can be selected to describe Link.

EIRP(Link=Link angle, Meas=beam peak direction): measurement of the EIRP of the UE such that the measurement angle is aligned with the beam peak direction within an acceptable measurement error uncertainty.

Fallback group: Group of carrier aggregation bandwidth classes for which it is mandatory for a UE to be able to fallback to lower order CA bandwidth class configuration. It is not mandatory for a UE to be able to fallback to lower order CA bandwidth class configuration that belong to a different fallback group

Inter-band carrier aggregation: Carrier aggregation of component carriers in different operating bands.

NOTE:	Carriers aggregated in each band can be contiguous or non-contiguous.

Intra-band contiguous carrier aggregation: Contiguous carriers aggregated in the same operating band.

Intra-band non-contiguous carrier aggregation: Non-contiguous carriers aggregated in the same operating band.

Link angle: a DL-signal AoA from the view point of the UE, as described in Table C.2-1 in [4].

Measurement angle: the angle of measurement of the desired metric from the view point of the UE, as described in Table C.2-1 in [4].

radiated interface boundary: operating band specific radiated requirements reference point where the radiated requirements apply

RX beam peak direction: direction where the maximum total component of RSRP and thus best total component of EIS is found

Sub-block: This is one contiguous allocated block of spectrum for transmission and reception by the same UE. There may be multiple instances of sub-blocks within an RF bandwidth.

TX beam peak direction: direction where the maximum total component of  EIRP is found

TRP(Link=Link angle): measurement of the TRP of the UE such that the measurement angle is aligned with the beam peak direction within an acceptable measurement uncertainty. TX beam peak direction and RX beam peak direction can be selected to describe Link.

NOTE:	For requirements based on EIRP/EIS, the radiated interface boundary is associated to the far-field region

UE transmission bandwidth configuration: Set of resource blocks located within the UE channel bandwidth which may be used for transmitting or receiving by the UE.

Vehicular UE: A UE embedded in a vehicle

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

∆EIRPBC	The beam correspondence tolerance, where ∆EIRPBC = EIRP2 – EIRP1

ΔFGlobal	Granularity of the global frequency raster

ΔFRaster	Band dependent channel raster granularity

ΔfOOB	Δ Frequency of Out Of Band emission

ΔRB	The starting frequency offset between the allocated RB and the measured non-allocated RB

ΔRIB	Allowed reference sensitivity relaxation due to support for inter-band CA operation

ΔMBP,n	Allowed relaxation to each, minimum peak EIRP and reference sensitivity due to support for multi-band operation, per band in a combination of supported bands

ΔMBS,n	Allowed relaxation to each, EIRP spherical coverage and EIS spherical coverage due to support for multi-band operation, per band in a combination of supported bands

∑MBP	Total allowed relaxation to each, minimum peak EIRP and reference sensitivity due to support for multi-band operation, for all bands in a combination of supported bands

∑MBS	Total allowed relaxation to each, EIRP spherical coverage and EIS spherical coverage due to support for multi-band operation, for all bands in a combination of supported bands

BWChannel	Channel bandwidth

BWChannel_CA	Aggregated channel bandwidth, expressed in MHz

BWGB	max( BWGB,Channel(k) )

BWGB,Channel(k)	Minimum guard band defined in sub-clause 5.3.3 of carrier k

BWinterferer	Bandwidth of the interferer

Ceil(x)	Rounding upwards; ceil(x) is the smallest integer such that ceil(x) ≥ x

EIRP1	The measured total EIRP based on the beam the UE chooses autonomously (corresponding beam) to transmit in the direction of the incoming DL signal, which is based on beam correspondence without relying on UL beam sweeping

EIRP2	The measured total EIRP based on the beam yielding highest EIRP in a given direction, which is based on beam correspondence with relying on UL beam sweeping

EIRPmax	The applicable maximum EIRP as specified in sub-clause 6.2.1

Floor(x)	Rounding downwards; floor(x) is the greatest integer such that floor(x) ≤ x

FC	RF reference frequency for the carrier center on the channel raster, given in table 5.4.2.2-1

FC,block, high	Fc of the highest transmitted/received carrier in a sub-block.

FC,block, low	Fc of the lowest transmitted/received carrier in a sub-block.

FC, low	The Fc of the lowest carrier, expressed in MHz.

FC, high	The Fc of the highest carrier, expressed in MHz.

Fedge, low	The lower edge of Aggregated Channel Bandwidth, expressed in MHz. Fedge, low = FC, low - Foffset, low.

Fedge, high	The upper edge of Aggregated Channel Bandwidth, expressed in MHz. Fedge, high = FC, high + Foffset, high.

Fedge,block,low	The lower sub-block edge, where Fedge,block,low = FC,block,low - Foffset, low.

Fedge,block,high	The upper sub-block edge, where Fedge,block,high = FC,block,high + Foffset, high.

FInterferer (offset)	Frequency offset of the interferer (between the center frequency of the interferer and the carrier frequency of the carrier measured)

FInterferer	Frequency of the interferer

Foffset, low	Frequency offset from FC, low to the lower UE RF Bandwidth edge, or from FC,block, low to the lower sub-block edge

Foffset, high	Frequency offset from FC, high to the upper UE RF Bandwidth edge, or from FC,block, high to the upper sub-block edge

F_center	The center frequency of an allocated block of PRBs

FDL_low	The lowest frequency of the downlink operating band

FDL_high	The highest frequency of the downlink operating band

FUL_low	The lowest frequency of the uplink operating band

FUL_high	The highest frequency of the uplink operating band

FInterferer (offset)	Frequency offset of the interferer (between the center frequency of the interferer and the carrier frequency of the carrier measured)

FInterferer	Frequency of the interferer

FIoffset	Frequency offset of the interferer (between the center frequency of the interferer and the closest edge of the carrier measured)

FOOB	The boundary between the NR out of band emission and spurious emission domains

FREF	RF reference frequency

FREF-Offs	Offset used for calculating FREF

FUL_Meas	The sub-carrier frequency for which the equalizer coefficient is evaluated

GBChannel	Minimum guard band defined in sub-clause 5.3.3

LCRB	Transmission bandwidth which represents the length of a contiguous resource block allocation expressed in units of resources blocks

LCRB,Max	Maximum number of RB for a given Channel bandwidth and sub-carrier spacing

Max()	The largest of given numbers

Min()	The smallest of given numbers

MPRf,c	Maximum output power reduction for carrier f of serving cell c

MPRnarrow	Maximum output power reduction due to narrow PRB allocation

MPRWT	Maximum power reduction due to modulation orders, transmit bandwidth configurations, waveform types

nPRB	Physical resource block number

NRACLR	NR ACLR

NRB	Transmission bandwidth configuration, expressed in units of resource blocks

NRB,low	Transmission bandwidth configurations according to Table 5.3.2-1 for the lowest assigned component carrier in section 5.3A.1

NRB,high	Transmission bandwidth configurations according to Table 5.3.2-1 for the highest assigned component carrier in section 5.3A.1

NREF	NR Absolute Radio Frequency Channel Number (NR-ARFCN)

NREF-Offs	Offset used for calculating NREF

PCMAX	The configured maximum UE output power

PCMAX, f, c	The configured maximum UE output power for carrier f of serving cell c

Pmax	The maximum UE output power as specified in sub-clause 6.2.1

Pmin	The minimum UE output power as specified in sub-clause 6.3.1

Pint	The intermediate power point as defined in table 6.3.4.2-2

PInterferer	Modulated mean power of the interferer

P-MPRf,c	The Power Management UE Maximum Power Reduction for carrier f of serving cell c

PPowerClass	Nominal UE power class (i.e., no tolerance) as specified in sub-clause 6.2.1

PRB	The transmitted power per allocated RB, measured in dBm

PTMAX,f,c	The measured total radiated power for carrier f of serving cell c

PUMAX	The measured configured maximum UE output power

Pw	Power of a wanted DL signal

RBstart	Indicates the lowest RB index of transmitted resource blocks

SCSlow	SCS for the lowest assigned component carrier in section 5.3A.1

SCShigh	SCS for the highest assigned component carrier in section 5.3A.1

SSREF	SS block reference frequency position

T(∆P)	The tolerance T(∆P) for applicable values of ∆P (values in dB)

TRPmax	The maximum TRP for the UE power class as specified in sub-clause 6.2.1

<<<end of change 1>>>

<<<start of change 2>>>

## 6.6	Beam correspondence

### 6.6.1	General

Beam correspondence is the ability of the UE to select a suitable beam for UL transmission based on DL measurements with or without relying on UL beam sweeping.  The beam correspondence requirement is satisfied assuming the presence of both SSB and CSI-RS signals and Type D QCL is maintained between SSB and CSI-RS.

### 6.6.2	Beam correspondence for PC1

### 6.6.3	Beam correspondence for PC2

### 6.6.4	Beam correspondence for PC3

#### 6.6.4.1	General

The beam correspondence requirement for PC3 UEs consists of three components: UE minimum peak EIRP (as defined in Clause 6.2.1.3), UE spherical coverage (as defined in Clause 6.2.1.3), and beam correspondence tolerance (as defined in Clause 6.6.4.2).  The beam correspondence requirement is fulfilled if the UE satisfies one of the following conditions, depending on the UE’s beam correspondence capability, as defined in TS38.306 [14]:

-	If [bit-1], the UE shall meet the minimum peak EIRP requirement according to Table 6.2.1.3-1 and spherical coverage requirement according to Table 6.2.1.3-3 with its autonomously chosen UL beams and without uplink beam sweeping.  Such a UE is considered to have met the beam correspondence tolerance requirement.

-	If [bit-0], the UE shall meet the minimum peak EIRP requirement according to Table 6.2.1.3-1 and spherical coverage requirement according to Table 6.2.1.3-3 with uplink beam sweeping.  Such a UE shall meet the beam correspondence tolerance requirement defined in Clause 6.6.4.2 and shall support uplink beam management, as defined in TS38.306 [14].

#### 6.6.4.2	Beam correspondence tolerance for PC3

The beam correspondence tolerance requirement ∆EIRPBC for power class 3 UEs is defined based on a percentile of the distribution of ∆EIRPBC, defined as ∆EIRP = EIRP2 - EIRP1 over the link angles spanning a subset of the beam peak search grid, such that

-	EIRP1 is the total EIRP calculated based on the beam the UE chooses autonomously (corresponding beam) to transmit in the direction of the incoming DL signal, which is based on beam correspondence without relying on UL beam sweeping.

-	EIRP2 is the best total EIRP (beam yielding highest EIRP in a given direction) which is based on beam correspondence with relying on UL beam sweeping.

-	The link angles are the ones corresponding to the top 50% of the EIRP2 measurement over the whole sphere.

For power class 3 UEs, the requirement is fulfilled if the UE’s corresponding UL beams satisfy the maximum limit in Table 6.6.4.2-1.

Table 6.6.4.2-1: UE beam correspondence tolerance for power class 3

| Operating band | Max ∆EIRPBC at X %-tile ∆EIRP CDF (dB) |
| --- | --- |
| n257 | TBD |
| n258 | TBD |
| n260 | TBD |
| n261 | TBD |

### 6.6.5	Beam correspondence for PC4

## 6.6A	Void

<<<end of change 2>>>