3GPP TSG-RAN WG4 Meeting #90bis	R4-1903015

# Xi'an, China,  8 Apr - 12 Apr 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-2 | CR | <CR#> | rev | <Rev#> | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.141-2 Addition of FR2 TRP OFF test procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 10-04-2019 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The method of translating TRP OFF requirement to the relative EIRP OFF was determined in the last meeting, however, FR2 TRP OFF test procedure is not documented in the current specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | FR2 TRP OFF test procedure is added to the corresponding section. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No test procedure for FR2 TRP OFF is documented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<START OF  CHANGE>

6.5.1	OTA transmitter OFF power

6.5.1.1	Definition and applicability

OTA transmitter OFF power requirements apply only to TDD operation of NR BS.

OTA transmitter OFF power is defined as the mean power measured over 70/N µs filtered with a square filter of bandwidth equal to the transmission bandwidth configuration of the BS (BWConfig) centred on the assigned channel frequency during the transmitter OFF period. N = SCS/15, where SCS is Sub Carrier Spacing in kHz.

For BS supporting intra-band contiguous CA, the transmitter OFF power is defined as the mean power measured over 70/N µs filtered with a square filter of bandwidth equal to the Aggregated BS Channel Bandwidth BWChannel_CA centred on (Fedge_high+Fedge_low)/2 during the transmitter OFF period.

For BS type 1-O, the transmitter OFF power is defined as the output power at the co-location test antenna conducted output(s). For BS type 2-O the transmitter OFF power is defined as TRP.

For multi-band RIBs or single band RIBs supporting transmission in multiple bands, the requirement is only applicable during the transmitter OFF period in all supported operating bands.

6.5.1.2	Minimum requirement

The minimum requirement for BS type 1-O is in TS 38.104 [2], subclause 9.5.2.2.

The minimum requirement for BS type 2-O is in TS 38.104 [2], subclause 9.5.2.3.

6.5.1.3	Test purpose

The purpose of this test is to verify the OTA transmitter OFF power is within the limits of the minimum requirements.

6.5.1.4	Method of test

6.5.1.4.1	Initial conditions

Test environment: Normal; see annex B.2.

RF channels to be tested for single carrier: M; see subclause 4.9.1.

Base Station RF Bandwidth positions to be tested for multi-carrier and/or CA:

-	MRFBW in single band operation, see subclause 4.9.1;

-	BRFBW_T'RFBW and B'RFBW_TRFBW in multi-band operation; see subclause 4.9.1.

Directions to be tested:

-	The requirement for FR1 is specified as co-location requirement. For general description of co-location requirements, refer to subclause 4.12.

-	The requirement for FR2 is specified as TRP requirement. Due to the infeasibility of measuring TRP directly, the test direction is selected as the peak EIRP direction to obtain relative EIRP measurement.

6.5.1.4.2	Procedure

6.5.1.4.2.1	General procedure

1)	Place the BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (D.2) of the BS with the test system.

6.5.1.4.2.2	BS type 1-O

3)	Set the BS in the direction of the declared beam peak direction of the beam direction pair, for the beam to be tested.

4)	Place the co-location test antenna as specified in subclause 4.12.

5)	Configure the beam peak direction of the BS according to the declared beam direction pair.

6)	Set the BS to transmit according to the applicable test configuration in subclause 4.8 using the corresponding test model NR-FR1-TM1.1 and set of physical channels in subclause 4.9.2.

For a BS declared to be capable of multi-carrier and/or CA operation, use the applicable test signal configuration and corresponding power setting specified in subclauses 4.7.2 and 4.8 using the corresponding test model NR-FR1-TM1.1 and set of physical channels in subclause 4.9.2 on all carriers configured.

7)	Measure the mean power spectrum density at the output(s) of co-location test antenna as power sum over all supported polarizations over 70/N μs filtered with a square filter of bandwidth equal to the RF bandwidth of the NR BS centred on the central frequency of the RF bandwidth. 70/N μs average window centre is set from 35/N μs after end of one transmitter ON period + 10 μs to 35/N μs before start of next transmitter ON period - 10 μs. N = SCS/15, where SCS is Sub Carrier Spacing in kHz.

8)	For an NR BS supporting contiguous CA, measure the mean power spectral density at the output(s) of co-location test antenna as power sum over all supported polarizations over 70/N μs filtered with a square filter of bandwidth equal to the Aggregated Channel Bandwidth BWChannel_CA centred on (Fedge_high+Fedge_low)/2. 70/N μs average window centre is set from 35/N μs after end of one transmitter ON period + 10 μs to 35/N μs before start of next transmitter ON period - 10 μs. N = SCS/15, where SCS is the smallest supported Sub Carrier Spacing in kHz in the Aggregated BS Channel Bandwidth.

In addition, for a multi-band RIB, the following steps shall apply:

9)	For a multi-band RIB and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

6.5.1.4.2.3	BS type 2-O

3)  Move the BS on the positioner in order that the reference direction of the transmitter aligns with the test antenna.

4)  Configure the BS such that the beam peak direction(s) applied during the power measurement step 6 are consistent with the grid and measurement approach for the TRP test.

5)  Set the BS to transmit according to the applicable test configuration in subclause 4.7 using the corresponding test models NR-FR2-TM1.1 and set of physical channels in subclause 4.9.

6)	 Measure the EIRP at the peak direction as P_EIRP_ON at maximum rated power when the transmitter is in ON state according to a) and b). EIRP is measured over 70/N μs filtered with a square filter of bandwidth equal to the RF bandwidth of the BS centred on the central frequency of the RF bandwidth.

a)	If the test facility only supports single polarization, then measure EIRP with the test facility's test antenna/probe polarization matched to the NR BS.

b)	If the test facility supports dual polarization then measure total EIRP for two orthogonal polarizations (denoted p1 and p2) and calculate total radiated transmit power for particular beam direction pair as EIRP = EIRPp1 + EIRPp2.

7)	 Measure the EIRP at the peak direction as P_EIRP_OFF when the transmitter is in OFF state according to a) and b). EIRP is measured over 70/N μs filtered with a square filter of bandwidth equal to the RF bandwidth of the BS centred on the central frequency of the RF bandwidth. 70/N μs average window centre is set from 35/N μs after end of one transmitter ON period + 3 μs to 35/N μs before start of next transmitter ON period -3 μs. N = SCS/15, where SCS is Sub Carrier Spacing in kHz.

a)	If the test facility only supports single polarization, then measure EIRP with the test facility's test antenna/probe polarization matched to the NR BS.

b)	If the test facility supports dual polarization then measure total EIRP for two orthogonal polarizations (denoted p1 and p2) and calculate total radiated transmit power for particular beam direction pair as EIRP = EIRPp1 + EIRPp2.

8)	For a BS supporting contiguous CA, measure P_EIRP_OFF as power sum over all supported polarizations. EIRP is measured over 70/N μs filtered with a square filter of bandwidth equal to the Aggregated Channel Bandwidth BWChannel_CA centred on (Fedge_high+Fedge_low)/2. 70/N μs average window centre is set from 35/N μs after end of one transmitter ON period + 3 μs to 35/N μs before start of next transmitter ON period - 3 μs. N = SCS/15, where SCS is Sub Carrier Spacing in kHz in the Aggregated BS Channel Bandwidth.

In addition, for a multi-band RIB, the following steps shall apply:

9)	For a multi-band RIB and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

10)	TRP in the ON state P_TRP_ON is measured in subclause 6.3 and corresponds to TRP of step 6. Calculate the TRP in OFF state as P_TRP_OFF = P_EIRP_OFF – P_EIRP_ON + P_TRP_ON.

6.5.1.5	Test requirements

6.5.1.5.1	BS type 1-O

The mean power spectral density measured according to subclause 6.5.1.4.2 shall be less than -102.6 dBm/MHz for carrier frequency f ≤ 3.0 GHz.

The mean power spectral density measured according to subclause 6.5.1.4.2 shall be less than -102.4 dBm/MHz for carrier frequency 3.0 GHz < f ≤ 6.0 GHz.

For multi-band RIB, the requirement is only applicable during the transmitter OFF period in all supported operating bands.

6.5.1.5.2	BS type 2-O

The measured mean power spectral density P_TRP_OFF according to subclause 6.5.1.4.2 shall be less than -36 dBm/MHz for carrier frequency 24.25 GHz < f ≤ 29.5 GHz.

The measured mean power spectral density P_TRP_OFF according to subclause 6.5.1.4.2 shall be less than -36 dBm/MHz for carrier frequency 37 GHz < f ≤ 40 GHz.

<END OF CHANGE>