3GPP TSG-RAN4 Meeting #90bis 	R4-1903404

Xi’an, China, 8th-12th April, 2019

Title:	Reply LS on capability of measurement gap patterns

Reply To:	R2-1902732

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	RAN WG 4

To:	RAN WG 2

Cc:

Contact Person:

Name:	Jie Cui

E-mail Address:	jie.cui@intel.com

Attachments:	N.A.

1. Overall Description:

RAN4 would like to thank RAN2 for the LS on capability of measurement gap patterns (R2-1902732). After discussion in RAN4, the feedback to RAN2 for their future work is as below,

- MG patterns 0 and 1 are mandatory for all cases except MG based RSTD measurements. Only MG pattern #0 is mandatory for MG based RSTD measurements.

- For LTE SA and EN-DC, RAN4 updated TS36.133 and TS38.133 to capture the UE capability signalling of both shortMeasurementGap-r14 and measGapPatterns via LTE RRC, which are duplicated as,

Revision to TS36.133:

Table 8.1.2.1-1: Gap Pattern Configurations supported by the UE

| Gap Pattern Id | MeasurementGap Length (MGL, ms) | Measurement Gap Repetition Period (MGRP, ms) | Minimum available time for inter-frequency and inter-RAT measurements during 480ms period (Tinter1, ms) | Measurement Purpose |
| --- | --- | --- | --- | --- |
| 0 | 6 | 40 | 60 | Inter-Frequency E-UTRAN FDD and TDD, UTRAN FDD, GERAN, LCR TDD, HRPD, CDMA2000 1x, inter-RAT NR |
| 1 | 6 | 80 | 30 | Inter-Frequency E-UTRAN FDD and TDD, UTRAN FDD, GERAN, LCR TDD, HRPD, CDMA2000 1x, inter-RAT NR |
| 2 | 3 | 40 | 24NOTE 1,2 | Inter-Frequency E-UTRAN FDD and TDD for cells with time difference as specified below. inter-RAT NR |
| 3 | 3 | 80 | 12NOTE 1,2 | Inter-Frequency E-UTRAN FDD and TDD for cells with time difference according as specified below. inter-RAT NR |
| [4] | [6] | [20] | 120 Note 1, 6 | inter-RAT NR, Inter-Frequency E-UTRAN FDD and TDD |
| [5] | 6 | 160 | Note 3 | inter-RAT NR |
| [6] | [4] | [20] | 72 Note 1, 5, 7 | inter-RAT NR, Inter-Frequency E-UTRAN FDD and TDD |
| [7] | 4 | 40 | 36 Note 1, 5, 8 | inter-RAT NR, Inter-Frequency E-UTRAN FDD and TDD |
| [8] | 4 | 80 | 18Note 1, 5, 9 | inter-RAT NR, Inter-Frequency E-UTRAN FDD and TDD |
| [9] | [4] | [160] | Note 3 | inter-RAT NR |
| [10] | 3 | 20 | 48 Note 1, 5 | inter-RAT NR, Inter-Frequency E-UTRAN FDD and TDD |
| [11] | 3 | 160 | Note 3 | inter-RAT NR |
| NOTE 1: When determing UE requirements using Tinter1 for GP2, 3, 4, 6, 7, 8, 10, Tinter1 = [60] for GP2, GP4, GP6, GP7, GP10 and Tinter1 = [30] for GP3 and GP8 shall be used. NOTE 2: This gap pattern can be configured for E-UTRA measurement when UEs support shortMeasurementGap-r14 or can be configured for inter-RAT NR measurement only. NOTE 3: This gap pattern can be used only for measurement of NR carriers, and Tinter is not applicable. NOTE 4: Void NOTE 5: This gap pattern is supported by UEs which are configured to perform both E-UTRA inter-frequency measurement and inter-RAT NR measurement when UEs indicate the capability in signalling measGapPatterns to network or supported by UEs configured to perform inter-RAT NR measurement only. NOTE 6: This gap pattern is supported by UEs which are configured to perform both E-UTRA inter-frequency measurement and inter-RAT NR measurement or supported by UEs configured to perform inter-RAT NR measurement only. NOTE 7: When this gap pattern is used, the Tinter for E-UTRA interfrequency measurements is 48ms corresponding to the first 3ms of the 4ms gap NOTE 8: When this gap pattern is used, the Tinter for E-UTRA interfrequency measurements is 24ms corresponding to the first 3ms of the 4ms gap NOTE 9: When this gap pattern is used, the Tinter for E-UTRA interfrequency measurements is 12ms corresponding to the first 3ms of the 4ms gap |  |  |  |  |

Revision to TS38.133:

Table 9.1.2-2: Applicability for Gap Pattern Configurations supported by the E-UTRA-NR dual connectivity UE or NR-E-UTRA dual connectivity UE

| Measurement gap pattern configuration | Serving cell | Measurement Purpose | Applicable Gap Pattern Id |
| --- | --- | --- | --- |
| Per-UE measurement gap | E-UTRA + FR1, or E-UTRA + FR2, or E-UTRA + FR1 + FR2 | non-NR RAT Note1,2 | 0,1,2,3 |
|  |  | FR1 and/or FR2 | 0-11 |
|  |  | non-NR RATNote1,2 and FR1 and/or FR2 | 0, 1, 2, 3, 4, 6, 7, 8,10 |
| Per FR measurement gap | E-UTRA and, FR1 if configured | non-NR RAT Note1,2 | 0,1,2,3 |
|  | FR2 if configured |  | No gap |
|  | E-UTRA and, FR1 if configured | FR1 only | 0-11 |
|  | FR2 if configured |  | No gap |
|  | E-UTRA and, FR1 if configured | FR2 only | No gap |
|  | FR2 if configured |  | 12-23 |
|  | E-UTRA and, FR1 if configured | non-NR RAT Note1,2 and FR1 | 0, 1, 2, 3, 4, 6, 7, 8,10 |
|  | FR2 if configured |  | No gap |
|  | E-UTRA and, FR1 if configured | FR1 and FR2 | 0-11 |
|  | FR2 if configured |  | 12-23 |
|  | E-UTRA and, FR1 if configured | non-NR RAT Note1,2 and FR2 | 0, 1, 2, 3, 4, 6, 7, 8,10 |
|  | FR2 if configured |  | 12-23 |
|  | E-UTRA and, FR1 if configured | non-NR RAT Note1,2 and FR1 and FR2 | 0, 1, 2, 3, 4, 6, 7, 8,10 |
|  | FR2 if configured |  | 12-23 |
| Note: In E-UTRA-NR dual connectivity mode, if GSM or UTRA TDD or UTRA FDD inter-RAT frequency layer is configured to be monitered, only measurement gap pattern #0 and #1 can be used for per-FR gap in E-UTRA and FR1 if configured, or for per-UE gap. NOTE 1: In E-UTRA-NR dual connectivity mode, non-NR RAT includes E-UTRA, UTRA and/or GSM. In NR-E-UTRA dual connectivity mode, non-NR RAT means E-UTRA. NOTE 2: In E-UTRA-NR dual connectivity mode, the gap patterns with short MGL (gap pattern #2, 3, 6, 7, 8, 10) are supported by UEs which support shortMeasurementGap-r14 and indicate the capability signalling of measGapPatterns to network. In NR-E-UTRA dual connectivity mode, the measurement gap pattern #2, 3, 6, 7, 8, 10 are supported by the UEs which indicate the capability signalling of supportedGapPattern to network. NOTE 3: When E-UTRA inter-frequency RSTD measurements are configured and the UE requires measurement gaps for performing such measurements, only Gap Pattern #0 can be used. |  |  |  |

2. Actions:

To RAN WG2 group.

ACTION: 	RAN4 kindly asks RAN2 to take the above information into account in their future work.

3. Date of Next TSG-RAN WG4 Meetings:

TSG-RAN4 #91			13th – 17th May 2019   Reno, Nevada, US

TSG-RAN4 #92			26th – 30th Aug 2019   Ljubljana, SI