3GPP TSG-RAN4 Meeting #90bis	R4-1903592

# Xi’an, China, 8th – 12th  April, 2019

Agenda item:	          	6.10.8.3

Source:		MediaTek Inc.

Title:		Discussion on L1-RSRP measurement accuracy

Document for:	Discussion

- Introduction

In this paper, the link level simulation results for the measurement accuracy of L1-RSRP reporting are updated based on [1], and the requirements for measurement accuracy of L1-RSRP reporting are proposed.

- L1-RSRP measurement accuracy

The simulation results of measurement accuracy of SSB and CSI-RS are shown in the Table 1 and Table 2 respectively.

Table 1: SSB L1-RSRP absolute accuracy (dB) @ SNR=-3dB, 1 sample

| Channel | STATIC/AWGN |  |  |  | TDL-C 300ns 100Hz (FR1) |  | TDL-A 30ns 75Hz (FR2) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCS | 15 kHz | 30 kHz | 120 kHz | 240 kHz | 15 kHz | 30 kHz | 120 kHz | 240 kHz |
| Accuracy (dB) | 1.40 | 1.38 | 1.49 | 1.29 | 1.65 | 1.84 | 2.10 | 2.10 |

Table 2: CSI-RS L1-RSRP absolute accuracy (dB) @ SNR=-3dB, 1 sample

| BW (PRBs) | STATIC/AWGN |  |  | TDL-C 300ns 100Hz (FR1) |  | TDL-A 30ns 75Hz (FR2) |
| --- | --- | --- | --- | --- | --- | --- |
|  | 15kHz | 30kHz | 120kHz | 15 kHz | 30 kHz | 120 kHz |
| 24 | 2.13 | 2.16 | 2.14 | 2.68 | 2.66 | 2.98 |
| 48 | 1.64 | 1.62 | 1.66 | 1.98 | 1.94 | 2.19 |
| 96 | 1.26 | 1.25 | 1.26 | 1.46 | 1.42 | 1.54 |

From Table 1, for 1 sample SSB measurement at SNR of -3dB, the measurement accuracy about 2 dB can be achieved. Considering RF margin and implementation margin, 5 dB accuracy is proposed in FR1. In FR2, extra 1.5 dB is assumed as SS-RSRP measurement.

Proposal 1: For SSB L1-RSRP for reporting, the absolute measurement accuracy is +/- 5.0 dB for FR1; +/- 6.5 dB for FR2.

From Table 2, for 1 sample CSI-RS measurement at SNR of -3dB, the measurement accuracy about 2 dB can be achieved if the measurement bandwidth is increased to 48 PRBs. So it proposes to achieve similar measurement accuracy as SSB and to avoid inaccurate measurement.

Observation 1: Measurement accuracy can be improved with measurement bandwidth of 48 PRBs.

Proposal 2: For CSI-RS L1-RSRP for reporting, the measurement accuracy is based on 48 PRBs.

Similar to SSB L1-RSRP for reporting, considering RF margin and implementation margin, 5 dB accuracy is proposed in FR1. In FR2, extra 1.5 dB is assumed as SS-RSRP measurement.

Proposal 3: For CSI-RS L1-RSRP for reporting with 48 PRBs, the absolute measurement accuracy is +/- 5.0 dB for FR1; +/- 6.5 dB for FR2.

- Conclusion

The link level simulation result for L1-RSRP measurement accuracy has been updated in this paper. We have the following observations and proposals:

Proposal 1: For SSB L1-RSRP for reporting, the absolute measurement accuracy is +/- 5.0 dB for FR1; +/- 6.5 dB for FR2.

Observation 1: Measurement accuracy can be improved with measurement bandwidth of 48 PRBs.

Proposal 2: For CSI-RS L1-RSRP for reporting, the measurement accuracy is based on 48 PRBs.

Proposal 3: For CSI-RS L1-RSRP for reporting with 48 PRBs, the absolute measurement accuracy is +/- 5.0 dB for FR1; +/- 6.5 dB for FR2.

- Reference

- R4-1902565, Way forward on L1 RSRP accuracy, Ericsson, MediaTek, Nokia