3GPP TSG-RAN WG4 Meeting#90bis                                                      R4-1903974

Xi’an, China, 8th April– 12th April, 2019

Agenda item:	6.8.5.3.1.2

Source: 	ZTE Corporation

Title: 	Further discussion on TS38.141-02 Annex F for EVM measurement

Document for:	Discussion

Introduction

In the last RAN4#90 meeting, there are some initial discussion on the Annex F for EVM measurement in the TS38.141-02, however more detailed parameters should be updated with further considerations from NR system, therefore in this contribution, we want to share some further considerations on Annex F.

Discussion

Annex F (normative):
Global In-channel Tx tests

F.1	General

Editor’s note: Placeholder for the remaining elements of the “Global in-channel Tx test”, e.g. reuse of the E-UTRA annex for frequency error measurement description which is performed together with the EVM.

The Global in-channel Tx test enables the measurement of all relevant parameters that describe the in-channel quality of the output signal of the TX under test in a single measurement process.

The parameters describing the in-channel quality of a transmitter, however, are not necessarily independent. The algorithm chosen for description inside this annex places particular emphasis on the exclusion of all interdependencies among the parameters.

F.2.1	Basic principles

The process is based on the comparison of the actual output signal of the TX under test, received by an ideal receiver, with a reference signal, that is generated by the measuring equipment and represents an ideal error free received signal. All signals are represented as equivalent (generally complex) baseband signals.

The description below uses numbers and illustrations as examples. These numbers are taken from frame structure 1 with normal CP length and a transmission bandwidth configuration of [NRB = 100]. The application of the text below, however, is not restricted to this parameterset.

ZTE:  frame structure 1/2/3  is only defined in  TS36.211 spec and it’s not used in TS38.211, suggest to remove that frame structure 1 in the Annex.

NRB=100 is based on 20MHz LTE system. For FR2 NR system, 400MHz with SCS 120KHz (NRB=264) is proposed as example case especially for considering band 257/258/260/261 where 400MHz is more typical deployment scenario.

F.2.2 Output signal of the TX under test

The output signal of the TX under test is acquired by the measuring equipment and stored for further processsing. It is sampled at a sampling rate of [30.72] Msps and it is named z(ν). In the time domain it comprises at least 1 frame: z(ν). It is modelled as a signal with the following parameters: demodulated data content, carrier frequency, amplitude and phase for each subcarrier.

ZTE:  30.72Msps is based on the FFT size 2048 and SCS 15KHz defined for LTE system, however for the NR system 400MHz with 120KHz SCS, the sampling rate for FR2 should be 491.52Msps (120KHz*4096)for 400MHz with 120KHz SCS where more than 2048 size FFT is needed.

F.2.3 Reference signal

Two types of reference signal are defined:

The reference signal i1(ν) is constructed by the measuring equipment according to the relevant TX specifications, using the following parameters: demodulated data content, nominal carrier frequency,  nominal amplitude and phase for each subcarrier.  It is represented as a sequence of samples at a sampling rate of [30.72] Msps in the time domain. The structure of the signal is described in the testmodells.

The reference signal i2(ν) is constructed by the measuring equipment according to the relevant TX specifications, using the following parameters: restricted data content: nominal Reference Symbols and the Primary Synchronisation Channel, (all other modulation symbols are set to 0 V), nominal carrier frequency,  nominal amplitude and phase for each applicable subcarrier, nominal timing.  It is represented as a sequence of samples at a sampling rate of [30.72] Msps in the time domain.

ZTE: the same story as before, the sampling rate 30.72Msps should be change to be 491.52Msps.

Primary synchronization channel should be removed at least as this is not defined in the test model.

F.2.4 Measurement results

The measurement results, achieved by the global in channel TX test are the following:

-	Carrier Frequency error

-	EVM (Error Vector Magnitude)

-	Resource Element TX power

-	RS TX power (RSTP)

-	OFDM Symbol TX power (OSTP)

Other side results are: residual amplitude- and phase response of the TX chain after equalisation.

ZTE: the RS TX power (RSTP) might be not needed anymore for NR spec as there are no DL RS power accuracy requirement defined for NR spec.

F.2.5 Measurement points

The Resource element TX power is measured after the FFT as described in figure F.2.5-1. The EVM shall be measured at the point after the FFT and a zero-forcing (ZF) equalizer in the receiver, as depicted in for FR1 in figure F.2.5-1 and for FR2 in figure F.2.5-2.

Figure F.2.5-1: Reference point for FR1 EVM measurement

Figure F.2.5-2: Reference point for FR2 EVM measurement

F.3	Void

F.3.1 Pre FFT minimization process

Sample Timing, Carrier Frequency in z(ν) are varied in order to minimise the difference between z(ν) and i1 (ν) , after the amplitude ratio of z(ν) and i1 (ν) has been scaled. Best fit (minimum difference) is achieved when the RMS difference value between z(ν) and i(ν) is an absolute minimum.

The carrier frequency variation  is  the measurement result: Carrier Frequency Error.

From the acquired samples one carrier frequency error can be derived.

Note 1:	The minimisation process, to derive the RF error can be supported by Post FFT operations. However the minimisation process defined in the pre FFT domain comprises all acquired samples (i.e. it does not exclude the samples inbetween the FFT widths and it does not exclude the bandwidth outside the transmission bandwidth configuration.

Note 2:	The algorithm would allow to derive Carrier Frequency error and Sample Frequency error of the TX under test separately. However there are no requirements for Sample Frequeny error. Hence the algorithm models the RF and the sample frequency commonly (not independently). It returns one error and does not distinuish between both.

After this process the samples z(ν) are called z0(ν).

ZTE: some editorial corrections are needed. e.g  minimisation or Frequeny or inbetween or distinuish

F.3.2 Timing of the FFT window

The FFT window length is [2048] samples per OFDM symbol. [140] FFTs ([286720] samples) cover less than the acquired number of samples ([307200] samples in 10 subframes) The position in time for FFT must be determined.

ZTE:  the FFT window length is 4096 samples per OFDM symbol. 1120 FFTs (4587520 samples) over less than the acquired number of samples (4915200 samples in 10 subfames).

In an ideal signal, the FFT may start at any instant within the cyclic prefix without causing an error. The TX filter, however, reduces the window. The EVM requirements shall be met within a window W<CP. There are three different instants for FFT:

Centre of the reduced window, called , ΔC –W/2 and ΔC +W/2,

The BS shall transmit a signal according to the Test models, intended for EVM. The primary synchronisation signal and the reference signal shall be used to find the centre of the FFT window.

ZTE: the primary synchronization signal should be removed as this is not defined in the test model.

The timing of the measured signal is determined in the pre FFT domain as follows, using z0(ν) and i2(ν) :

1.	The measured signal is delay spread by the TX filter. Hence the distinct boarders between the OFDM symbols and between Data and CP are also spread and the timing is not obvious.

2.	In the Reference Signal i2(ν) the timing is known.

Correlation between (1.) and (2.) will result in a correlation peak. The meaning of the correlation peak is approx. the “impulse response” of the TX filter.

3.	The meaning of “impulse response” assumes that the autocorrelation of the reference signal i2(ν)  is a Dirac peak and that the correlation between the reference signal i2(ν)  and the data in the measured signal is 0. The correlation peak, (the highest, or in case of more than one highest, the earliest) indicates the timing in the measured signal.

The number of samples, used for FFT is reduced compared to z0(ν). This subset of  samples is called z’(ν).

From the acquired samples one timing can be derived.

The timing of the centre  with respect to the different CP length in a slot is as follows: (Frame structure 1, normal CP length)

[ is on Tf=72 within  the CP of length 144  (in OFDM symbol 1 to 6)]

[ is on Tf=88 (=160-72)  within  the CP of length 160 (in OFDM symbol 0)]

ZTE: As mentioned at the beginning, frame structure 1/2/3 is only applied for LTE spec, propose to remove that item from the Annex.

For the configurations here assuming  sampling rate 491.52Msps, therefore is 3 where is updated not like for Tc (1/480KHz*4096).

is on Tf=144 () within  the CP of length 288 ()  (in OFDM symbol    or  )

is on Tf=208 () within  the CP of length 352 ()  (in OFDM symbol   or  ).

Throughout this specification, unless otherwise noted, the size of various fields in the time domain is expressed as a number of time units where Hz and . The constant where , and . %-------------------------------------------------------------------------------------------------------% The starting position of OFDM symbol for subcarrier spacing configuration in a subframe is given by where

F.3.3	Resource Element TX power

Perform FFT (z’(ν))  with the FFT window timing

The result is called Z’(t,f). The RE TX power is then defined as:

ZTE:  the above formula should be revised, NR numerology includes 15KHz, 30KHz, 60KHz and revised as following:

From this the Reference Signal Transmit power (RSTP) is derives as follows:

,

It is an average power and accumulates the powers of the reference symbols within a sub frame divided by n, the number of reference symbols within a sub frame.

ZTE:  as mentioned before, this formula should be removed, as DL RS power accuracy requirement has been removed from TS 38.104 spec.

From RETP the OFDM Symbol TX power (OSTP) is derived as follows:

It accumulates all sub carrier powers of the 4th OFDM symbol. The 4th (out of 14  OFDM symbols within a subframe (in case of frame type 1 , normal CP length)) contains exclusively PDSCH.

From the acquired samples 10 values for each RSTP and OSTP can be derived

ZTE:  The OFDM symbol TX power is required for testing Total power dynamic range. The upper limit of dynamic range requirement is the OFDM symbol power for BS at max power configured with NR-FR1-TM3.1. The lower limit of the dynamic range is the OFDM symbol power for BS when one 64QAM resource block is transmitted as defined by NR-FR1-TM2. OFDM symbol shall carry PDSCH and not contain RS, PBCH or synchronization signals. The 4th OFDM symbol in each slot fullfils this condition and it could be kept unchanged.

Some revision is needed as following:

It accumulates all sub carrier powers of the 4th OFDM symbol. The 4th (out of 14  OFDM symbols within a subframe slot (in case of frame type 1 , normal CP length)) contains exclusively PDSCH.

From the acquired samples 10 values for each RSTP and OSTP can be derived.

F.4	Void

F.5	Void

F.6	Void

F.7	Post FFT equalisation

Perform [140] FFTs on z’(ν), one for each OFDM symbol comprising the full frame with the FFT window timing . (in case of frame type 1 , normal CP length) The result is an array of samples, 140 in the time axis t times [2048] in the frequency axis f.

ZTE: Perform 1120 FFTs on z’(ν), one for each OFDM symbol comprising the full frame with the FFT window timing . (in case of  normal CP length) The result is an array of samples, 1120 in the time axis t times 4096 in the frequency axis f.

The equalizer coefficients and  are determined as follows:

1.	Calculate the complex ratios (amplitude and phase) of the post-FFT acquired signal  and the post-FFT Ideal signal , for each reference symbol, over [10 subframes]. This process creates a set of complex ratios:

Where the post-FFT Ideal signal  is constructed by the measuring equipment according to the relevant TX specifications, using the following parameters:

- For FR1: restricted content: i.e. nominal Reference Symbols and the Primary Synchronisation Channel, (all other modulation symbols are set to 0 V), nominal carrier frequency,  nominal amplitude and phase for each applicable subcarrier, nominal timing.

ZTE: Primary Synchronisation Channel should be removed.

i.e. nominal Reference Symbols and the Primary Synchronisation Channel, (all other modulation symbols are set to 0 V), nominal carrier frequency,  nominal amplitude and phase for each applicable subcarrier, nominal timing.

- For FR2: restricted content: i.e. nominal Demodulation Reference.

2.	Perform time averaging at each reference signal subcarrier of the complex ratios, the time-averaging length is [10 subframes]. Prior to the averaging of the phases  an unwrap operation must be performed according to the following definition: The unwrap operation corrects the radian phase angles of  by adding multiples of 2*PI when absolute phase jumps between consecutive time instances ti are greater then or equal to the jump tolerance of PI radians. This process creates an average amplitude and phase for each reference signal subcarrier (i.e. every second subcarrier with the exception of the reference subcarrier spacing across the DC subcarrier).

Where N is the number of reference symbol time-domain locations ti from Z’(f,t) for each reference signal subcarrier .

ZTE: editorial corrections are needed for greater then  than.

In addition, as the DC carrier are allowed to transmit the Data and reference signals, therefore DC carrier is not exceptional case anymore.

with the exception of the reference subcarrier spacing across the DC subcarrier

- The equalizer coefficients for amplitude and phase  and  at the reference signal subcarriers are obtained by computing the moving average in the frequency domain of the time-averaged reference signal subcarriers, i.e. every second subcarrier. The moving average window size is 19. For reference subcarriers at or near the edge of the channel the window size is reduced accordingly as per figure F.7-1.

4.	Perform linear interpolation from the equalizer coefficients  and  to compute coefficients ,  for each subcarrier.

Figure F.7-1: Reference subcarrier smoothing in the frequency domain

a)	In case of FR2 EVM, to account for the common phase error (CPE) experienced in millimetre wave frequencies, φ(f), in the estimated coefficients contain phase rotation due to the CPE, θ, in addition to the phase of the equalizer coefficient φf, that is

φf=φf+θ(t)

For OFDM symbols where PT-RS does not exist, θ(t) can be estimated by performing linear interpolation from neighboring symbols where PT-RS is present.

In order to separate component of the CPE, θ, contained in, φ(f), estimation and compensation of the CPE needs to follow. θ(t) is the common phase error (CPE), that rotates all the subcarriers of the OFDM symbol at time t.

Estimate of the CPE, θ(t), at OFDM symbol time, t, can then be obtained from using the PT-RS employing the expression

θ(t)=argf∈fptrsZ't,fIptrst,f afe-jφf

In the above equation, fptrs is the set of subcarriers where PT-RS are mapped, t∈tptrs where  tptrs is the set of OFDM symbols where PT-RS are mapped while Z't,f and Iptrst,f are is the post-FFT acquired signal and the ideal PT-RS signal respectively. That is, estimate of the CPE at a given OFDM symbol is obtained from frequency correlation of the complex ratios at the PT-RS positions with the conjugate of the estimated equalizer complex coefficients. The estimated CPE can be subtracted from φf to remove influence of the CPE, and obtain estimate of the complex coefficient’s phase

φf=φf-θ(t)

F.7.1 EVM

For EVM create two sets of Z’eq(f,t)., according to the timing ”ΔC –W/2 and ΔC +W/2”, using the equalizer coefficients from F.3.4.

The equivalent ideal samples are calculated form  i1(ν) (clause F.2.3) and are called I(f,t).

The EVM is the difference between the ideal waveform and the measured and equalized waveform.

,

where

is the set of symbols with the considered modulation scheme being active within the subframe or within the sTTI,

ZTE: is the set of symbols with the considered modulation scheme being active within the slot subframe or within the sTTI,

is the set of subcarriers within the  resource blocks with the considered modulation scheme being active in symbol t,

is the ideal signal reconstructed by the measurement equipment in accordance with relevant Test models,

is the equalized signal under test.

Note1:	Although the basic unit of measurement is one subframe or one sTTI, the equalizer is calculated over the entire 10 subframes measurement period to reduce the impact of noise in the reference symbols.

Note 2:	Applicability of EVM calculation:

One EVM value is associated to 12 subcarriers times 1 subframe = pair of 2 RBs = 168 resource elements. For sTTI, one EVM value is associated to 12 subcarriers times 1 slot or 84 resource elements (slot TTI) or times 1 subslot or 24/36 resrouce elements (subslot TTI).

ZTE:

For LTE system, one RB consists of  resource elements, corresponding to one slot in the time domain and 180 kHz in the frequency domain where  is 7 and is 12 for normal CP.

For NR system, one RB is defined as  consecutive subcarriers in the frequency domain.

In addition, for sTTI, this is not defined for NR system, propose to remove that description from the Annex F.

Based on the above considerations, some corrections are needed as following:

One EVM value is associated to 12 subcarriers times 1 slot subframe = pair of 2 RBs = 168 resource elements. For sTTI, one EVM value is associated to 12 subcarriers times 1 slot or 84 resource elements (slot TTI) or times 1 subslot or 24/36 resrouce elements (subslot TTI).

But only a reduced number of REs in this pair of 2 RBs contribute to EVM. Those are the PDSCH or sPDSCH REs, containing the considered modulation scheme. Only those pairs of 2 RBs are evaluated with respect to EVM, which contain the maximum number of PDSCH or sPDSCH REs. (EVM-relevant location in the time/frequency grid ) The others are not evaluated.

ZTE: as this NR system is not scheduled based on the subframe level (the pair of 2 RBs). In addition, for flexible slots with enough OFDM symbols could also be used for EVM measurement as proposed in R4-1901553, however no conclusion were reached in the last RAN4 meeting. Some editorial corrections are needed.

Therefore some corrections are needed  as following:

But only a reduced number of REs in this slot pair of 2 RBs contribute to EVM. Those are the PDSCH or sPDSCH REs, containing the considered modulation scheme. [Only those slots pairs of 2 RBs are evaluated with respect to EVM, which contain the maximum number of PDSCH or sPDSCH REs. (EVM-relevant location in the time/frequency grid ) The others are not evaluated.]

In specific:

-	For all other Bandwidths:

-	Only the pairs of 2 RBs containing 150 PDSCH REs are used for EVM. Only those 150 REs contribute to EVM

-	All pairs of 2 RBs, which contain less than 150 PDSCH REs, are not evaluated with respect to EVM.

This restriction serves to avoid weighted averaging in F.8.

ZTE: for all other bandwidth, in fact, it’s assumed for larger than or equal to 5MHz with 1 PDCCH symbol and single CRS port, then the number of REs for PDSCH should be  168-8(CRS)-(12-2 (2 CRS )) .  However for NR system, as the TM for EVM testing is still under discussion, therefore we propose to update it once the TM is finalized.  Some general corrections are needed as following:

-	For all other Bandwidths:

-	Only the slot pairs of 2 RBs containing [150 ]PDSCH REs are used for EVM. Only those [150] REs contribute to EVM

-	All slots pairs of 2 RBs, which contain less than [150] PDSCH REs, are not evaluated with respect to EVM.

F.8	Averaged EVM

EVM is averaged over all allocated downlink resource blocks with the considered modulation scheme in the frequency domain, and a minimum of 10 downlink subframes:

For FDD the averaging in the time domain equals the [10] subframe duration of the [10] subframes measurement period from the equalizer estimation step.

For TDD the averaging in the time domain can be calculated from subframes of different frames and should have a minimum of [10] subframes averaging length. TDD special fields (i.e. GP) are not included in the averaging.

Where Ni is the number of resource blocks with the considered modulation scheme in subframe i and Ndl is the number of allocated downlink subframes in one frame.

ZTE:  remove the square bracket for 10 subframes average lenght. TDD special field is used in the TDD LTE system as it’s indicated in TS36.211 that specical subframe includes three special fields (DwPTS, GP, UpPTS). In addition,as mentioned, slots with enough DL OFDM symbols could also be used for EVM measurement,  however no conclusion were reached in the last RAN4 meeting. Some editorial corrections are needed.

Where Ni is the number of resource blocks with the considered modulation scheme in slot subframe i and Ndl is the number of [allocated downlink slots]subframes in one frame.

The EVM requirements shall be tested against the maximum of the RMS average at the window W extremities of the EVM measurements:

Thus   is calculated using in the expressions above and is calculated using  in the  calculation.

Thus we get:

The averaged EVM with the minimum averaging length of at least [10] subframes is then achieved by further averaging of the  results

,

ZTE: some editorial corrections are needed as following:

The averaged EVM with the minimum averaging length of at least [10] subframes is then achieved by further averaging of the  results

,

For FDD LTE, for Ndl is 10 subframes per frame, then Nframe=ceil(10/Ndl)=1

For TDD LTE without in-band NB-IoT and guardband NB-IoT operation, TDD configuration for test model is configuration 3. Then Ndl is 6 subframes per frame, then Nframe=ceil(10/Ndl)=2.

Table 6.1.1-1: Configurations of TDD eNB test models

| TDD Configuration | Downlink-to-Uplink Switch-point periodicity | Number of UL/DL sub-frames per radio frame (10 ms) |  | DwPTS | GP | UpPTS |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | DL | UL |  |  |  |
| 1 | 5 ms | 4 | 4 |  |  |  |
| 3 | 10ms | 6 | 3 |  |  |  |

For NR system, for SCS 60KHz case,  Nframe=ceil(10/Ndl)=ceil(10/(3*8*0.25))=2

For NR system, for SCS 120KHz case, Nframe=ceil(10/Ndl)=ceil(10/(7*8*0.125))=2

Where as TDD periodicity for NR system is specified as 1.25ms, then 8 times of downlink slots should be considered, meanwhile with the increasing SCS,  time duration per slot should also be scaled correspondingly.

Table 4.9.2.2-1: Configurations of TDD for BS type 2-O test models

| Field name | Value |  |
| --- | --- | --- |
| referenceSubcarrierSpacing (kHz) | 60 | 120 |
| Periodicity (ms) for dl-UL-TransmissionPeriodicity | 1.25 | 1.25 |
| nrofDownlinkSlots | 3 | 7 |
| nrofDownlinkSymbols | 10 | 6 |
| nrofUplinkSlots | 1 | 2 |
| nrofUplinkSymbols | 2 | 4 |

Based on the above considerations, the formula for Nframe should be revised accordingly:

F.8.1 Averaged EVM (TDD)

For TDD the averaging in the time domain can be calculated from subframes or slots (for slot TTI) of different frames and should have a minimum of 10 subframes averaging length. TDD special fields are not included in the averaging.

ZTE:  similar comments as before, corrections on the sTTI and TDD special filed are needed.

For TDD the averaging in the time domain can be calculated from subframes or slots (for slot TTI) of different frames and should have a minimum of  10 subframes averaging length. TDD special fields  (DwPTS and GP) are not included in the averaging.

frame is derived by: Square the EVM results in a frame. Relevant for EVM are subframes or slots (for slot TTI) in a frame, which are active in the DL, Ndl. Within these subframes or slots, those RBs are relevant, that carry the maximum number of PDSCH REs (same as FDD) or sPDSCH REs (slot TTI). Sum the squares, divide the sum by the number of EVM relevant locations, square-root the quotient. (RMS)

ZTE: similar comments as before.

frame is derived by: Square the EVM results in a frame. Relevant for EVM are subframes or slots (for slot TTI) in a frame, which are active in the DL, Ndl. Within these subframes or slots, [ those RBs are relevant, that carry the maximum number of PDSCH REs (same as FDD) or sPDSCH REs (slot TTI) ]. Sum the squares, divide the sum by the number of EVM relevant locations, square-root the quotient. (RMS)

The EVMframe is calculated, using the maximum of  frame at the window W extremities. Thus   is calculated using  and is calculated using . (l and h, low and high. Where l is the timing ΔC –W/2 and and high is the timing ΔC +W/2)

In order to unite at least 10 subframes, consider the minimum integer number of radio frames, containing at least 10 EVM relevant subframes or 20 l. Unite by RMS.

,

The result, , is compared against the limit.

ZTE: similar considerations before, the formula for Nframe should be revised accordingly:

In order to unite at least 10 subframes, consider the minimum integer number of radio frames, containing at least 10 EVM relevant subframes or 20 l. Unite by RMS.

,

The result, , is compared against the limit.

<End of modified section>

References.

- R4-1901340 draft CR to 38.141-2 additions to Annex F, endorsed.