3GPP TSG RAN WG4 Meeting #90bis	R4-1904316

Xian, China, 8th– 12th April, 2019

Title:	Draft Response LS on UL sharing applicability in different scenarios

Response to:	-

Release:	Release 15

Work Item:	NR_newRAT-Core

Source:	Nokia  (TSG RAN WG4)

To:	TSG RAN WG2

Cc:	TSG RAN WG1

Contact Person:

#### Name:	Toni Lähteensuo

E-mail Address:	toni.h.lahteensuo@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN4 has discussed RAN2’s questions on UE capabilities for UL and DL E-UTRA – NR carrier sharing and intra-band EN-DC E-UTRA – NR switching times. RAN4 would like to provide the following answers to the questions addressed to RAN4.

- Is the shared E-UTRA-NR FDD carrier supported in RAN4, i.e. the case where both FDD DL carrier and FDD UL carrier are shared from UE perspective by E-UTRA and NR?

Answer: Yes, RAN4 has defined UL carrier sharing including UL sharing from the UE perspective both for SUL and FDD bands. DL carrier sharing is introduced by RAN1 related UE features and capabilities and thus, transparent in the RAN4 specifications.

- Does the UL sharing from UE perspective need to be supported together with any DL sharing capabilities that are required the shared E-UTRA-NR FDD carrier case?

Answer: Yes, for enabling EN-DC operations for the shared DL and UL E-UTRA - NR carriers with E-UTRA and NR FDD bands the UE needs to support both UL sharing from the UE perspective and DL sharing. This can be achieved by UE indicating ul-SharingEUTRA-NR for UL sharing and e.g. rateMatchingLTE-CRS for DL sharing.

- How should RAN2 refer to RAN4 specification to indicate conditions when UE is required to support ul-SwitchingTimeEUTRA-NR? Specifically, does the ul-SwitchingTimeEUTRA-NR apply to both TDM based E-UTRA-NR in UL sharing from UE perspective and intra-band contiguous scenario in adjacent channels when single UL transmission is allowed between E-UTRA and NR UL?

Answer: E-UTRA and NR switching time mask requirements are defined in the sub-sections of  Section 6.3 ‘Output power dynamics’ of TS38.101-3. The following three different types of E-UTRA and NR switching time mask requirements are defined in the March 2019 version of TS38.101-3;

6.3B.1	Output power dynamics for EN-DC with UL sharing from UE perspective

6.3B.2	Output power dynamics for intra-band EN-DC without dual PA capability

6.3B.3	Output power dynamics for intra-band EN-DC with dual PA capability

- Is it mandatory for UE to report switching time type for an intra-band contiguous EN-DC band combination via the capability ul-SwitchingTimeEUTRA-NR if UE requires single UL transmission for that band combination?

Answer: In the current version of TS38.101-3 only E-UTRA and NR switching time mask requirements for TDM based UL sharing from the UE perspective are using switching time capabilities; Type 1 and Type 2 for differentiating different UE E-UTRA and NR switching time performances. Different intra-band EN-DC E-UTRA and NR switching time mask requirements are differentiated using dual PA capability. RAN4 would, however, like to note from the backwards compatibility perspective that in the December 2018 version of TS38.101-3 UE E-UTRA – NR switching time capability Types 1 and 2 were also used for differentiating different intra-band EN-DC E-UTRA and NR switching time mask requirements.

2. Actions:

To RAN2 group.

ACTION: 	RAN4 respectfully asks RAN2 to take the above-mentioned answers into account in its specification work

3. Date of Next TSG-RAN WG4 Meetings:

3GPP RAN4#91  13 - 17 May 2019     Reno, Nevada   US

3GPP RAN4#92  26 - 30 Aug 2019     Ljubljana   SI