3GPP TSG-RAN WG4 Meeting #90bis	R4-1904562

Xi’an, China, 8th April– 12th April, 2019

Source: 	Huawei, HiSilicon

Title: 	MSD analysis for CA_n28_n50

Agenda Item:	9.2.2

Document for:	Approval

# Introduction

In the case of 2 UL, this contribution provides the MSD analysis for CA_n28-n50. The co-existence analysis for CA_n28-n50 has been provided in the contribution.

- 2nd order harmonic generated by n28 may fall into n50.

- 2nd order IMD generated by dual uplink of Band n28 and n50 may fall into Band n28.

- 4th order IMD generated by dual uplink of Band n28 and n50 may fall into Band n28.

- 5th order IMD generated by dual uplink of Band n28 and n50 may fall into Band n28.

We can also note that in the case of single UL:

- There is no interference in the case of the single n50 UL,

- There is only an interference made by “2nd order harmonic generated by n28 may fall into n50” in the case of single n28 UL.

# Discussion

## UE schema

## As listed in the introduction, there are several interferences made by the harmonic and IMDs. Thus, it is proposed the schema for the UE radio:

Figure 2.1-1 main/diversity antenna topology

## Harmonic issue

A diplexer is adopted to combine the two bands, i.e. diplexer + duplexer architecture is used to estimate the MSD due to 2nd order harmonic. Table 2.2-1 show the calculation for harmonic interference.

Table 2.2-1 MSD calculation for harmonic interference

|  | primary |  | diversity |  |
| --- | --- | --- | --- | --- |
| Parameter | value | H2 level | value | H2 level |
| B28 PA output power, dBm | 27 |  |  |  |
| B28 Duplexer IL, dB | 2.5 |  |  |  |
| antenna switch IL, dB | 0.7 |  |  |  |
| Diplexer IL,dB | 0.8 |  |  |  |
| B28 PA H2, Attenuation,dBc | 40 |  |  |  |
| B28 PA H2, dBm |  | -13 |  |  |
| B28 Duplexer Rejection,dB | 48 |  |  |  |
| B28 H2 output at duplexer,dBm |  | -61 |  |  |
| B28 H2 output at antenna switch,dBm |  | -61.7 |  |  |
| Diplexer Rejection&isolation,dB | 15 | -76.7 |  |  |
| Antenna isolation, dB |  |  | 15 | -91.7 |
| antenna switch IL, dB | 0.7 | -77.4 | 0.7 | -92.4 |
| B50 filter IL, dB | 2 | -79.4 | 2 | -94.4 |
| PA to LNA PCB isolation, dB | 70 | -83 | 70 | -83 |
| Single Chip DA to LNA,dBm | -90 | -90 | -90 | -90 |
| Composite H3 at LNA port,dBm |  | -77.5712556 |  | -81.95523 |
| Chip Noise Figure, dB | 4 |  |  |  |
| Noise Floor(10M), dBm |  | -100 |  | -100 |
| Composite noise |  | -77.5465001 |  | -81.88764 |
| MSD, dB |  | 22.45349988 |  | 18.112364 |
| CBW,MHz | 10 |  |  |  |
| Combined MSD, dB | 19.7617 |  |  |  |

Reference sensitivity exceptions for CA_n28-n50 are proposed in table 2.2-2 with uplink configuration specified in Table 2.2-3.

Table 2.2-2: Reference sensitivity exceptions due to UL harmonic for NR CA FR1

| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n28 | n501,2 |  | 19.8 | 18.0 | 16.8 |  |  | 13.8 | 12.8 | 12.0 | 10.8 |  |  |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 2: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 2.2-3: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for NR CA, FR1

| NR Band / Channel bandwidth of the high band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| n28 | n50 |  | 25 | 25 | 25 |  |  |  |  |  |  |  |  |
| NOTE 1: 15 kHz SCS is assumed for UL band. NOTE 2: The UL configuration applies regardless of the channel bandwidth of the low band unless the UL resource blocks exceed that specified in Table 7.3.2-3 for the uplink bandwidth in which case the allocation according to Table 7.3.2-3 applies. |  |  |  |  |  |  |  |  |  |  |  |  |  |

## IMD issue

This inter-band NR CA combination is subjected to the 2nd, 4th and 5th order IM issues. The frequency points related to the MSD test conditions are summarized as the following table.

Table 2.3-1 MSD test conditions for CA_n28-n50

| NR CA | UL CA | IMD |  | UL Fc [MHz] | UL BW [MHz] | UL [RB] | DL Band | IMD center [MHz] | DL BW [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n28-n50 | n28 | IMD2 | ¦fn28 –fn50¦ | 730 | 10 | 50 | n28 | 770 | 10 |
|  | n50 |  |  | 1500 | 10 | 50 |  |  |  |
| CA_n28-n50 | n28 | IMD4 | ¦3* fn28 - fn50¦ | 740 | 10 | 50 | n28 | 770 | 10 |
|  | n50 |  |  | 1450 | 10 | 50 |  |  |  |
| CA_n28-n50 | n28 | IMD5 | ¦3* fn28 – 2*fn50¦ | 740 | 10 | 50 | n28 | 780 | 10 |
|  | n50 |  |  | 1500 | 10 | 50 |  |  |  |

We use main/diversity antenna topology as RF front-end architecture in our analysis, as shown in figure 2.1-1. The component linearity assumptions are listed in table 2.3-2, and the component attenuation and isolation values are listed in table 2.3-3.

Table 2.3-2 General linearity parameters

| Component | IP2(dBm) | IP4(dBm) | IP5(dBm) |
| --- | --- | --- | --- |
| Antenna switch | 112 | 56 | 53 |
| Diplexer | 115 | 55 | 53 |
| Duplexer | 110 | 55 | 53 |
| PA forward mixing | 28 | 30 | 28 |
| PA reverse mixing | 40 | 30 | 30 |
| LNA | 5 | -6 | -10 |

Table 2.3-3 Attenuation and isolation values

| Attenuation and Isolation Parameter | Value (dB) | Comment |
| --- | --- | --- |
| Antenna ISO | 15 | Main antenna to diversity antenna |
| L-M Diplexer IL | 0.8 |  |
| Antenna switch IL | 0.7 |  |
| PA gain | 25 |  |
| PCB isolation Paout-Pain | 70 | PCB isolation (PA forward mixing) |
| Duplexer IL | 2.5 |  |
| B50 filter IL | 2 |  |
| Diplexer isolation T1-T2 | 50 |  |

Table 2.3-4 and table 2.3-5 summarize the MSD analysis results for CA_n28-n50.

Table 2.3-4 MSD calculation for victim Rx of band n28

| Component | IMD2(dBm) |  | IMD4(dBm) |  | IMD5(dBm) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Main_@Antenna | Div_@Antenna | Main_@Antenna | Div_@Antenna | Main_@Antenna | Div_@Antenna |
| n28 PA Forward mixing interference | -72.3 |  | -85.8 |  | -134 |  |
| n50 PA Forward mixing interference | -91.7 |  | -206 |  | -204 |  |
| n28 PA Reserve mixing interference | -96.9 |  | -98.9 |  | -163 |  |
| n50 PA Reserve mixing interference | -123.1 |  | -266 |  | -272 |  |
| n28 Duplexer | -124.4 |  | -131.4 |  | -215.8 |  |
| n28 LNA via filters | -75.7 |  | -94.2 |  | -125.5 |  |
| n28 LNA via PCB coupling | -89.3 |  | -147.8 |  | -172.7 |  |
| Diplexer (L-M) | -73.4 | -104.2 | -81.8 | -149 | -108 | -186 |
| Antenna switch | -84.6 | -102.1 | -97.6 | -144 | -136.5 | -189 |
| interference From main path |  | -83.6 |  | -95.0 |  | -122.9 |
| Total | -68.6 | -83.3 | -80.2 | -95.0 | -107.9 | -122.5 |

Table 2.3-5 MSD requirements for CA_n28-n50

| NR CA | UL CA | IMD |  | UL Fc [MHz] | UL BW [MHz] | UL [RB] | DL Band | IMD center [MHz] | DL BW [MHz] | MSD[dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n28-n50 | n28 | IMD2 | ¦fn28 –fn50¦ | 730 | 10 | 50 | n28 | 770 | 10 | 15.3 |
|  | n50 |  |  | 1500 | 10 | 50 |  |  |  |  |
| CA_n28-n50 | n28 | IMD4 | ¦3* fn28 - fn50¦ | 740 | 10 | 50 | n28 | 770 | 10 | 6.0 |
|  | n50 |  |  | 1450 | 10 | 50 |  |  |  |  |
| CA_n28-n50 | n28 | IMD5 | ¦3* fn28 – 2*fn50¦ | 740 | 10 | 50 | n28 | 780 | 10 | 0.5 |
|  | n50 |  |  | 1500 | 10 | 50 |  |  |  |  |

# References

[1]	R4-1902132, “TP to TR 38.716-02-00: CA_n28A_n50A”, Huawei, Hisilicon, Etisalat