3GPP TSG-RAN WG4 Meeting#91	R4-1905401

Reno, US, 13th– 17th May, 2019

Agenda Item:	9.10.2

Source:	CATT

Title:	TP for 38.716-03-01: UE requirements for CA_n8A-n41A-n79A

Document for:	Approval

- Introduction

This contribution provides a text proposal on UE requirements for inter-band CA_n8A-n41A-n79A.

- Discussion

The followings are proposed for this combination

- ∆TIB,c and ∆RIB,c values are derived based those values for the 3 fall back modes. If the ∆TIB,c and ∆RIB,c values for a specific bands are difference between different combinations, the largest value will be used for the band in this combination.

- Sensitivity degradation is allowed for Band n41 and n79 when there is uplink transmission in Band n8.

- Conclusion

It is proposed to approve the text proposal provided in the annex.

- Annex

----- Start of TP -----

6.2	CA_n8A-n41A-n79A

- 6.2.1	Operating bands for CA

Table 6.2.1-1: 3DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n8-n41-n79 | n8 | 880MHz | – | 915MHz | 925MHz | – | 960MHz | FDD |
|  | n41 | 2496MHz | – | 2690MHz | 2496MHz | – | 2690MHz | TDD |
|  | n79 | 4400MHz | – | 5000MHz | 4400MHz | – | 5000MHz | TDD |

- 6.2.2	Channel bandwidths per operating band for CA

Table 6.2.2-1: Supported channel bandwidths per CA configuration for 3DL inter-band CA

| NR CA Configuration | UL Config | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 100 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n8A-n41A-n79A | - | n8 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
| CA_n3A-n41A-n79A | - | n8 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 1 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all single carrier bandwidths for the constituent bands as defined in Table 5.3.5-1 of  TS 38.101-1 [3] and in Table 5.3.5-1 of TS 38.101-2 when operating in single carrier mode.

- 6.2.3	Co-existence studies

Table 6.2.3-1 summarizes frequency ranges where harmonics occur due to 3DL bands CA with 1 UL. It can be seen that the 3rd order harmonic of Band n8 will fall into Band n41 and the 5th order harmonic of Band 8 will fall into band n79.

Table 6.2.3-1: Harmonic Interference for 3DLs/1UL

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th harmonic |  | 6th harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n8 | 880 | 915 | 925 | 960 | 1760 | 1830 | 2640 | 2745 | 3520 | 3660 | 4400 | 4575 | 5280 | 5490 |
| n41 | 2496 | 2690 | 2496 | 2690 | 4992 | 5380 | 7488 | 8070 |  |  |  |  |  |  |
| n79 | 4400 | 5000 | 4400 | 5000 | 8800 | 10000 | 1320 | 15000 |  |  |  |  |  |  |

Table 6.2.3-2 gives harmonic mixing issue for CA with Band n8, n41 and n79. It is seen that part of Band n79 Tx frequency coincides with Band 8x5.

Table 6.2.3-2 Harmonic mixing for 3DLs/1UL

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th harmonic |  | 6th harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n8 | 880 | 915 | 925 | 960 | 1850 | 1920 | 2775 | 2880 | 3700 | 3840 | 4625 | 4800 | 5550 | 5760 |
| n41 | 2496 | 2690 | 2496 | 2690 | 4992 | 5380 | 7488 | 8070 |  |  |  |  |  |  |
| n79 | 4400 | 5000 | 4400 | 5000 | 8800 | 10000 | 1320 | 15000 |  |  |  |  |  |  |

For single uplink, the UE coexistence is already considered for these bands in TS 38.101-1 [3].

- 6.2.4	∆TIB,c and ∆RIB,c values

For three simultaneous DLs and one UL of Band n8, n41 and n79, the TIB,c and RIB,c values are shown in table 6.2.4-1 and  table 6.2.4-2, respectively.

Table 6.2.4-1: ΔTIB,c for 3DL aggregation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n8A-n41A-n79A | n8 | 0.6 |
|  | n41 | 0.3 |
|  | n79 | 0.8 |

Table 6.2.4-2: ΔRIB,c for 3DL aggregation

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n8A-n41A-n79A | n8 | 0 |
|  | n41 | 0.5 |
|  | n79 | 0.5 |

- 6.2.5	REFSENS requirements

For this combination, sensitivity degradation is allowed for Band n41 and n79 when there is uplink transmission in Band n8. The reference sensitivity exceptions are specified in Table 6.2.5-1 with the uplink configuration specified in Table 6.2.5-2.

Table 6.2.5-1: MSD due to harmonic issue for CA_n8-n41-n79

|  | MSD due to harmonic exception for the DL band |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
|  |  | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB |
| n8 | n411,2 | N/A | 13 | 11.3 | 10.1 |  |  | 7.0 | 6.1 | 5.5 | 4.3 | 3.5 |
|  | n793,4 |  |  |  |  |  |  | [6.8] | 6.2 | [5.6] | 4.9 | 4.4 |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 2 The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 3: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 5th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 4: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.5-2 Uplink configuration for the low band (exceptions)

| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n8 | n41 | - | 16 | 25 | 25 | - | - | 25 | 25 | 25 | 25 | 25 |
|  | n79 | - | - | - | - | - | - | 25 | 25 | 25 | 25 | 25 |

----- Start of TP -----