3GPP TSG-RAN4 Meeting #91 	R4-1905619

# Reno, USA, 13th – 17th May, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 6469 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | endorsed CR for serving cell RRM measurement relaxation test case R16 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh2-Perf |  |  |  |  |  | Date: |  |  | 2019-05-15 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The CR had been endorsed but not formally agreed. Serving cell measurement relaxation with WUS test case is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | RRM test for serving cell RRM measurement relaxation for WUS-capable UE needs to be introduced for HD-FDD. Additional editorial changes are made upon endorsed CR R4-1813668. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.2.38 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.521-3 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<< Start of Change 1 >>

A.4.2.38 HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with serving cell RRM measurement relaxation

A.4.2.38.1	Test Purpose and Environment

This test is to verify the requirement for the HD-FDD intra frequency cell reselection requirements for Cat-NB1 UE specified in clause 4.6.2.1A when UE is configured to monitor WUS according to Table A.4.2.38.1-1 and under the serving cell RRM measurement relaxation according to the subclause 4.6.2.1A and under the intra-frequency neighbor cell measurement relaxation according to the subclause 4.6.2.2.

The test scenario comprises of 1 E-UTRA carrier with two eCells of different cell ID and one NB-IoT carrier with 2 nCells of different physical cell ID, as given in tables A.4.2.38.1-1, A.4.2.38.1-2 and A.4.2.38.1-3. The test consists of two successive time periods, with time duration of T1 and T2, respectively. Only nCell1 is already identified by the UE prior to the start of the test, i.e. nCell 2 is not identified. nCell 1 and nCell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing nCell 2.

Table A.4.2.38.1-1: General test parameters for HD-FDD intra frequency cell reselection test case for Cat-NB1 UE in normal coverage

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NB-IOT operational mode |  |  | In-band |  |
| Initial condition | Active cell |  | nCell1 |  |
|  | Neighbour cells |  | eCell1, eCell2, nCell2 |  |
| T2 end condition | Active cell |  | nCell2 |  |
|  | Neighbour cells |  | eCell1, eCell2, nCell1 |  |
| Final condition | Visited cell |  | nCell1 |  |
| E-UTRA RF Channel Number |  |  | 1 | One carrier frequency is used for eCell1 and eCell2. |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| NPRACH Configuration |  |  | NPRACH.R-1 | Refer to A.3.18 |
| SSearchDeltaP |  | dB | [12] | Threshold for relaxed monitoring criterion as specified in 5.2.4.12.1 in [1] |
| Rmax |  |  | [128] |  |
| maxDurationFactor |  |  | [one4th] | WUS config. Wmax = 32 (=1/4*Rmax) |
| numPOs |  |  | [n1] | WUS config. Single PO mapped to each WUS occasion |
| timeOffsetDRX |  |  | [ms40] | WUS config. Gap between the end of WUS duration to the associated PO |
| numDRX-CycleRelaxed |  |  | [4] | Serving cell RRM measurement is relaxed by |
| DRX cycle length |  | s | 1.28 | The value shall be used for all cells in the test. |
| T1 |  | s | >[30] | During T1, nCell2 shall be powered off, and during the off time the physical cell identity shall be changed. The intention is to ensure that nCell2 has not been detected by the UE prior to the start of period T2 |
| T2 |  | s | [70] | T2 is defined so that cell re-selection time is taken into account. |

Table A.4.2.38.1-2: nCell 1, nCell 2 specific test parameters for HD-FDD intra frequency cell reselection test case for Cat-NB1 UE in normal coverage

| Parameter | Unit | nCell 1 |  | nCell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| BWchannel | kHz | 180 |  | 180 |  |
| PRB location within eCell | - | eCell 1 BWchannel 5MHz: 18 eCell 1 BWchannel 10MHz: 30 |  | eCell 2 BWchannel 5MHz: 18 eCell 2 BWchannel 10MHz: 30 |  |
| NPBCH_RA | dB | -3 |  | -3 |  |
| NPBCH_RB | dB |  |  |  |  |
| NPSS_RA | dB |  |  |  |  |
| NSSS_RA | dB |  |  |  |  |
| NPDCCH_RA | dB |  |  |  |  |
| NPDCCH_RB | dB |  |  |  |  |
| NPDSCH_RA | dB |  |  |  |  |
| NPDSCH_RB | dB |  |  |  |  |
| NOCNG_RANote 1 | dB |  |  |  |  |
| NOCNG_RBNote 1 | dB |  |  |  |  |
| Qrxlevmin | dBm | -140 | -140 | -140 | -140 |
| Pcompensation | dB | 0 | 0 | 0 | 0 |
| Qhysts | dB | 0 | 0 | 0 | 0 |
| Qoffsets, n | dB | 0 | 0 | 0 | 0 |
| Cell_selection_and_ reselection_quality_measurement |  | NRSRP |  | NRSRP |  |
|  | dBm/15 kHz | Specified in Table A.4.2.18.1-3 |  |  |  |
|  | dB | 17 | 13 | -infinity | 17 |
| Note2 | dB | 17 | -4.09 | -infinity | 3.79 |
| NRSRP Note2 | dBm/15 kHz | -81 | -85 | -infinity | -81 |
| Treselection | s | 0 | 0 | 0 | 0 |
| Propagation Condition |  | AWGN |  | AWGN |  |
| Antenna Configuration |  | 2x1 |  | 2x1 |  |
| Timing offset to nCell 1 | ms | - |  | 3 |  |
| Note 1: NOCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Es/Iot and NRSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

Table A.4.2.38.1-3: eCell 1 and eCell2 specific test parameters for HD-FDD intra frequency cell reselection test case for Cat-NB1 UE in normal coverage

| Parameter | Unit | eCell 1 |  | eCell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | T1 | T2 | T1 | T2 |
| BWchannel | MHz | 5 or 10 |  | 5 or 10 |  |
| NOCNG Pattern defined in clause D.3 | - | BWchannel 5MHz: NOP.4 FDD BWchannel 10MHz: NOP.1 FDD |  | BWchannel 5MHz: NOP.4 FDD BWchannel 10MHz: NOP.1 FDD |  |
| PBCH_RA |  | dB | -3 |  | -3 |  |
| PBCH_RB |  | dB |  |  |  |  |
| PSS_RA |  | dB |  |  |  |  |
| SSS_RA |  | dB |  |  |  |  |
| PDCCH_RA |  | dB |  |  |  |  |
| PDCCH_RB |  | dB |  |  |  |  |
| PDSCH_RA |  | dB |  |  |  |  |
| PDSCH_RB |  | dB |  |  |  |  |
| OCNG_RANote 1 |  | dB |  |  |  |  |
| OCNG_RBNote 1 |  | dB |  |  |  |  |
| Qrxlevmin |  |  |  |  | dBm | -140 | -140 | -140 | -140 |
| Pcompensation |  |  |  |  | dB | 0 | 0 | 0 | 0 |
| Qhysts |  |  |  |  | dB | 0 | 0 | 0 | 0 |
| Qoffsets, n |  |  |  |  | dB | 0 | 0 | 0 | 0 |
| Note2 | dBm/15 kHz | -98 |  | -98 |  |
| Note2 |  |  |  |  | dBm | 3 | 3 | 3 | 3 |
| Treselection |  |  |  |  | s | 0 | 0 | 0 | 0 |
| Propagation Condition |  | AWGN |  | AWGN |  |
| Antenna Configuration |  | 2x1 |  | 2x1 |  |
| Timing offset to eCell 1 | ms | - |  | 3 |  |
| Note 1: OCNG shall be used such that the Cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power . Note 3: Void |  |  |  |  |  |

A.4.2.38.2	Test Requirements

Before the beginning of T2, UE is under relaxed monitoring where the serving cell measurement is performed every 5.12s and the infra-frequency measurement for the neighbor cells is relaxed according to 5.2.4.12.0 in [1].

The cell reselection delay to a newly detectable cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on nCell 2, and starts to send preambles on the PRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure on nCell 2.

The cell re-selection delay to a newly detectable cell shall be less than [69.56]s.

The cell reselection delay to an already detected cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on nCell 2, and starts to send preambles on the PRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure on nCell 2.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a newly detectable cell can be expressed as: Tevaluate, serv_NB-NC + Tdetect,NB_Intra_NB-IoT-NC + TSI.

Where:

Tdetect,NB_Intra_NB-IoT-NC		         See Table 4.6.2.2-1 in clause 4.6.2.2, based on the configured DRX cycle

Tevaluate, serv_NB-NC	 See Table 4.6.2.2-1 in clause 4.6.2.2, based on the effective DRX cycle after relaxation; [10.24]s is assumed in this test case.

TSI	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 8.32 s is assumed in this test case.

This gives a total of [69.56] s, allow [70] s for the cell re-selection delay to a newly detectable in the test case.

<< End of Change 1 >>