3GPP TSG-RAN4 WG4 Meeting #91	R4-1905658

# Reno, Nevada, USA, 13 - 17 May, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | - | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR: Addition of test applicability for mandatory features with capability signalling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-04-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | RAN1 defines many mandatory with capability signalling features, those should not have the equal priority with mandatory without capability signalling features, related test applicability is needed for those requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the test applicability for requirements defined for mandatory with capability signalling features. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE will be mandate to support all mandatory with capability signalling features from day one. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1, 6.1.1, 7.1.1, 8.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

5	Demodulation performance requirements (Conducted requirements)

5.1	General

5.1.1	Applicability of requirements

5.1.1.1	General

The minimum performance requirements are applicable to all FR1 operating bands defined in [TS 38.101-1].

The minimum performance requirements in Clause 5 are mandatary for UE supporting NR operation, except test cases listed in Clause 5.1.1.3 and Clause 5.1.1.4.

5.1.1.2	Applicability of requirements for different number of RX antenna ports

The number of RX antenna ports for different RF operating bands is up to UE declaration.

The UE shall support 2 or 4 RX antenna ports for different RF operating bands. The operating bands, where 4 RX antenna ports shall be the baseline, are defined in Clause 7.4 of TS 38.101-1 [6]. The UE requirements applicability for UEs with different number of RX antenna ports is defined in Table 5.1.1.2-1.

Table 5.1.1.2-1: Requirements applicability

| Supported RX antenna ports | Test type | Test list |
| --- | --- | --- |
| UE supports only 2RX | PDSCH | All tests in Clause 5.2.2 |
|  | PDCCH | All tests in Clause 5.3.2 |
|  | PBCH | All tests in Clause 5.4.2 |
| UE supports only 4RX or both 2RX and 4RX | PDSCH | All tests in Clause 5.2.3 |
|  | PDCCH | All tests in Clause 5.3.3 |
|  | PBCH | All tests in Clause 5.4.2 or 5.4.3 Note1 |
| Note1: Requirements for PBCH with 4Rx is up to UE declaration |  |  |

5.1.1.3	Applicability of requirements for optional UE capabilities

For UE which supports optional UE capabilities the additional performance requirements from Table 5.1.1.3-1 should be applied.

Table 5.1.1.3-1: Requirements applicability for optional UE capabilities

| UE feature/capability | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| [Enhanced Type X receiver] | FR1 FDD | PDSCH | 5.2.2.1.1 Minimum requirements for PDSCH Mapping Type A (Test 3-1) 5.2.3.1.1 Minimum requirements for PDSCH Mapping Type A (Test 5-1) |  |
|  | FR1 TDD | PDSCH | 5.2.2.2.1 Minimum requirements for PDSCH Mapping Type A (Test 3-1) 5.2.3.2.1 Minimum requirements for PDSCH Mapping Type A (Test 5-1) |  |
| [Support alternative additional DMRS position for co-existence with LTE CRS] | FR1 FDD | PDSCH | 5.2.2.1.4 Minimum requirements for PDSCH Mapping Type A and LTE-NR coexistence (Test 1-2) 5.2.3.1.4 Minimum requirements for PDSCH Mapping Type A and LTE-NR coexistence (Test 1-2) |  |

5.1.1.4	Applicability of requirements for mandatory with UE capabilities

The requirements in this clause are applicable to UEs that support the corresponding mandatory with capability signalling features. The test case applicability is in according to table 5.1.1.4-1 depending on set of supported UE capabilities.

Table 5.1.1.4-1: Requirements applicability for mandatory with capability

| UE feature/capability |  | Test type |  |  |  |  | Test list |  | Applicability notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Support maximum 4 MIMO layers | FR1 FDD |  |  | PDSCH |  | 5.2.3.1.1 (Test 4-1) |  |  |  |
|  | FR1 TDD |  |  | PDSCH |  | 5.2.3.2.1 (Test 4-1) |  |  |  |
| Support 2 additional DMRS with single-symbol FL DMRS for DMRS configuration type 1 | FR1 FDD | PDSCH | 5.2.3.1.1 (Test 1-1) |  |
|  | FR1 TDD | PDSCH | 5.2.3.2.1 (Test 1-1) |  |
| Support of PDSCH mapping type B | FR1 FDD | PDSCH | 5.2.3.1.3 (Test 1-1) |  |
|  | FR1 TDD | PDSCH | 5.2.3.2.3 (Test 1-1) |  |
| 2 configured TCI states per CC for PDSCH |  |  | All test cases |  |

<Unchanged Sections Skipped>

6	CSI reporting requirements (Conducted requirements)

6.1	General

This section includes conducted requirements for the reporting of channel state information (CSI).

6.1.1	Applicability of requirements

6.1.1.1	General

The minimum performance requirements are applicable to all FR1 operating bands defined in TS 38.101-1 [6].

The minimum performance requirements in Clause 6 are mandatary for UE supporting NR operation, except test cases listed in Clause 6.1.1.3.

6.1.1.2	Applicability of requirements for different number of RX antenna ports

The number of RX antenna ports for different RF operating bands is up to UE declaration.

The UE shall support 2 or 4 RX antenna ports for different RF operating bands. The operating bands, where 4 RX antenna ports shall be the baseline, are defined in TS 38.101-1 [6, Clause 7.4]. The UE requirements applicability for UEs with different number of RX antenna ports is defined in Table 6.1.1.2-1.

Table 6.1.1.2-1: Requirements applicability

| Supported RX antenna ports | Test type | Test list |
| --- | --- | --- |
| UE supports only 2RX | CQI | All tests in Clause 6.2.2 |
|  | PMI | All tests in Clause 6.3.2 |
|  | RI | All tests in Clause 6.4.2 |
| UE supports only 4RX or both 2RX and 4RX | CQI | All tests in Clause 6.2.3 |
|  | PMI | All tests in Clause 6.3.3 |
|  | RI | All tests in Clause 6.4.3 |

6.1.1.3	Applicability of requirements for optional UE capabilities

<Unchanged Sections Skipped>

7	Demodulation performance requirements (Radiated requirements)

7.1	General

7.1.1	Applicability of requirements

7.1.1.1	General

The minimum performance requirements are applicable to the FR2 operating bands defined in TS 38.101-2 [7] with FDL_high not exceeding 40000 MHz.

The minimum performance requirements in Clause 7 are mandatary for UE supporting NR operation, except test cases listed in Clause 7.1.1.3 and Clause 7.1.1.4.

7.1.1.2	Applicability of requirements for different number of RX antenna ports

UE shall support 2 RX ports for different RF operating bands. The UE requirements applicability is defined in Table 7.1.1.2-1.

Table 7.1.1.2-1: Requirements applicability

| Supported RX antenna ports | Test type | Test list |
| --- | --- | --- |
| UE supports 2RX antenna ports | PDSCH | All tests in Clause 7.2.2 |
|  | PDCCH | All tests in Clause 7.3.2 |
|  | PBCH | All tests in Clause 7.4.2 |

7.1.1.3	Applicability of requirements for optional UE capabilities

For UE which supports optional UE capabilities the additional performance requirements from Table 7.1.1.3-1 should be applied.

Table 7.1.1.3-1: Requirements applicability for optional UE capabilities

| UE feature/capability | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| [Enhanced Type X receiver] | FR2 TDD | PDSCH | 7.2.2.2.1 Minimum requirements for PDSCH Mapping Type-A (Test 3-1) |  |

7.1.1.4	Applicability of requirements for mandatory with UE capabilities

The requirements in this clause are applicable to UEs that support the corresponding mandatory with capability signalling features. The test case applicability is in according to table 7.1.1.4-1 depending on set of supported UE capabilities.

Table 7.1.1.4-1: Requirements applicability for mandatory with capability

| UE feature/capability | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| Support maximum 2 MIMO layers | FR1 FDD | PDSCH |  |  |
|  | FR1 TDD | PDSCH | 7.2.2.2.1 (Test 2-1 ~ 2-6) |  |
| Support 1 port of PTRS for FR2 | FR1 FDD | PDSCH |  |  |
|  | FR1 TDD | PDSCH | All test cases |  |
| Support of PDSCH mapping type B | FR1 FDD | PDSCH |  |  |
|  | FR1 TDD | PDSCH |  |  |
| 2 configured TCI states per CC for PDSCH | FR1 TDD | PDSCH | All test cases |  |

<Unchanged Sections Skipped>

8	CSI reporting requirements (Radiated requirements)

8.1	General

This section includes radiated requirements for the reporting of channel state information (CSI).

8.1.1	Applicability of requirements

<TBA>

<End of Changes>