3GPP TSG-RAN WG4 Meeting #91	 	R4-1905706

Reno, USA, 13th – 17th May 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to 38.133 on TCI State Switching Requirements (Section 8.10) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2019-05-03 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Update to TCI state switching delay requirements |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updated definition of known TCI state - Updated requirement for MAC CE based TCI state switching delay - Deleted Notes on support for power class 2/3/4 - Added section for Active TCI state list update delay |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TCI state swithing delay requirements are incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.10.2, 8.10.3, 8.10.4, 8.10.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS/TR 38.533 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of Change

### 8.10.2	Known conditions for TCI state

The TCI state is known if it has been meeting the following conditions:

- TCI state switch is within X ms of last transmission of the resource for beam reporting/ measurement

  - There shall be at least 1 report from the UE between the measurement opportunity and TCI state switch

  - X = [1280] ms for UE supporting power class 1

  - X = [160] ms for UE supporting power class 2/3/4

- The TCI state is detectable during the TCI state switching period

  - The UE shall be configured with highest possible aggregation level for PDCCH with the new TCI state for MAC CE based switch

  - The UE shall be scheduled with lowest possible MCS for PDSCH with the new TCI state for DCI based switch

Otherwise, the TCI state is unknown.

### 8.10.3	MAC-CE based TCI state switch delay

If the target TCI state is known, upon receiving PDSCH carrying MAC-CE activation command at slot n, UE shall be able to receive PDCCH on the new beam of the serving cell on which TCI state switch occurs no later than at slot n+ THARQ +3ms, where THARQ is the timing between DL data transmission and acknowledgement as specified in [7].

If the target TCI state is unknown, upon receiving PDSCH carrying MAC-CE activation command at slot n, UE shall be able to receive PDCCH on the new beam of the serving cell on which TCI state switch occurs no later than at slot n+ THARQ +3ms + TRxBeamAcq, where TRxBeamAcq is the time for Rx beam refinement for the new TCI state.

Where TRxBeamAcq can be defined as

  - For SSB: 8xTSSB-RS

  - For periodic CSI-RS: NxTCSI-RS ; Where N=8 for Repetition ‘OFF’ set; N=ceil(MaxNumRxBeam/#CSI-RS in set) for Repetition ‘ON’ set

  - For aperiodic CSI-RS: TL1-RSRP ; Where TL1-RSRP is the time for Rx beam measurement/ refinement for the target TCI state

### 8.10.4	DCI based TCI switch delay

If the target TCI state is known, when a UE is configured with the higher layer parameter tci-PresentInDCI which is set as 'enabled' for the CORESET scheduling the PDSCH at slot n, UE shall be able to receive PDSCH or transmit PUSCH on the new beam of the serving cell on which TCI state switch occurs no later than at slot n+timeDurationForQCL, where, timeDurationForQCL is the time required by the UE to perform PDCCH reception and applying spatial QCL information received in DCI for PDSCH processing as described in TS 38.214 [26], the value of timeDurationForQCL is defined in TS 38.306[14].

The known condition for TCI state defined in section 8.7.2 is applied.

## 8.10.5	Active TCI state list update delay

If the target TCI state is known, upon receiving PDSCH carrying MAC-CE active TCI state list update at slot n, UE shall be able to receive PDSCH on the new beam of the serving cell on which TCI state switch occurs no later than at slot n + THARQ + 3ms +1slot + timeDurationForQCL. Where THARQ is the timing between DL data transmission and acknowledgement as specified in [7] and , timeDurationForQCL is the time required by the UE to perform PDCCH reception and applying spatial QCL information received in DCI for PDSCH processing as described in TS 38.214 [26], the value of timeDurationForQCL is defined in TS 38.306[14].

End of Change