3GPP TSG-RAN WG4 Meeting #91	     R4-1905715

Reno, USA, 13th – 17th May 2019

Agenda item:      13

Source:                Intel Corporation

Title:	                     Scope of Rel-16 NR RRM URLLC Requirements

Document for:     Discussion

# Introduction

The Rel-15 NR RRM requirements mainly cover the key NR functionality for the mandatory UE features and basic scenarios. Many important scenarios, use cases and features were deprioritized in the Rel-15 timeframe. In RAN #83 meeting several WI proposals on the continuation of the NR RRM requirements work were made and a possible scope of Rel-16 NR RRM work was discussed, but no final conclusions were reached. The WF [1] captures the status of the discussion and it is expected that interested companies will continue discussion till RAN #84 to make the scope stable. In this contribution we provide views on the scope of the Rel-16 NR RRM work for URLLC.

# Discussion

Ultra-reliable low-latency communication (URLLC) is one of the use cases supported by NR. The key goal for URLLC is to achieve reliable quality and low latency. The requirement in Rel-15 is 99.999% error free transmission (10-5 BLER) and latency of 1ms. In order to meet the stringent requirements for URLLC, a number of features have been introduced in NR for Rel-15 to support high reliability and low latency both in the UL and DL.

Features for improved reliability in the DL

- Additional CQI and MCS table targeting low spectral efficiency and target BLER of 10-5

- Aggregation level of 16 for PDCCH

- Slot aggregation

Features to support low latency

- Scalable numerology

- Non-slot based scheduling – Mapping TypeB for  PDSCH and PUSCH

- Pre-emption indication

- CBG based re-transmissions

- Shortened UE processing time

- Self-contained slot

- Low latency CSI feedback

Observation #1: In Rel-15 several features are introduced to support URLLC use case in NR

Most of the features to enable URLLC have not been tested in Rel-15 NR RRM requirements. URLLC is one of the key use cases for NR and it is critical that demodulation and CSI requirements are introduced to verify these features.

Observation #2: In Rel-15 no features that support URLLC have RRM requirements

As part of Rel-16 scope for RRM requirements, we propose to study the impact on RRM requirements due to the following features related to URLLC and introduce the requirements accordingly.

- 99.999% Reliability: For URLLC the target BLER is 0.001%. In order to meet the reliability requirement specified for URLLC, the link needs to be reliable. The RLM and BFD thresholds in Rel-15 might not be suitable for URLLC. The impact on radio link monitoring and beam failure detection threshold configuration shall be studied in Rel-16.

- UE processing time: To support low latency transmission for URLLC, UE capability for shortened processing time has been introduced. In Rel-16 the impact to RRM requirements due to this feature shall be studied.

- Self-contained slot: In Rel-15 flexible TDD configurations like DL and UL transmission within the same slot are introduced to enable shorter HARQ round trip time in order to reduce latency. In Rel-16 NR RRM impact due to this feature shall be evaluated.

Proposal #1: In Rel-16 RRM WI study the impact to RRM requirements due to the following features to support URLLC

- 99.999% Reliability

- Shortened UE processing time

- Self-contained slot

# Conclusion

In this paper we present our views on Rel-16 NR RRM requirements impact due to features to support URLLC. We have the following observations and proposals.

Observation #1: In Rel-15 several features are introduced to support URLLC use case in NR

Observation #2: In Rel-15 no features that support URLLC have RRM requirements

Proposal #1: In Rel-16 RRM WI study the impact to RRM requirements due to the following features to support URLLC

- 99.999% Reliability

- Shortened UE processing time

- Self-contained slot

# Reference

- RP-190659 “WF on the scope of R-16 RRM and Demod work in RAN4”, Intel, RAN #83, March 2019