3GPP TSG-RAN WG4 Meeting #91                                 R4-1906155

Reno, Nevada, USA, May 13th – 17th, 2019

Source: 	MediaTek Inc.

Title: 	OBB issue for n77/n78/n79 pairing with low-band in CA/SUL/EN-DC

Agenda Item:	6.5.6

Document for:	Approval

# Introduction

The issue on out-of-band blocking (OBB) requirement for n77 and n78 pairing with a low frequency band (< 1 GHz) in CA or SUL due to the 2nd order intermodulation product generated from the low-band UL and the blocker which may fall onto the n77/n78 DL carrier to cause significant sensitivity degradation has been brought up in [1]. To mitigate this issue, a separate OBB exception in addition to the general exceptions for spurious response has been defined in the current technical specifications TS 38.101-1 [2]. In last RAN4 meeting, a draft CR was proposed to remove this exception for the combinations with n78 [3], based on the assessment that UE supporting n78 does not necessarily support n77. However, the counter argument was that even with n78 filter, the rejection into and above n77 frequency range was still quite minimal. To further highlight this potential issue, in this contribution, we provide both n78 and n77 filter data from a few component vendors. Based on these filter data and our link budget analysis, we think there is sufficient technical justification to preserve this exception in the spec. In addition, we think the same exception should also apply to the similar combinations with n79 and the EN-DC counterparts.

# Discussion

The potential out-of-band blocker (OBB) issue for n77 and n78 pairing with a low frequency band (< 1 GHz) in CA or SUL can be illustrated in Figure 2-1. Owing to the wide frequency range in the band, the OBB near the band edges can be separated from the wanted signal by a frequency gap which equals to the pairing low-band (LB) UL carrier frequency. As a result, the 2nd order intermodulation product generated from the LB UL and the blocker would fall onto the wanted signal to cause sensitivity degradation.

Figure 2-1 The potential OBB issue for (a) n77 and (b) n78 pairing with a low frequency band

To quantify this IMD2 desensitization issue, we have re-performed link analysis based on the typical UE RF parameters as summarized in Table 2-1 and the filter data collected from a few component vendors as summarized in Table 2-2 and Table 2-3 for n78 and n77, respectively. Notice that the IMD2 is expected to be dominated by the LNA 2nd order nonlinearity, therefore, only LNA IIP2 is specified in the table below.

| Parameter | Value | Comment |
| --- | --- | --- |
| Front-end Loss (dB) | 4 |  |
| Diplexer Rejection at LB (dB) | 20 | Shared antenna architecture assumed |
| LB UL Power under OBB (dBm), PLB_UL | 19 | Referred to antenna port (4 dB below PCMAX_L,f,c) |
| n78 Filter Rejection at LB (dB) | 32 | See Table 2-2 |
| LNA IP2 (dBm) | 7 | Referred to antenna port |
| Pw (dBm) | -89.8 | REFSENS + 6 dB for n78 CBW = 10 MHz, SCS = 15 kHz |
| SNR Requirement (dB) | -1 | QPSK |

Table 2-1 Typical UE RF parameters for IMD2 link analysis

| n78 Filter | IL (dB) | Rejection (dB) in MHz Frequency Range |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | < 2700 | 2700 - 3150 | 3900 - 4000 | 4000 - 4100 | 4100 - 4200 | 4200 - 4300 | 4300 - 4400 | 4400 - 5000 |
| Vendor A | 1.54 (typ) | 46.5 (typ) | N/A | 0.2 (typ) | 5.4 (typ) | 5.4 (typ) | N/A | N/A | N/A |
| Vendor B | 1.25 (max) | 32 (min) | 1.5 (typ) | N/A |  | 4.1 (typ) | 4.1 (typ) | 7 (min) | N/A |
| Vendor C - 1 | 1.8 (max) | 32 (min) | N/A | 18 (min) | 18 (min) | 18 (min) | N/A | N/A | 23 (min) |
| Vendor C - 2 | 2.5 (max) | 34 (min) | N/A | 19 (min) | 19 (min) | 19 (min) | N/A | N/A | 36 (min) |

Table 2-2 Collective n78 filter data from component vendors

| n77 Filter | IL (dB) | Rejection (dB) in MHz Frequency Range |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | < 2700 | 2700 - 3050 | 4450 - 4900 | 4900 - 5925 | 5150 - 5850 |
| Vendor X | 1.25 (typ) | 44 (typ) | 5.4 (typ) | 2.9 (typ) | 39.7 (typ) | N/A |
| Vendor Y | 1.90 (max) | 40 (min) | N/A | N/A | N/A | 20 (min) |

Table 2-2 Collective n77 filter data from component vendors

To pass the OBB requirement, the resultant IMD2 power level shall not exceed the wanted signal power level (Pw), that is,

PIMD2 = PLB_UL – 32dB – 20dB + Pblocker – IP2LNA ≤ -89.8 dBm

or equivalently,

Pblocker ≤ -89.8 dBm – 19 dBm + 32 dB + 20 dB + 7 dBm = -49.8 dBm

Which means the blocker power level needs to be lower than -49.8 dBm if n78 filter does not provide any rejection in the OBB frequency range.

Observation 1: For UE to pass OBB requirement, the blocker level needs to be lower than -49.8 dBm if n78 filter does not provide any rejection in the OBB frequency range.

Based on the n78 filter data as summarized in Table 2-2, it can be seen that for Vendor A and Vendor B, the filters virtually do not provide any rejection even to up 4300 MHz and down to 2700 MHz. For Vendor C, although the filters can provide 18 dB or more rejection at above 4000 MHz, it is at the cost of higher in-band insertion loss. On the other hand, even with 18dB OBB rejection, according to the above calculation, it is still not sufficient to pass both range 2 and range 3 OBB requirements where the blocker level is specified at -30 dBm and -15 dBm respectively.

Observation 2: Some n78 filters virtually do not provide any out-of-band rejection for frequency range up to 4300 MHz and down to 2700 MHz.

Observation 3: For n78 filters which provide 18 dB or more rejection at above 4000 MHz, they are associated with higher in-band insertion loss.

Observation 4: Even with 18dB OBB rejection, it is still not sufficient to pass both range 2 and range 3 OBB requirements where the blocker level is specified at -30 dBm and -15 dBm respectively.

For UEs supporting both n77 and n78, it is very likely that both bands would share the same RF front-end filter which covers the n77 band range. In this case, there is virtually no out-of-band rejection for frequency range up 4900 MHz and down to 2700 MHz.

Observation 5: For UEs supporting both n77 and n78 and share a common n77 filter, there is virtually no out-of-band rejection for frequency range up 4900 MHz and down to 2700 MHz.

Based on the above assessment, we think there is sufficient technical justification to preserve this exception in the spec. We also think it would not be reasonable to force a UE to use a higher cost or more filter components to simply pass the artificially engineered OBB requirement without exceptions where the blocking condition may not exist in real field and yet such filters may render higher insertion loss.

Observation 6: It would not be reasonable to force a UE to use a higher cost or more filter components to simply pass the artificially engineered OBB requirement without exceptions where the blocking condition may not exist in real field and yet such filters may render higher insertion loss.

In addition to the combinations with low frequency bands pairing with n77 and n78, we also think this exception should apply for the combinations with low frequency bands pairing with n79 as n79 has wider frequency range than n78 and is expected to have similar out-of-band filter rejection characteristic with n77 and n78.

Proposal 1: The proposed OBB exception also applies to the combinations with low frequency bands pairing with n79.

The proposed OBB exception should also apply to the similar inter-band EN-DC combinations when NR band OBB is tested with E-UTRA UL carrier output power set to 4dB below PCMAX_L to complete the specifications work.

Proposal 2: The proposed OBB exception should also apply to the similar inter-band EN-DC combinations when NR band OBB is tested.

Proposal 3: The following CA, SUL, and EN-DC combinations should be included in the OBB exception band combination list in Rel-15 technical specifications.

CA_n8-n78

CA_n28-n78

CA_n8-n79

SUL_n78-n81

SUL_n78-n82

SUL_n78-n83

SUL_n79-n81

DC_5_n78

DC_8_n77

DC_8_n78

DC_8_n79

DC_11_n77

DC_18_n77

DC_18_n78

DC_18_n79

DC_19_n77

DC_19_n78

DC_19_n79

DC_20_n77

DC_20_n78

DC_21_n77

DC_26_n77

DC_26_n78

DC_26_n79

DC_28_n77

DC_28_n78

DC_28_n79

The OBB exception defined in the current technical specifications [2] only considers the blocker above the upper band edge. However, the IMD2 issue can also occur for blocker located below the lower band edge. Therefore, the formula in the NOTE 1 should be modified as below.

Table 7.6A.3.3-2: Requirement for out-of-band blocking exceptions

| Parameter | Unit | Level |
| --- | --- | --- |
| PInterferer (CW) | dBm | -501 |
| NOTE 1: The requirement applies when¦fInterferer- fSUL- fDL¦≤(BWSUL+ BWDL)/2 fIntereferer±- fULLB- fDLHB≤(BWULLB+ BWDLHB)/2, where fULLB and fDLHB are the carrier frequencies for lower frequency band UL and higher frequency band DL, respectively. BWULLB and BWDLHB are the channel bandwidths configured for lower frequency band UL carrier and higher frequency band DL carrier in MHz, respectively. |  |  |

Proposal 4: The proposed OBB exception should consider both the blocker above the upper band edge and below the lower band edge.

To address the above proposed changes, the corresponding draft CRs will be submitted to complete the specifications work for the intended OBB exception requirement.

# Conclusion

In this contribution, we provide further technical justification on the need of additional OBB exception requirement for the band combinations having n77/n78/n79 paring with a low frequency band.

# References

- R4-1810235 “Out-of-band blocker issue for n77/n78 pairing with low-band in CA/SUL”, MediaTek Inc., 3GPP TSG-RAN WG4 Meeting #88, Gothenburg, Sweden, August 20th – 24th, 2018

- 3GPP TS 38.101-1 V15.5.0 (2019-03), clauses 7.6A.3.3 and 7.6C.3

- R4-1904587 “Draft CR for correction of out-of-band blocking exceptions for CA in TS 38.101-1”, Vodafone, 3GPP TSG-RAN WG4 Meeting #90bis, Xi’an, China, April 8th – 12th, 2019