3GPP TSG-RAN WG4 Meeting #91	R4-1906269

# Reno, NV, 13 – 17 May, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | <CR#> | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Amendments to the channel raster to resouce grid mapping |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-05-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The mapping between the channel raster (carrier frequency) and the resource grid is ambigous in case the UE channel bandwidth is different from the BS channel (cell) bandwidth. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 5.4.2.2: the UE carrier frequency is determined by the resource grids provided for determining the location of the UE-specific channel bandwidth. The cell specific resource grids are used if UE-specific bandwidth are absent. If the UE-specific channel bandwidths are absent and the SCS-depndent resource grids are not aligned then the carrier frequency can be ambigous (the mapping applies to the resource grid configured for at least one sub-carrier configuration). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguous specification of the UE carrier frequency. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

< start of changes >

#### 5.4.2.2	Channel raster to resource element mapping

The mapping between the RF reference frequency on the channel raster and the center of the corresponding resource element is given in Table 5.4.2.2-1 and can be used to identify the RF channel position. The mapping depends on the total number of RBs that are allocated in the channel and applies to both UL and DL. The mapping applies to the resource grid of size Ngridsize,μprovided in the uplinkChannelBW-PerSCS-List field of the UplinkConfig IE for the UL and the DownlinkChannelBW-PerSCS-List field of the ServingCellConfig IE for the DL for the largest μ value among the subcarrier spacing configurations provided. If any of these fields are absent the mapping applies to a resource grid of size Ngridsize,μ configured in the scs-SpecificCarrierList of the respective UplinkConfigCommon / UplinkConfigCommonSIB IE and DownlinkConfigCommon / DownlinkConfigCommonSIB IE for at least one the μ values among the subcarrier spacing configurations supported by the UE.

Table 5.4.2.2-1: Channel raster to resource element mapping

|  | Ngridsize,μmod 2=0 | Ngridsize,μmod 2=1 |
| --- | --- | --- |
| Resource element index k within resource block | 0 | 6 |
| Physical resource block number nPRB within the resource grid | nPRB=Ngridsize,μ2 |  |

The parameters μ and Ngridsize, μ are as defined in [6]. For the carrier bandwidths configured in the uplinkChannelBW-PerSCS-List and DownlinkChannelBW-PerSCS-List fields  Ngridsize, μ=NRB as specified in [7], with  NRB according to sub-clause 5.3.2.

#### 5.4.2.3	Channel raster entries for each operating band

< end of changes >