3GPP TSG-RAN WG4 Meeting #91             	R4-1906317

Reno, Nevada, USA, 13 May – 17 May 2019

Title:	RRC based BWP Switching Delay

Source:	NEC

Agenda item:	6.10.7.5.1

Document for:	Approval

Introduction

In previous RAN4 meetings, most of the issues are addressed for defining requirements for BWP switching, other than delay requirements and scheduling restriction for RRC based BWP switching. In this contribution we provide our views on the delay requirements.

Discussion

Upon reception of BWP switch request, BWP switching delay generally involves, processing time to process the BWP switching command and BWP switching time. At the end of BWP switch, UE should be able to Transmit/Receive on new BWP.

In RAN4#90-Bis meeting, following agreements [1] are made for RRC based BWP switching.

- A single delay in millisecond for all SCSs

- Regardless whether the carrier bandwidths containing old and new BWPs are overlapping or not, no AGC tuning time is needed during delay period

- Worst-case delay requirements for RRC-based BWP switch is within the range of [5~8] ms

- For RRC based BWP switching, the scheduling restriction will be specified in the next meeting

After RAN4#90-Bis, open issues involve, delay requirement for RRC based BWP switch and scheduling restriction due to RRC based BWP switch. In the following sections, we provide our views on delay requirements

RRC-based BWP switching delay

RRC processing delay

BWP switch delay RRC

RRC Reconfiguration Complete

RRC Reconfiguration Command

Figure 1: RRC based BWP Switching

In the last RAN4 meeting it was agreed that total RRC based BWP switching delay is equal to

TBWPSwitchDelay = TRRCprocessingDelay + TBWPswitchDelayRRC

TRRCprocessingDelay - RRC processing delay is defined by RAN2. According to TS 38.331 this is 10ms [2]

TBWPswitchDelayRRC – RRC based BWP switch delay

BWP switch can potentially change SCS, BW, Local Oscillator (LO) frequency, AGC settling or subset of the those [4]. In other words BWP switch could potentially change baseband reconfiguration, RF reconfiguration.

RRC based BWP switch delay

In TS 36.331 clauses 11.2, RRC connection re-configuration (intra-LTE mobility) delay is defined as 15ms. In broader terms BWP switch for scenario 1 and 2 can be treated as the similar scenario with difference of beam (SSB) acquisition time. In the 15 ms, since RRC processing delay of 10ms also present, it can be considered 5ms as the time required for BWP switch using RRC based BWP switching.

Total RRC based BWP switch delay is given by TBWPSwitchDelay = TRRCprocessingDelay+ TBWPswitchDelayRRC as discussed above.

TBWPSwitchDelay =15 ms; TRRCprocessingDelay = 10 ms;

TBWPswitchDelayRRC = 15 ms - 10 ms = 5 ms

RRC based BWP switch delay is equal to 5ms

Proposal 1: RRC based BWP switch delay to be defined as TBWPswitchDelayRRC = 5 ms

Proposal 2: Total RRC based BWP switch delay to be defined as TBWPSwitchDelay=15 ms

- Conclusion

In this contribution we have discussed requirements for RRC-based BWP switching and made the following proposals:

Proposal 1: RRC based BWP switch delay to be defined as TBWPswitchDelayRRC = 5ms

Proposal 2: Total RRC based BWP switch delay to be defined as TBWPSwitchDelay=15 ms

References

- RAN4#90-Bis meeting report

- TS 38.331-f40 Radio Resource Control (RRC) protocol specification

- TS 36.331-f40 Radio Resource Control (RRC) protocol specification

- R4-1802270