3GPP TSG-RAN WG4 Meeting #91                                              	 R4-1906386

Reno, Nevada, US, 13th to 17th May. 2019

Agenda Item:	9.3.2

Source: 	MediaTek Inc.

Title: 	Correction TP for TR37.716-11-11 for DC_66A_n25A

Document for:	Approval

- 1. Introduction

The EN DC 66A_n25A has been completed in Release-16 basket WI in RAN4 meetings (R4-1904437 RAN4#90-bis). The contribution is to provide missing MSD requirement TP for TR37.716-11-11 for the combination.

- 2. Discussion

TP for TR37.716-11-11

<Unchanged sections omitted>

6.1.41	DC_66_n25

<Unchanged sections omitted>

6.1.41.7	MSD

Based on the co-existence studies for DC_66_n25, there is a need to have MSD defined. The 3dB more MSD due to IMD3 fall into B66 was result from REFSENS difference.

Table 7.3B.2.3.5.1-1: Reference sensitivity exceptions for PCell due to dual uplink operation for EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_66A_n25A | 66 | 1775 | 5 | 25 | 2175 | N/A | TDD | N/A |
|  | n25 | 1855 | 5 | 25 | 1935 | 20 | FDD | IMD3 |
| DC_66A_n25A | 66 | 1712.5 | 5 | 25 | 2112.5 | 23 | TDD | N/A |
|  | n25 | 1912.5 | 5 | 25 | 1992.5 | N/A | FDD | IMD3 |
| DC_66A_n25A | 66 | 1750 | 5 | 25 | 2150 | 4 | TDD | IMD5 |
|  | n25 | 1883.3 | 5 | 25 | 1963.3 | N/A | FDD | N/A |

<Unchanged sections omitted>