3GPP TSG-RAN WG4 Meeting #91	R4-1906746

Reno, Nevada, US, 13 May – 17 May 2019

Source:	Ericsson, Telstra

Title:	TP for TR 37.716-21-11: MSD values for 1A-28A_n5A

Agenda item:			9.4.2

Document for:	Approval

# 1. Introduction

Rel-16 LTE-NR DC combinations LTE 2 band + NR 1 band combination are defined in the WID [1]. This contribution is a text proposal for TR 37.716-21-11 v0.5.0 to add the missing MSD values for 1A-28A_n5A

# 2. Text Proposal

# ---Start of changes---

## 5.1.44	DC_1A-28A_n5A

### 5.1.44.1	Operating bands for EN-DC

Table 5.2B.5.3-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-28_n5 | CA_1-28 | n5 | No |

### 5.1.44.2	Configuration for DC

Table 5.5B.5.3-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A-28A_n5A | DC_1A_n5A DC_28A_n5A | CA_1A-28A | n5 |

#### 5.1.44.3	Co-existence Studies

Co-existence analysis from DC_28_n5 shows that there is IMD5 impact from DC_28_n5 UL to Band 1 DL.

Co-existence analysis from DC_1_n5 shows that there is no impact on impact from DC_1_n5 UL to Band 28 DL.

### 5.1.44.4		∆TIB and ∆RIB values

For DC_1-28_n5, the TIB,c and RIB,c values are reused from the LTE combination CA_1-18-28, and are given in the tables below.

Table 5.1.44.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-28_n5 | 1 | 0.3 |
|  | 28 | 0.5 |
|  | n5 | 0.5 |

Table 5.1.44.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-28_n5 | 1 | 0 |
|  | 28 | 0 |
|  | n5 | 0 |

### 5.1.44.5	REFSENS requirements

Co-existence analysis show that there is IMD5 impact from DC_28_n5 UL to Band 1 DL. MSD values are therefore needed to be introduced in 38.101-3.

MSD values are reused from DC_18-28_n78 which has IMD5 to band 18 DL.

Table 7.3B.2.3.5.2-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-28A_n5A | 1 | 1961 | 5 | 25 | 2151 | 3.8 | FDD | IMD5 |
|  | 28 | 745 | 5 | 25 | 778 | N/A | FDD | N/A |
|  | n5 | 829 | 5 | 25 | 874 | N/A | FDD | N/A |

---End of changes---

# Reference

[1]	RP-190379, “Revised WID on Dual Connectivity (EN-DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL)”, Huawei