3GPP TSG RAN WG4 Meeting #92	R4-1908004

Ljubljana, SIovenia, 26 - 30 August 2019

Agenda item:	10.16.1

Source:	SoftBank Corp.

Title:	Summary of n41 30MHz configuration study

Document for:	Approval

1	Introduction

The WI of adding 30MHz CBW to n41 was approved in RAN#82 [1]. This paper is intended to summarize/record necessary aspects on 30MHz operation in 2545 -2575MHz.

2     Consideration

The following are basic assumptions for this study:

- Protection requirements are -25dBm/MHz @ 2530 – 2535MHz and -30dBm/MHz @ 2505 – 2530MHz.

- For 10MHz/20MHz CBW, as per LTE B41 of the same range, the protection requirements above can be satisfied with CBW position constraints and only with SEM [2].

- It is assumed that REL-15 n41 UE does not support 30MHz operation. Thus we have to conclude how to accommodate REL-15 UE (supporting 10MHz and 20MHz CBW) in 30MHz CBW operation.

Outstanding issue is:

1)  UE CBW determination in relation to BWP/carrierBW (sec 2.1 below),

As we have defined n90 as n41 with 100k raster in the last meeting, some considerations are added but impacts are fairly minor.

  - UE CBW determination

As a summary, at present for REL-15:

- in RRC_connected, a BS can instruct UE CBW using “uplink/downlinkChannelBW-PerSCS-List”,

- in RRC_idle, UE CBW is still outstanding and we received a relevant LS from RAN2 [3].

Judging from the LS, it is likely to use an existing IE with confined values to indicate UE CBW. Then it seems that we can instruct an appropriate CBW to a UE in the long run. Thus, we can rely on a mechanism to be defined to indicate UE CBW in RRC_idle.

[Observation-1] We can assume that there will be an appropriate method to provide UE CBW in RRC_idle.

2.2 Actual CBW arrangement, taking REL-15 UEs into account.

Based on assumptions above, we proposed the following schemes:

1) For legacy UEs, any 10MHz (CBW-1) and/or upper 20MHz (CBW-2) schemes are needed where no A-MPR is required.

2) For new UEs, 30MHz (CBW-3) is likely under A-MPR to make use of the expanded UE CBW.

3) Any 10MHz/upper 20MHz scheme without A-MPR (CBW-1/2) can also be used for the new UEs.

Figure 1 : UE CBW configurations

In addition, how to interpret “upper 20MHz(CBW-2)” should be clarified since 30MHz CBW will have GB or total RB numbers different from 10MHz, 20MHz or composite 10+20MHz. Since protection for 20MHz below (RED) is realized based on SEM, it is likely to set "20MHz CBW" in 30MHz operation so that RBs used are confined within original 20MHz position.

Figure 2: Upper 20MHz CBW arrangement (SCS=15k case)

Calculation results are included in Table 1. Note also that these results can be retained when Fc=2560[MHz] is considered for 100k raster in n90.

Table 1 : Conditions for 30MHz operation in 2545-2575MHz

|  | CBW-1 (10MHz) | CBW-2 (20MHz) | CBW-3 (30MHz) |
| --- | --- | --- | --- |
| UE not supporting 30MHz | Yes | Yes | No |
| UE supporting 30MHz | Yes | Yes | Yes |
| A-MPR | No | No | Yes |
| RB range for SCS=15k | Any 52RBs | upper 105RBs (RB55-RB159) | All RBs |
| RB range for SCS=30k | Any 24RBs | upper 50RBs (RB28-RB77) | All RBs |
| RB range for SCS=60k | Any 11RBs | upper 24RBs (RB14-RB37) | All RBs |

The table above summarizes the conditions of 30MHz operation. Then,

[Proposal-1] Table 1 is approved as operational conditions for 2545 -2575MHz for Japan, covering n90 also.

2.3 New NS

A new NS for 30MHz CBW can be added similar to a method supported in LTE Rel-10 (with NS-PmaxList) and backoff values/schemes were agreed in the last meeting. This time, a CR [4] for 38.101-1 is provided to support the NS (NS_47 assumed).

One thing to note is on n90. Since the simulation campaign this time assumed Fc=2560MHz, the result can be reused to n90 with 100k raster. Given that requirements for n90 are described collectively such as “ Unless otherwise stated, the applicability of requirements for Band n90 is in accordance with that for Band n41; a UE supporting Band n90 shall meet the requirements for Band n41”, this time Fc range is extended to corresponding 100k raster point.

[Observation-2] Simulation results can cover n90 deployment.

3	Conclusion

This paper summarizes 30MHz CBW operation of n41 in Japan, taking n90 into consideration. A proposal is:

[Proposal-1] Table 1 is approved as operational conditions for 2545 -2575MHz for Japan, covering n90 also.

Reference

RP-182828		New WI proposal: Addition of 30MHz channel bandwidth for NR band n41, KDDI

R4-1900452		On protection requirement in Japan for n41 30MHz CBW, SoftBank Corp.

R4-1907911	   LS on supported BW for initial BWP, RAN2, Nokia (R2-1908301)

R4-1908005		Addition of NS information on 30MHz support for n41, (Cat B CR), SoftBank Corp.