3GPP TSG-RAN4 Meeting #92	R4-1908026

# Ljubljana, Slovenia, 26-30 Aug 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR |  | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to 38.101-1 rel. 15 to fix missing Out-of-band Blocking exceptions for NE-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-08-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Out-of-band Blocking exceptions have been introduced in RAN4#91, however, for NE-DC they haven’t been copied |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the EN-DC OOBB exceptions to EN-DC |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No OOBB exceptions are allowed and the UE may fail the OOBB test |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.6B.3.3a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <Start of Changes>

#### 7.6B.3.3	Inter-band EN-DC within FR1

Out-of band blocking requirements for E-UTRA single carrier and CA operation specified in subclauses 7.6.2.1 and 7.6.2.1A of [4] and for NR single carrier and CA operation specified in subclauses 7.6.3 and 7.6A.3 of [2] apply for lowest level EN-DC fallbacks (two bands) in section 5.2.B.4.1 with following conditions

-	one E-UTRA uplink carrier with the output power set to 4 dB below PCMAX_L and the NR band whose downlink is being tested has its uplink carrier output power set to minimum output power as defined in subclause 6.3.1 of [2]

-	one NR uplink carrier with the output power set to 4 dB below PCMAX_L on the NR band with both E-UTRA and NR downlinks being tested with E-UTRA output power set to minimum output power as defined in subclause 6.3.2.1 of [4].

For each of the two test cases in subclauses 7.6.2.1 and 7.6.2.1A of [4] and for NR single carrier and CA operation specified in subclauses 7.6.3 and 7.6A.3 of [2] for all interferer frequency ranges a maximum of

exceptions are allowed for spurious response frequencies in each assigned frequency channel when measured using a step size of   MHz with NRB the number of resource blocks in the downlink transmission bandwidth configuration, CBW the bandwidth of the frequency channel in MHz and n = 1, 2, 3 for SCS = 15, 30, 60 kHz, respectively. For these exceptions, the requirements in subclause 7.7 apply.

#### 7.6B.3.3a	Inter-band NE-DC within FR1

Out-of band blocking requirements for E-UTRA single carrier and CA operation specified in subclauses 7.6.2.1 and 7.6.2.1A of [4] and for NR single carrier and CA operation specified in subclauses 7.6.3 and 7.6A.3 of [2] apply for lowest level NE-DC fallbacks (two bands) in section 5.5.B.4a.1 with following conditions

-	one E-UTRA uplink carrier with the output power set to 4 dB below PCMAX_L and the NR band whose downlink is being tested has its uplink carrier output power set to minimum output power as defined in subclause 6.3.1 of [2]

-	one NR uplink carrier with the output power set to 4 dB below PCMAX_L on the NR band with both E-UTRA and NR downlinks being tested with E-UTRA output power set to minimum output power as defined in subclause 6.3.2.1 of [4].

For each of the two test cases in subclauses 7.6.2.1 and 7.6.2.1A of [4] and for NR single carrier and CA operation specified in subclauses 7.6.3 and 7.6A.3 of [2] for all interferer frequency ranges a maximum of

exceptions are allowed for spurious response frequencies in each assigned frequency channel when measured using a step size of   MHz with NRB the number of resource blocks in the downlink transmission bandwidth configuration, CBW the bandwidth of the frequency channel in MHz and n = 1, 2, 3 for SCS = 15, 30, 60 kHz, respectively. For these exceptions, the requirements in subclause 7.7 apply.

## <End of Changes>