3GPP TSG-RAN WG4 Meeting #92              	R4-1908116

Ljubljana, SI, 26th– 30th Aug, 2019

Agenda item:	7.12.2.2.1

Source:	Samsung

Title:	Updated simulation results for NR UCI on PUSCH

Document for:	Discussion

# 1	Introduction

During RAN#91 meeting, the general assumption for performance requirement of UCI on PUSCH are agreed and captured in the WF [1].

In this contribution, the initial simulation results with ideal and impairment are provided.

# 2	Simulation Results

In this section, based on the agreed simulation assumption for UCI on PUSCH as table 1, the ideal and impairment simulation results are presented as table 2 and table 3

Table 1: Simulation assumption for UCI on PUSCH for CP-OFDM

| Parameter | value |  |
| --- | --- | --- |
|  | FR1 | FR2 |
| Transform precoding | Disabled | Disabled |
| Number of Tx | 1 | 1 |
| Number of Rx | 2 | 2 |
| Number of layers | 1 | 1 |
| [Uplink-downlink allocation for TDD] | 30 kHz SCS: 7D1S2U, S=6D:4G:4U | 120kHz SCS: 3D1S1U, S=10D:2G:2U |
| Code block group based PUSCH transmission | Disabled | Disabled |
| MCS index | 16 | 16 |
| Propagation Condition | MCS 16: TDL-C 300ns, 100 Hz | MCS 16: TDL-A 30ns, 300Hz |
| The number of HARQ transmission | 1 | 1 |
| Transmission scheme | Identity matrix (TPMI index 0) | Identity matrix (TPMI index 0) |
| DMRS type | type 1 | type 1 |
| Number of DMRS symbols | 1+1 (2,11) for type A 1+1 (0,10) for type B | 1 (0) for type B 1+1 (0,8) for type B |
| symbols length | 14 | 10 |
| Start symbol index | 0 | 0 |
| Time domain resource allocation type | type A, type B | type B |
| Frequency domain resource | Full RB allocation of the applicable BW | Full RB allocation of the applicable BW |
| Carrier frequency (GHz) | 4 | 30 |
| Propagation channel | MCS 16: TDL-C 300ns, 100 Hz | MCS 16: TDL-A 30 ns, 300Hz |
| SCS and BW | 30 KHz: 10MHz 120Khz: 50MHz |  |
| Timing offset | 0 | 0 |
| Frequency offset | 0 | 0 |
| PTRS | Not configured | Configured with KPTRS =2, LPTRS =1 |
|  |  | Not configured |
| Code block group | Disabled | Disabled |
| Frequency hopping | Disabled | Disabled |
| Limited buffer rate matching | Disabled | Disabled |
| UCI | With PUSCH data | With PUSCH data |
| Payload | Case1: 7( csi part1=5, csi_part=2) Case2:40 (csi_part1=20, csi_part2=20 ) | Case1: 7 (csi part1=5, csi_part=2) Case2 : 40( csi_part1=20, csi_part2=20) |
| Payload content | Case 1:[0 1 0 1 0] for CSI part1 with 5bit;[1 0] for CSI part2 with 2bit Case 2: Random payload content selection for (20,20) configuration | Case 1:[0 1 0 1 0] for CSI part1 with 5bit;[1 0] for CSI part2 with 2bit Case 2: Random payload content selection for (20,20) configuration |
| alpha Uci_on_pusch_scallng | 1 | 1 |
| BetaOffset-HARQ_ACK_index | 11 | 11 |
| BetaOffsetCSI-part1_Index and BetaOffsetCSI-part2_Index | 13 | 13 |
| Test Metric | 0.1% BLER of UCI with CSI part1, 1% BLER of UCI with CSI part2 | 0.1% BLER of UCI with CSI part1, 1% BLER of UCI with CSI part2 |

Table2: The ideal performance for NR PUSCH with 1 Tx with CP-OFDM

| SCS,BW | DMRS configuration | channel | UCI bit | PT-RS configuration | Target SNR with 0.1% BLER of CSI part1 | Target SNR with 1% BLER of CSI part2 |
| --- | --- | --- | --- | --- | --- | --- |
| 30kHz: 10MHz | 1+1 (2,11) | TDL-C 300ns, 100Hz | 7 bit (csi1=5, csi2 =2) | Not configured | 4.51 | -1.83 |
|  | 1+1 (2,11) |  | 40 bit (csi1=20, csi2 =20) |  | 2.73 | 0.45 |
|  | 1+1 (0,10) |  | 7 bit (csi1=5, csi2 =2) |  | 4.77 | -1.89 |
|  | 1+1 (0,10) |  | 40 bit (csi1=20, csi2 =20) |  | 1.86 | 0.43 |
| 120KHz, 50MHz | 1(0) | TDL-A 30ns, 300Hz | 7 bit (csi1=5, csi2 =2) | With PT-RS configured (Kpt-rs=2,Lpt-rs=1) | 5.2 | -0.2 |
|  | 1(0) |  | 40 bit (csi1=20, csi2 =20) |  | 4.1 | 2.1 |
|  | 1+1(0,8) |  | 7 bit (csi1=5, csi2 =2) |  | 5.6 | -0.4 |
|  | 1+1(0,8) |  | 40 bit (csi1=20, csi2 =20) |  | 4.1 | 1.8 |
|  | 1(0) |  | 7 bit (csi1=5, csi2 =2) | Without PT-RS configuration | 4 | -0.8 |
|  | 1(0) |  | 40 bit (csi1=20, csi2 =20) |  | 3.8 | 1.5 |
|  | 1+1(0,8) |  | 7 bit (csi1=5, csi2 =2) |  | 4.3 | -1.2 |
|  | 1+1(0,8) |  | 40 bit (csi1=20, csi2 =20) |  | 3.3 | 1.1 |

Table3: The impairment performance for NR PUSCH with 1 Tx with CP-OFDM

| SCS,BW | DMRS configuration | channel | UCI bit | PT-RS configuration | Target SNR with 0.1% BLER of CSI part1 | Target SNR with 1% BLER of CSI part2 |
| --- | --- | --- | --- | --- | --- | --- |
| 30kHz: 10MHz | 1+1 (2,11) | TDL-C 300ns, 100Hz | 7 bit (csi1=5, csi2 =2) | Not configured | 6.51 | 0.17 |
|  | 1+1 (2,11) |  | 40 bit (csi1=20, csi2 =20) |  | 4.73 | 2.45 |
|  | 1+1 (0,10) |  | 7 bit (csi1=5, csi2 =2) |  | 6.77 | 0.11 |
|  | 1+1 (0,10) |  | 40 bit (csi1=20, csi2 =20) |  | 3.86 | 2.43 |
| 120KHz, 50MHz | 1(0) | TDL-A 30ns, 300Hz | 7 bit (csi1=5, csi2 =2) | With PT-RS configured (Kpt-rs=2,Lpt-rs=1) | 7.2 | 1.8 |
|  | 1(0) |  | 40 bit (csi1=20, csi2 =20) |  | 6.1 | 4.1 |
|  | 1+1(0,8) |  | 7 bit (csi1=5, csi2 =2) |  | 7.6 | 1.6 |
|  | 1+1(0,8) |  | 40 bit (csi1=20, csi2 =20) |  | 6.1 | 3.8 |
|  | 1(0) |  | 7 bit (csi1=5, csi2 =2) | Without PT-RS configuration | 6 | 1.2 |
|  | 1(0) |  | 40 bit (csi1=20, csi2 =20) |  | 5.8 | 3.5 |
|  | 1+1(0,8) |  | 7 bit (csi1=5, csi2 =2) |  | 6.3 | 0.8 |
|  | 1+1(0,8) |  | 40 bit (csi1=20, csi2 =20) |  | 5.3 | 3.1 |

# 3	Conclusion

In this contribution, the initial ideal and impairment simulation results for UCI on PUSCH are provided.

# References

R4-1907240 WF on performance requirements of UCI Multiplexing on PUSCH, Samsung.